3GPP TSG-RAN WG4 Meeting #107									              R4-2307646

Incheon, Korea, May 22 – 26, 2023

Agenda item:	8.25.2.1

Source:	Apple

Title:	Discussion on general aspects and scenarios of L1/L2 based inter-cell mobility

Document for:	Discussion

# Introduction

L1/L2 based inter-cell mobility - General aspects and scenarios were widely discussed during the previous RAN4 meetings. The last agreements can be found in [1], in which there are still some open issues. In this contribution, we continue discussing the open issues.

# Discussion

The first open issue is about DL pre-sync starting point and UE capability requirements for DL pre-sync:

Issue 1-2-2: DL pre-sync starting point and UE capability requirements for DL pre-sync

< Way forward >: FFS

  - Proposal 1: RAN4 to agree that UE starts performing DL pre-sync with LTM candidates when UE configured with TA establishment or reception of configuration for TA establishment.

  - Proposal 2: RAN4 to discuss the UE capability aspects of downlink synchronisation to multiple cells so that UE can transmit PRACH to the candidate cell on the first PRACH occasion after the PDCCH order reception.

We assume ‘DL pre-sync’ here means fine time tracking since rough sync is done during cell search and L3 measurement phase. With this assumption, one possible starting point is the time when UE receives PDCCH order to trigger PRACH to candidate cell. Because the UL PRACH transmission is supposed to meet Te requirements, which requires UE to perform fine time tracking first. Regarding the UE capability in P2, it is being discussed in RAN1 and there is already some agreements [2]:

Agreement For PDCCH ordered RACH mechanism in R18 LTM, when reception of RAR is configured, - the UE stores(remembers/maintains/handles) a TA for at least one candidate cell - storing(remembering/maintaining/handling) corresponding TAs for more than one candidate cell is up to UE capability - detailed number of candidate cell is up to UE capability

Proposal 1: starting point of fine time tracking is the time when UE receives PDCCH order to trigger PRACH to candidate cell. FFS for RACH-less scenario.

Proposal 2: since UE capability regarding number of candidate cells for UE to stores(remembers/maintains/handles) TA is being discussed in RAN1, it shall not be discussed in RAN4 in parallel.

The next issue is about UL timing accuracy of PDCCH ordered RACH before cell switch command.

Issue 1-3-1: Transmit timing accuracy of PDCCH ordered RACH before cell switch command

< Agreement>:

  - The legacy transmit timing accuracy requirement in 38.133 cl.7.1.2 is also applicable to PDCCH ordered RACH transmission for candidate cell(s) before cell switch command.

    - FFS: SSB based T/F fine tracking is needed for UE to meet Te requirements

We believe SSB based T/F fine tracking is necessary for UE to meet Te requirements. Even in existing Te requirements, the availability of SSB is one of the applicability conditions.

Proposal 3: SSB based T/F fine tracking is necessary for UE to meet Te requirements for PDCCH ordered RACH before cell switch command.

The next issue is about delay requirement for PDCCH ordered RACH before cell switch command:

Issue 1-3-2: Delay requirements for PDCCH ordered RACH before cell switch command

< Way forward >: FFS the following options

  - Option 1 (Apple, Ericsson, QC): Better to define similar delay requirements for PDCCH ordered RACH for candidate cell(s) in RAN1 but not RAN4

  - Option 2 (MTK, Xiaomi, vivo, Huawei, ZTE, Nokia): RAN4 to study delay requirements for PDCCH ordered PRACH transmission to neighbour cell.

    - Option 2a (xiaomi):

      - If the reception of RAR is not configured/indicated, the delay of PDCCH-order based RACH transmission is defined as the time between the time of DCI command reception for PDCCH-order based RACH and the end of preamble transmission to the candidate cell plus RF retuning back time.

      - If the reception of RAR is configured/indicated, the delay of the PDCCH-order based RACH transmission is defined as the time between the time of DCI command reception for PDCCH-order based RACH and the end of RAR window plus RF retuning back time.

    - Option 2b (vivo):

      - RAN4 discuss and clarify whether the UL BWP of target cell is activated during uplink synchronization. Delay requirements need to be discussed after RAN1/2 concludes the corresponding procedure.

      - There is potential application delay and interruption for a cell on which DL sync is indicated to be performed by UE and/or UE needs to be prepared to transmit PRACH to the target cell. RAN4 will discuss the corresponding delay requirements.

    - Option 2c (QC):

      - RAN4 to discuss whether and how to define delay requirements for PDCCH ordered PRACH transmission to LTM cell for which UE needs additional processing to build and load RF scripts. It is also up to decisions from other working groups.

      - LTM requirements are applicable only when a QCL source reference signal of “PDCCH ordered PRACH to an LTM candidate cell before LTM handover” is the same or one of the reference signals configured and used for LTM L1-RSRP measurements from the cell.

We believe this shall be captured in RAN1 spec, same as usual. Actually, RAN1 is already discussing this issue. Corresponding agreement can be found in their LS [2]:

- Time gap between a PDCCH order and the corresponding PRACH transmission RAN1 discussed the time gap between a PDCCH order and the corresponding PRACH transmission for LTM. RAN1 believes that this will require that the time gap is increased at least for the following scenario - For PDCCH-order based PRACH on a candidate cell that is not a current serving cell with PUCCH/PUSCH or inter-frequency with the current serving cell RAN1 relies on RAN4: - to verify the need for the above additional latency and, if so, the corresponding value is needed. - to investigate any impact/interruption on UL Tx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH - to verify the need for any update is required to ΔBWPSwitching, ΔDelay if so, the corresponding values and whether UE capability is needed Potential RAN1 spec update will be based on RAN4’s feedback.

What RAN4 needs to do is to study additional delay and corresponding conditions as requested by RAN1. We assume RAN1 will capture this additional delay in their spec since they mentioned ‘Potential RAN1 spec update will be based on RAN4’s feedback’.

Proposal 4: RAN4 shall discuss the additional delay and corresponding interruption requirement for PDCCH ordered RACH on candidate cell before cell switch command.

Proposal 5: delay requirement for PDCCH ordered RACH on candidate cell before cell switch command shall be captured in RAN1.

Proposal 6: requirements of interruption on source serving cell(s) due to PDCCH ordered RACH on candidate cell before cell switch command shall be specified in RAN4.

# Conclusion

In this contribution, we provide further discussion on general aspects and scenarios for LTM. After discussion, the following conclusions are provided:

Proposal 1: starting point of fine time tracking is the time when UE receives PDCCH order to trigger PRACH to candidate cell. FFS for RACH-less scenario.

Proposal 2: since UE capability regarding number of candidate cells for UE to stores(remembers/maintains/handles) TA is being discussed in RAN1, it shall not be discussed in RAN4 in parallel.

Proposal 3: SSB based T/F fine tracking is necessary for UE to meet Te requirements for PDCCH ordered RACH before cell switch command.

Proposal 4: RAN4 shall discuss the additional delay and corresponding interruption requirement for PDCCH ordered RACH on candidate cell before cell switch command.

Proposal 5: delay requirement for PDCCH ordered RACH on candidate cell before cell switch command shall be captured in RAN1.

Proposal 6: requirements of interruption on source serving cell(s) due to PDCCH ordered RACH on candidate cell before cell switch command shall be specified in RAN4.

# References

- R4-2306395, WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek Inc.

- R1-2304276, LS on beam indication of target cell(s) and time gap between a PDCCH order and the corresponding PRACH transmission for LTM, Fujitsu, MediaTek

- R4-2303175, WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek Inc.