3GPP TSG-RAN WG4 Meeting#107                                    R4-2307670

Incheon, Korea, 22th – 26th May, 2023

Source: 	Apple

Title: 	On testing for beam correspondence

Agenda Item:	8.7.3.3

Document for:	Discussion

# 1. Introduction

Open issues to be concluded:

- Beam lock function

- How to reach maximum output power for BC test in IA

- How to maintain output power for BC test in IA

There have been some discussions on test issue for Beam correspondence in initial access [1-7] but no consensus has been reached. The following issues need to be further discussed according to the WF in [8].

This contribution will submit our views for the open issues on beam correspondence test.

# 2. Discussion

2.1 Beam lock function

Issue 3-1-1: Beam lock function

- Option 1: Beam lock function could be used to solve the polarization issue during the test and no further discussion is needed in RAN4.

- Option 2: It is not justified to rely on UBF for IA EIRP testing. DL polarizations during msg1 EIRP verification follow same practice as PUSCH EIRP testing.

- Option 3: In case RAN5 finds it feasible to define a BEAMLOCK function for IDLE/INACTIVE modes, how the System simulator can communicate/ instruct the UE to lock its beam during the Random-access procedure needs to be further studied.

- Option 4: Further discuss if a beam lock function is needed for beam correspondence in initial access based on the understanding that the objective of the BC IA test is NOT to lock the beam during the initial access.

- Option 5: It is up to RAN5. All the potential approaches have no direct impact on the minimum requirement.

The following options were capture in [8] for further study.

Regarding Option 1, it seems no obvious objection from the last RAN4 meeting.

Regarding Option 2 and Option 3, they can be further addressed in detail in future RAN5 work.

Proposal 1: It is proposed to leave Option 1-3 to RAN5 future work and no further discussion in RAN4.

Regarding Option 4, it comes from the founding in [9], where it is mentioned that the behaviour to seek out the best UL beam is desired and should not be suppressed for tests where single direction measurement is possible. As clarified in that paper, the intention is to ensure that the UE can generate sufficient EIRP in the direction of the DL at least for the duration of a power measurement for Msg1 (or MsgA).

We think the motivation on Option 4 needs further clarification in RAN4 before involving RAN5 for test solutions. E.g. why there such one shot test has more advantage than using beam lock function. Further, such test is only relevant for minimum peak EIRP testing. For spherical coverage testing, beam lock function is still needed in our understanding.

Proposal 2: Option 4 could be further discussed if RAN4 has conclusion on introducing minimum peak EIRP.

2.2 How to achieve and maintain maximum output power in IA test

Issue 3-1-2: Holding RAR

- Option 1: The maximum output power in initial access is achievable for the first preamble by well design the parameter. the maximum output power can be maintained during the test by holing RAR through parameter setting on preamble power step and number of retransmissions.

- Option 2: UE’s real performance in field shall be verified by ‘power ramping’ behaviour in initial access. With proper parameter setting, maximum output power could be easily achieved by holding RAR message for several times.

- Option 3: we should carefully take care of ra-ResponseWindow parameter to make sure the EIRP testing has been finished based on max power before fourth re-transmission of PREAMBLE.

- Option 4: It is up to RAN5. All the potential approaches have no direct impact on the minimum requirement.

The following options were captured in [8] for further study. This section will resubmit our analysis on Option 1. In general, we are ok to capture all Option 1-3 in TR 38.891 so that RAN5 can further check the feasibility of each solution and work on the details.

How to achieve Maximum output power (Option 1)

As has been discussed in [6], during the initial access, the UE measures the RSRP and estimates the PL based on the broadcasted DL RS power at first. The required power for PRACH transmission will be calculated based on the estimated PL and the target Rx power at gNB and compares with the Pcmax. Hence the PRACH power is determined by the following formula as defined in 38.213.

[dBm],

Where,  is the PRACH target reception power PREAMBLE_RECEIVED_TARGET_POWER provided by higher layers, and  is a pathloss calculated by the UE in dB as referenceSignalPower – higher layer filtered RSRP in dBm, and referenceSignalPower is provided by ss-PBCH-BlockPower

To ensure the UE is transmitting at Pcmax,f,c, we can choose to set the referenceSignalPower sufficiently high so that the calculated power of the second term in {} is always larger than Pcmax,f,c and the min{} function is dominated by Pcmax,f,c.

Observation: The maximum output power in initial access is achievable for the first preamble by well design the parameter.

One issue to be considered further is that the open loop power control accuracy should be well accounted when setting the parameter. The current open loop power control accuracy is specified in Table 6.3.4.2-1 of 38.101-2 where the main factor for tolerance largely comes from pathloss estimation and have non-negligible impact. So, the referenceSignalPower should have sufficient margin to cover the pathloss estimation error and ensure UE is always set its transmission at maximum output power state.

Table 6.3.4.2-1: Absolute power tolerance

| Power Range | Tolerance |
| --- | --- |
| Pint ≥ P ≥ Pmin | ± 14.0 dB |
| Pmax ≥ P > Pint | ± 12.0 dB |

Proposal 3: The impact of absolute power inaccuracy should be well accounted during test procedure design and parameter development.

How to maintain maximum output power (Option 1)

In the following measurement process, the UE maximum transmission power should be maintained until the test is done with a reasonable measurement period. As mentioned in [4], it can be maintained by holding RAR (msg2) and the target received power, preamble power step and number of preamble retransmissions. Parameter details can be further discussed in RAN5.

Proposal 4: The maximum output power can be maintained during the test by holing RAR through parameter setting on preamble power step and number of retransmissions.

2.4 Test coverage

Regarding the scenarios to be tested, RAN4 has agreed to focus on initial access since it is the most critical one among all 3 scenarios of IA, CG-SDT and RA-SDT. This conclusion needs to be sent to RAN5 so that they can focus on the beam lock function development for initial access only.

Proposal 5: Send LS to RAN5 about RAN4 decision on focusing beam correspondence test in IA.

Proposal 6: Capture all test solutions discussed in RAN4 into TR38.891 and ask RAN5 to check the feasibility. RAN5 can further discuss and decide how to achieve maximum output power during the test.

# 3. Summary

This contribution presented our consideration on beam correspondence test in initial access. The following observation and proposals are concluded. A test proposal is also provided for TR 38.891 based on Option 1 in this contribution.

Proposal 1: It is proposed to leave Option 1-3 to RAN5 future work and no further discussion in RAN4.

Proposal 2: Option 4 could be further discussed if RAN4 has conclusion on introducing minimum peak EIRP.

Observation: The maximum output power in initial access is achievable for the first preamble by well design the parameter.

Proposal 3: The impact of absolute power inaccuracy should be well accounted during test procedure design and parameter development.

Proposal 4: The maximum output power can be maintained during the test by holing RAR through parameter setting on preamble power step and number of retransmissions.

Proposal 5: Send LS to RAN5 about RAN4 decision on focusing beam correspondence test in IA.

Proposal 6: Capture all test solutions discussed in RAN4 into TR38.891 and ask RAN5 to check the feasibility. RAN5 can further discuss and decide how to achieve maximum output power during the test.

# 4. References

[1] R4-2217782, Email discussion summary for [104-bis-e][130] FR2_enh_req_Ph3_part1, Nokia

[2] R4-2217727, WF on BC in RRC_INACTIVE and initial access, Nokia

[3]	R4-1914121, Test configuration for beam correspondence during initial access, Ericsson

[4] R4-2215632, discussion on Beam correspondence test issues, Apple

[5] R4-2218691, Further consideration on beam correspondence test issue, Apple

[6] R4-2302822, Topic summary for [106][129] FR2_enh_req_Ph3_part1, Nokia

[7] R4-2303717, WF on beam correspondence requirements for initial access and RRC_inactive mode, Apple

[8] R4-2306600,WF on beam correspondence in initial access and RRC_INACTIVE, Nokia