3GPP TSG-RAN WG4 Meeting # 107	R4-2307674

# Incheon, Korea, 22th – 26th May, 2023

Source: 	Apple

Title: 	TP for TR 38.891: Testing impact

Agenda Item:	8.7.3.3

Document for:	Approval

# Introduction

This contribution presents a text proposal on testing impact analysis for beam correspondence in FR2 for TR 38.891.

# Reference

[1] R4-2301618, TR38.891

[2] R4-2307670, On testing for beam correspondence, Apple

# Text proposal

----------------------------------------------------------Start of Text Proposal------------------------------------------------------------

# 6	Study for beam correspondence

## 6.1	General

## 6.2	Implementation impact to UE

## 6.3	Specification Impact

### 6.3.1	RF

### 6.3.2	RRM

## 6.4	Testing impact

The test for beam correspondence requirements in RRC_CONNECTED state is done at UE maximum output power which is realized by closed loop power control. The beam direction is also locked with beam lock function during the test. The maximum output power and beam direction should be maintained during beam correspondence test for initial access.

While RAN5 has confirmed the feasibility to develop new beam lock function or extending the current beam lock function for initial, new mechanisms are still under development for establishing and maintaining maximum output power during beam correspondence test in IA. RAN4 has identified several potential solutions as captured below. Further revisit of this solutions in RAN5 is not precluded.

Option 1: Achieve maximum output power by well design parameter + maintain maximum output power by holding RAR

During the initial access, the UE measures the RSRP and estimates the PL based on the broadcasted DL RS power at first. The required power for PRACH transmission will be calculated based on the estimated PL and the target Rx power at gNB and compares with the Pcmax. Hence the PRACH power is determined by the following formula as defined in 38.213.

[dBm],

Where,  is the PRACH target reception power PREAMBLE_RECEIVED_TARGET_POWER provided by higher layers, and  is a pathloss calculated by the UE in dB as referenceSignalPower – higher layer filtered RSRP in dBm, and referenceSignalPower is provided by ss-PBCH-BlockPower

To ensure the UE is transmitting at Pcmax,f,c, we can choose to set the referenceSignalPower sufficiently high so that the calculated power of the second term in {} is always larger than Pcmax,f,c and the min{} function is dominated by Pcmax,f,c.

One issue to be considered further is that the open loop power control accuracy should be well accounted when setting the parameter. The current open loop power control accuracy is specified in Table 6.3.4.2-1 of 38.101-2 where the main factor for tolerance largely comes from pathloss estimation and have non-negligible impact. So, the referenceSignalPower should have sufficient margin to cover the pathloss estimation error and ensure UE is always set its transmitting target at maximum output power state.

| Power Range | Tolerance |
| --- | --- |
| Pint ≥ P ≥ Pmin | ± 14.0 dB |
| Pmax ≥ P > Pint | ± 12.0 dB |

In the following measurement process, the UE maximum transmission power should be maintained until the test is done with a reasonable measurement period. It can be maintained by holding RAR (msg2) and the target received power, preamble power step and number of preamble retransmissions. Parameter details can be further discussed when developing test cases in RAN5.

Option 2:

<To be added>

Option 3:

<To be added>

Option 4:

Other more suitable solutions are not precluded if any identified by RAN5 in its future work.

## 6.5	Conclusion

---------------------------------------------------------End of Text Proposal--------------------------------------------------------------