3GPP TSG-RAN WG4 Meeting #107	R4-2307688

# Incheon, South Korea, 22nd – 26th May 2023

Source:	Ericsson

Title:	Discussion on general issue for ATG demodulation

Agenda item:	8.14.5.1

Document for:	Discussion

# Introduction

In the last meeting, RAN4 start the discussion on the ATG demodulation scenario, channel model, requirement scope etc. Companies got some agreements but there are many open issues in WF [1]. In this contribution, the open issues on general part will be furtherly analysed.

# Discussion

## 2.1 Doppler shift in channel model

Doppler shift assumption

Agreement from GTW:

- Align with RF session agreement on frequency error requirements with below values as starting point

  - For FDD channel model, set Doppler as [220Hz]

  - For TDD channel model, set Doppler as [400/500Hz].

FFS:

- For FDD channel model,

  - Option 1: set Doppler as 220Hz

  - Option 2: Set Doppler as 200Hz for UL, 220Hz for DL

- For TDD channel model:

  - Option 1: set Doppler as 400Hz.

  - Option 2: set Doppler as 500Hz

As agreed in previous meeting, 0.1ppm is assumed as Doppler shift after ATG UE compensation at current stage. Regarding different band will have different values, companies proposed options above for FDD and TDD separately. But it should be noted that the Doppler shift here is not Doppler spread and the agreed channel is AWGN. The frequency difference is small between options.  It should be no trouble to use relative higher value considering DM-RS 1+1 configuration is agreed.

Proposal 1: Take following Doppler shift for ATG channel.

- FDD: 220Hz for both UL and DL

- TDD: 500Hz for both UL and DL

## 2.2 TDD pattern

TDD pattern

- Option 1: Define a new TDD pattern 30D4S6U which only applied for ATG scenario.

- Option 2: Proceed with typical TDD pattern for requirement, further check the performance if the new TDD pattern is agreed to be introduced

  - RF, RRM and Demod sessions should keep the consistency on the agreement for the necessary of new TDD pattern for ATG scenario.

  - Start with FDD requirement configuration if companies cannot get consensus on TDD pattern issue easily

- Option 3: If new dedicated TDD pattern is considered, note should be added in the specification that this is pattern is for ATG scenario used only.

The TDD issue is raised due to the very large propagation delay between ATG UE and BS, the timing advance for UL feedback would be possibly aggress the DL slot in the worst case (i.e., ISD>=200km). To avoid this happening, some companies propose to consider new TDD pattern with much longer DL slots and guard period. This issue would impact RF coexistence, RRM timing and also Demod, so an aligned agreement should be made among all three sessions. Otherwise, it is hard to say a new TDD pattern can be applied in commercial network, and it is not reasonable let product to support such kind of TDD pattern.

In previous meeting, the new TDD pattern (30D4S6U) was only brought up in Demod session and the feasibility of the new TDD pattern was discussed. Following are main arguments:

- Co-existing interference with legacy TDD pattern. This is the major issue caused by new TDD pattern. Some companies argued that the new pattern could be considered in only ATG scenario which means the ATG network is far away from TN network. But the isolation distance could be at least 50km based on our simulations [6]. With this distance, the throughput degradation on victim TN cell could be under 5%. But it seems not feasible to have such large isolation distance regarding the widely deployed TN network.

- HARQ ACK and process number. The RRC parameters DL-DataToUL-ACK and nrofHARQ-Process are extended in Rel-17 to support higher values which could solve the very long DL slot issue caused by new TDD pattern (see DL-DataToUL-ACK-v1700 and nrofHARQ-Process-r17 in TS38.331 [7]). But this also means that ATG UE should have new capability to support these Rel-17 RRC parameters.

- The 4 GP slot is to solve the very large propagation distance between BS and UE, i.e.,300km, but less GP slot would be configured if the ATG BS deployment is denser than current assumption and the propagation delay gets smaller. It means that different deployments/operators would have different TDD pattern preferences to solve the issue. In that case, ATG UE need to support multiple TDD patterns because it might fly across different operators’ networks. The implementation complexity could also be very high and hard for compatibility. A generic solution for large TDD cell should be considered by RAN4.

It seems not possible to avoid co-existing interference caused by new TDD pattern in real network.

New UE capability of support Rel-17 RRC parameters is needed to support new TDD pattern.

If only depending on new TDD pattern to solve guard period issue, an ATG UE might need to support different TDD patterns used by different ATG networks which will increase the complexity of implementation.

In RRM session, a new method to mitigate this impact was proposed by Ericsson [3] is under discussion. The thinking is to dynamically configure GP period and UL slot at ATG UE side according to the information of propagation distance. The method could let ATG BS use same TDD pattern as TN BS to avoid co-existence interference and reduce the capacity loss caused by multiple GP slots. ATG UE could also be adapted to different network configurations. Of course, it might impact on the specification at a certain level but it could avoid the implementation complexity in the long run.

Proposal 2: RAN4 to discuss a generic method to mitigate the guard period impact to let ATG UE can support different deployment networks.

## 2.3 link budget of ATG

Companies didn’t get consensus on if introducing the requirements of 256QAM for ATG scenario. One argument is the link budget might be an issue to support 256QAM. We delivered link budget analysis at the beginning of ATG discussion [2]. In this chapter, a simplified link budget calculation with agreed ATG BS and UE assumption is delivered.

The distance between ATG UE and BS could be 10km ~ 300km. The typical propagation channel is single tap channel, so free space propagation loss could be used. Other parameters for ATG BS and ATG UE are captured in table 2-1 which refers to TS38.876 6.2.2 [4]. Subarray antenna model is used for both FDD and TDD ATG BS in following calculation.

Table 2-1 System and antenna parameters for ATG BS and UE

| Centre frequency [GHz] |  | 2 | 4 |
| --- | --- | --- | --- |
| CBW [MHz] |  | 20 | 100 |
| PRB number |  | 106 | 273 |
| ATG BS system | Tx max power [dBm] | 43 | 50 |
|  | noise figure [dB] | 5 | 5 |
| ATG BS antenna | Element gain [dBi] | 6.4 |  |
|  | Array configuration (Row × Column) | 4x8 |  |
|  | Sub-array elements | 3 |  |
| ATG UE system | TRP or EIRP [dBm] | 40 | 43 |
|  | noise figure [dB] | 9 | 9 |
| ATG UE antenna | element gain [dBi] | 0 | 5 |
|  | Array configuration (Row × Column) | \ | (8x2) or (16x1) |

The intention here is to show the possibility of using 256QAM in ATG network, so the largest antenna gain is assumed for all propagation distance. The SNR at receiver side for 2GHz and 4GHz are showed in figure 2-1.

Figure 2-1 Received SNR vs. propagation distance

According to our simulation for PDSCH [5] and PUSCH, the SNR vs normalized throughput for 256QAM MCS27 and MCS20 are showed in following figures. The results show that TDD scenario could have clear degradation compared to FDD due to larger pathloss and larger channel bandwidth.

The results are based 2Rx to 8Rx as normal demodulation configurations for 256QAM MCS20 and MCS27. Based on link simulation, the operating SNR (95% peak throughput) are ~20dB for 256QAM MCS20 and ~28dB for 256QAM MCS27 at the receiver after antenna combination. Compared with link budget results, we can see that:

- For FDD 20MHz CBW, 256QAM could be used for both DL and UL when distance is less than 140km.

- For TDD 100MHz CBW, 256QAM could be used for DL when distance is less than 70km and could be used for UL when distance is less than 50km. But CBW could less than 100MHz, i.e., 40MHz, then the applicable distance could be extended to 100km for DL and 80km for UL.

Normally, 256QAM MCS22 could have higher peak throughput than 64QAM MCS28 under same CBW and its operating SNR could be ~22dB, but the link budget still show enough margin for it especially for FDD deployment. From the requirement perspective, it is necessary to introduce 256QAM requirements since it can be used by the ATG scenario.

There are obvious margins in link budget to support 256QAM in ATG scenario.

Proposal 3: 	Introduce 256QAM requirements for ATG DL/UL demodulation.

# Conclusions

In the previous sections we made the following observations:

Observation 1	It seems not possible to avoid co-existing interference caused by new TDD pattern in real network.

Observation 2	New UE capability of support Rel-17 RRC parameters is needed to support new TDD pattern.

Observation 3	If only depending on new TDD pattern to solve guard period issue, an ATG UE might need to support different TDD patterns used by different ATG networks which will increase the complexity of implementation.

Observation 4	There are obvious margins in link budget to support 256QAM in ATG scenario.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Take following Doppler shift for ATG channel.

	FDD: 220Hz for both UL and DL

	TDD: 500Hz for both UL and DL

Proposal 2: RAN4 to discuss a generic method to mitigate the guard period impact to let ATG UE can support different deployment networks.

Proposal 3: 	Introduce 256QAM requirements for ATG DL/UL demodulation.

# References

[1]	R4-2305899, WF on NR ATG Demod, CMCC

[2] 	R4-2212617, ATG link budget, Ericsson

[3]	R4-2304834, On timing requirements in ATG, Ericsson

[4]	TS38.876, v0.2.0

[5]	R4-2307835, Simulation results for ATG UE demodulation, Ericsson

[7]	R4-2308533, Co-existing simulation for ATG unsynchronized scenario, Ericsson

[8]	TS38.331, v17.4.0