TSG-RAN Working Group 4 (Radio) meeting #107	R4-2307701

Incheon, Korea, 22nd to 26th May 2023

Source:	Ericsson

Title:	TP to TR 38.858: Addition of coexistence simulation assumptions to Annex D

Agenda item:	8.20.2.1

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#106-bis-E) simulation assumptions related to the adjacent carrier coexistence evaluation part of SBFD SI (FS_NR_duplex_evo) was discussed extensively. The agreements related to simulation assumptions was captured in a way-forward contribution in [1] and a detailed summary of all assumptions was created in [2]. In addition, it was also agreed to update blocking models as described in LS to RAN1 [4].

In this contribution a text proposal is created to collect all simulation assumptions in TR 38.858, Annex D. The text proposal is attached at the end of this contribution, and it is presented for approval.

# Discussion

The SBFD coexistence evaluation relies on modelling two network deployed in the same geographical area operating at adjacent carriers. The victim impact of DL and UL performance is studied for different deployment scenarios defined by RAN4. The currently defined deployment scenarios include Urban Macro to Urban Macro, Urban Hotspot to Urban Hotspot, Indoor to Indoor and Urban Macro to Urban Micro.

The impact is studied both for SBFD and TDD as victim system using different configuration cases. For each case the victim is either DL or UL for TDD and SBFD. Which gives four different cases to study.

The coexistence evaluation is based on simulation results produced by a model requiring an extensive set of parameter assumptions. Parameter can be divided into following categories:

- For each deployment scenario and FR, assumptions for related to the network deployment is required.

- For each deployment scenario and FR, assumptions related to the carrier configuration is required.

- For each BS class (Wide area, Medium Range, Local Area) and FR, BS RF characteristics is required.

- For each FR, UE RF parameter assumptions are required.

In addition to parameter assumptions and models used for previous coexistence evaluations in RAN4, new aspects unique for SBFD needs to be modelled. For this purpose, new models are defined for power scaling, receiver blocking, UE ACLR versus UE power, etc. The details related to these new modes needs to be captured in TR 38.858. In this contribution a text proposal to TR 38.858, Annex D is provided for discussion with the intention to be approved at the end of the meeting. Since the existence evaluation have started and simulation results have been presented by several companies it is essential to capture the technical background information in TR 38.858.

The information to this text proposal have been collected from agreed documents form last RAN4 meeting:

- WF from last meeting [1]

- Parameter overview in [2]

- WF on Macro to Micro simulation parameters in [3]

- LS to RAN1 [4]

- In BS RF chairman notes it’s also agreed to use the blocking model in [4] for coexistence evaluation.

# Conclusion

This contribution carries a text proposal to TR 38.858, Annex D with all simulation assumptions used for the adjacent carrier coexistence evaluation part of SBFD SI. The text proposal is created based on agreements from last meeting.

The text proposal is attached at the end of this contribution, and it is presented for approval.

# References

[1]	R4-2305921, “WF for SBFD adjacent channel co-existence study”, CMCC

[2] 	R4-2305923, “Summary of co-existence simulation parameters”, CMCC, CableLabs, Samsung

[3]	R4-2305922, “WF on Uma- Umi scenario simulation parameters”, CableLabs

[4]	R4-2305920, “Reply LS on interference modelling for duplex evolution”, Samsung

TEXT PROPOSAL:

Annex <D>:
Adjacent channel co-existence evaluation

# D.1	Simulation scenarios

The coexistence evaluation is conducted considering the different scenarios listed in Table D.1-1.

Table D.1-1: Network deployment scenarios

| Scenario | FR | Aggressor | Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro | High |
| 2 | FR1 | Urban Hotspot | Urban Hotspot | Low |
| 3 | FR1 | Indoor | Indoor | Low |
| 4 | FR1 | Urban Macro | Micro | Low |
| 5 | FR2-1 | Urban Macro | Urban Macro | High |
| 6 | FR2-1 | Urban Hotspot | Urban Hotspot | Low |
| 7 | FR2-1 | Urban Micro | Urban Micro | Low |
| 8 | FR2-1 | Indoor | Indoor | Low |

The Urban Hotspot uses the same parameter assumption as Urban Macro, except that Urban Macro uses random dropping method for UE while Urban Hotspot uses cluster-based dropping method for UE.

The coexistence evaluation captures cases where TDD and SBFD is both victim and aggressor network. This to evaluate impact on legacy TDD networks if SBFD is introduced in a neighbouring channel, also to understand impact on SBFD network due to the legacy TDD network, as described in Table D.1-2.

Table D.1-2: Coexistence cases

| Case | Aggressor | Victim | Slot allocation Aggressor Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | SBFD | TDD DL |  | High |
| 2 | SBFD | TDD UL |  | Low |
| 3 | TDD DL | SBFD |  | High |
| 4 | TDD UL | SBFD |  | Low |

# D.2	Simulation assumptions

## D.2.1	Deployment

Simulation assumptions related to network layout is captured for FR1 in Table D.2.1-1 and for FR2-1 in Table D.2.1-2.

Table D.2.1-1: FR1 deployment parameters

| Parameter | Urban Macro | Micro | Indoor |
| --- | --- | --- | --- |
| Layout | Single layer with 19 hexagonal cells with wrap around. | Single layer with 19 hexagonal cells with wrap around. | Total 12 BSs (Operator A: 6 BSs & Operator B: 6 BSs) 120 x 50 m |
| Inter-BS distance | 500 m | 289 m | 20 m |
| Grid shift | 0, 10, 100 % For calibration use 100 % | 100% as starting point The victim network (UMi) in the center, the aggressor network (UMa) moved by the grid offset. 100% is relative to the micro ISD. | N/A |
| Carrier frequency | 4 GHz | 4 GHz | 4 GHz |
| Path-loss model | BS-to-UE: UMa see TR 38.803 BS-to-BS: Option 1 (2nd priority): If the 2D distance between two BS are less than or equal to the ISD, set the LOS probability to X; Otherwise, reuse BS-to-UE LOS probability equation in TR 38.828. X = 0.75. For other cases, reuse BS-to-UE LOS probability equation in TR 38.803. Option 2 (1st priority): Reuse the same model as in TR 38.828 with h_UT equals to 25m. UE-to-UE: UMi (h_BS=1.5m) + penetration loss, see TR 38.803. UMi model is not applicable when 2D distance is less than 10 m, instead FSPL is applicable. | BS-to-UE: UMi see TR 38.803 BS-to-BS: UMa (h_UE = 10 m) see TR 38.803. For LoS probability for Macro-to-Micro case: • Reuse the same model as in TR 38.828 with h_UT = 10 m UE-to-UE: Outdoor UE – Outdoor UE see TR 36.828 + penetration loss see TR 38.803. UMi model is not applicable when 2D distance is less than 10m, instead free space model is applicable | BS-to-BS: InH-office BS-to-UE: InH-office UE-to-UE: InH-office As described in TR 38.803, subclause 5.2.2.1 |
| Site-interference receiver sensitivity degradation | Case 1: 1 dB Case 2: Reported by company | Case 1: 1 dB Case 2: Reported by company | N/A |
| BS height | 25 m | 10 m | 3 m |
| UE height | 1.5 m | Use TR 38.828 UE dropping assumption hUT=3(nfl-1)+1.5 nfl for outdoor UEs: 1 nfl for indoor UEs: nfl~uniform(1,Nfl) where Nfl = 1 | 1.5 m |
| UE distribution | Random dropping Baseline: 20% indoor and 80% outdoor Optional: 80% indoor and 20% outdoor | Uniformly UE distribution in the cell 20% indoor and 80% outdoor | Cluster-based dropping 100% indoor |
| Minimum BS-UE (2D) distance | 35 m | 5 m | 0 m |
| Minimum UE-UE (2D) distance | 3 m 1 m when UEs are in cluster | 3 m | 1 m |
| DL power control | No | No | No |
| UL power control | Yes | Yes | Yes |
| Handover margin | 3 dB | 3 dB | 3 dB |
| BS mechanical down-tilt angle | 6 degrees | 6 degrees | 90 degrees |

Table D.2.1-2: FR2-1 deployment parameters

| Parameter | Urban Macro | Urban Micro | Indoor |
| --- | --- | --- | --- |
| Layout | Single layer with 19 hexagonal cells with wrap around. | Fixed cluster circle within a macro cell. Number of micro BSs per macro cell is 3. Radius of UE dropping within a micro cell is < 28.9 m. Minimum distance between micro BSs in different operator is 10 m. Shadowing correlation is 0.5. | Total 12 BSs (Operator A: 6 BSs & Operator B: 6 BSs) 120 x 50 m |
| Inter-BS distance | 200 m | 10 m Minimum distance between micro BSs in different operator | 20 m |
| Grid shift | 0, 10, 100 % For calibration use 100 % | N/A | N/A |
| Carrier frequency | 30 GHz | 30 GHz | 30 GHz |
| Path-loss model | BS-to-UE: UMa + penetration loss see TR 38.803 BS-to-BS: Option 1 (2nd priority): If the 2D distance between two BS are less than or equal to the ISD, set the LOS probability to X; Otherwise, reuse BS-to-UE LOS probability equation in TR 38.828. X = 0.75. For other cases, reuse BS-to-UE LOS probability equation in TR 38.803. Option 2 (1st priority): Reuse the same model as in TR 38.828 with h_UT equals to 25m. UE-to-UE: UMi (h_BS=1.5m) + penetration loss, see TR 38.803. UMi model is not applicable when 2D distance is less than 10 m, instead FSPL is applicable. | BS-to-UE: UMi + penetration loss BS-to-BS: UMi UE-to-UE: UMi (h_BS=1.5m~22.5m) + penetration loss, see TR 38.803. UMi model is not applicable when 2D distance is less than 10 m, instead FSPL is applicable. | BS-to-BS: InH-office BS-to-UE: InH-office UE-to-UE: InH-office As described in TR 38.803, subclause 5.2.2.1 |
| Site-interference receiver sensitivity degradation | Case 1: 1 dB Case 2: Reported by company floor. | N/A | N/A |
| BS height | 25 m | 10 m | 3 m |
| UE height | 1.5 m | 1.5 m ≦ hUT ≦ 22.5 m | 1.5 m |
| UE distribution | Indoor UE ratio: 0% Uniform UE distribution | Indoor UE ratio: 80 % 50% low loss, 50% high loss Uniform UE distribution | 100% indoor |
| Minimum BS-UE (2D) distance | 35 m | 3 m | 0 m |
| Minimum UE-UE (2D) distance | 3 m 1 m when UEs are in cluster | 3 m | 1 m |
| DL power control | No | No | No |
| UL power control | Yes | Yes | Yes |
| Handover margin | 3 dB | 3 dB | 3 dB |
| BS mechanical down-tilt angle | 6 degrees | 10 degrees | 90 degrees |

## D.2.2	Traffic

Assumptions related to network traffic load configuration for FR1 is captured in Table D.2.2-1 and for FR2-1 in Table D.2.2-2.

Table D.2.2-1: FR1 network traffic parameters

| Parameter | Urban Macro | Micro | Indoor |
| --- | --- | --- | --- |
| Carrier bandwidth | 100 MHz | 100 MHz | 100 MHz |
| Scheduled channel bandwidth per UE (DL) | For legacy TDD: 100 MHz For SBFD {DU}: 80 MHz | For legacy TDD: 10 MHz For SBFD {DU}: 80 MHz | For legacy TDD: 100 MHz For SBFD {DU}: 80 MHz |
| Scheduled channel bandwidth per UE (UL) | For legacy TDD: 100 MHz For SBFD {DU}: 20 MHz | For legacy TDD: 100 MHz For SBFD {DU}: 20 MHz | For legacy TDD: 100 MHz For SBFD {DU}: 20 MHz |
| SBFD BS PSD | The PSD of SBFD is the same as TDD | The PSD of SBFD is the same as TDD | The PSD of SBFD is the same as TDD |
| Traffic load | Full buffer | Full buffer | Full buffer |

Table D.2.2-2: FR2-1 network traffic parameters

| Parameter | Urban Macro | Urban Micro | Indoor |
| --- | --- | --- | --- |
| Carrier bandwidth | 200 MHz | 200 MHz | 200 MHz |
| Scheduled channel bandwidth per UE (DL) | For legacy TDD: 200 MHz For SBFD {DU}: 160 MHz | For legacy TDD: 200 MHz For SBFD {DU}: 160 MHz | For legacy TDD: 200 MHz For SBFD {DU}: 160 MHz |
| Scheduled channel bandwidth per UE (UL) | For legacy TDD: 200 MHz For SBFD {DU}: 40 MHz | For legacy TDD: 200 MHz For SBFD {DU}: 40 MHz | For legacy TDD: 200 MHz For {DU}: 40 MHz |
| SBFD BS PSD | The PSD of SBFD is the same as legacy TDD | The PSD of SBFD is the same as legacy TDD | The PSD of SBFD is the same as legacy TDD |
| Traffic load | Full buffer | Full buffer | Full buffer |

## D.2.3	BS RF characteristics

The SBFD system is defined for two different antenna configurations:

- SBFD config 1: Which corresponds to splitting the antenna in half and reducing the number element in the vertical domain by 2.

- SBFD config 2: Which corresponds to maintain antenna area for transmitter and receiver. Therefore, the AAS base station length will increase by a factor 2.

The array antenna model, for single element configuration, is described in TR 38.803, subclause 5.2.3, and the antenna model extension, required for modelling sub-array configurations, is described in TR 38.803, subclause 5.2.3.2.4.

Power boosting of 3 dB can be considered to compensate for fewer transmitter branches in the case of SBFD config 1.

Assumptions related to BS RF characteristics relevant for different deployment scenarios are captured for FR1 in Table D.2.3-1 and for FR2-1 in Table D.2.3-2.

Table D.2.3-1: FR1 BS parameters

| Parameter | Urban Macro (Wide Area) | Micro (Medium Range) | Indoor (Local Area) |
| --- | --- | --- | --- |
| Transmitter power (Total conducted power) | Option 1: TDD: 49 dBm SBFD config 1: 46 dBm SBFD config 2: 49 dBm Option 2: TDD: 53 dBm SBFD config 1: 50 dBm SBFD config 2: 53 dBm | TDD: 46 dBm SBFD: 46 dBm | TDD: [24] dBm SBFD config 1: [21] dBm SBFD config 2: [24] dBm |
| Antenna configuration | TDD: (Mg,Ng,M,N,P)=(1,1,8,8,2) SBFD config 1: (Mg,Ng,M,N,P)= (1,1,4,8,2) SBFD config 2: (Mg,Ng,M,N,P)= (1,1,8,8,2) (dH,dV)=(0.5,0.8)λ, (q3dB,j3dB)=(65,65) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi Optional: Extended AAS model in TR 38.803, subclause 5.2.3.2.4. | TDD: (Mg,Ng,M,N,P)=(1,1,2,2,2) SBFD: (Mg,Ng,M,N,P)=(1,1,2,2,2) (dH,dV)=(0.5,0.8)λ, (q3dB,j3dB)=(65,65) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi | TDD: (Mg,Ng,M,N,P) =(1,1,4,4,2) SBFD config 1: (Mg,Ng,M,N,P) =(1,1,2,4,2) SBFD config 2: (Mg,Ng,M,N,P) =(1,1,4,4,2) (dH,dV)=(0.5,0.5)λ, (q3dB,j3dB)=(90,90) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi |
| Receiver noise figure parameters | P1=-33 dBm P2=-15 dBm F1=5 dB F2=14 dB See Figure D.2.3-1 | P1=-38 dBm P2=-20 dBm F1=10 dB F2=19 dB See Figure D.2.3-1 | P1=-35 dBm P2=-17 dBm F1=13 dB F2=22 dB See Figure D.2.3-1 |
| ACLR | 45 dBc | 45 dBc | 45 dBc |
| ACS | 46 dBc | 46 dBc | 46 dBc |
| BS self-interference receiver sensitivity degradation | 1 dB | 1 dB | 1 dB |

Table D.2.3-2: FR2-1 BS parameters

| Parameter | Urban Macro | Urban Micro | Indoor |
| --- | --- | --- | --- |
| Transmitter power (Total conducted power) | Option 1 (1st priority): TDD: 30 dBm SBFD config 1: 27 dBm SBFD config 2: 30 dBm Option 2 (2nd priority): TDD: 40 dBm SBFD config 1: 37 dBm SBFD config 2: 40 dBm | TDD: 30 dBm SBFD config 1: 27 dBm SBFD config 2: 30 dBm | TDD: [24] dBm SBFD config 1: [21] dBm SBFD config 2: [24] dBm |
| Antenna configuration | TDD: (Mg,Ng,M,N,P)=(1,1,8,16,2) SBFD config 1: (Mg,Ng,M,N,P)=(1,1,4,16,2) SBFD config 2: (Mg,Ng,M,N,P)=(1,1,8,16,2) (dH,dV)=(0.5,0.5)λ (q3dB, j3dB)=(65,65) o, SLAv=30 dB, Am=30 dB, GE,max=3 dBi Optional: (q3dB, j3dB)=(90,90) o, SLAv=30 dB, Am=30 dB, GE,max=5.5 dBi | TDD: (Mg,Ng,M,N,P)=(1,1,8,16,2) SBFD config 1: (Mg,Ng,M,N,P)=(1,1,4,16,2) SBFD config 2: (Mg,Ng,M,N,P)=(1,1,8,16,2) (dH,dV)=(0.5,0.5)λ (q3dB, j3dB)=(65,65) o, SLAv=30 dB, Am=30 dB, GE,max=3 dBi | TDD: (Mg,Ng,M,N,P)=(1,1,4,8,2) SBFD config 1: (Mg,Ng,M,N,P)=(1,1,2,8,2) SBFD config 2: (Mg,Ng,M,N,P)=(1,1,4,8,2) (dH,dV)=(0.5,0.5)λ (q3dB, j3dB)=(90,90) o, SLAv=30 dB Am=30 dB, GE,max=3 dBi |
| Receiver noise figure parameters | Option 1: P1=-63 dBm P2=-45 dBm F1=10 dB F2=19 dB Option 2: P1=-52 dBm F1=10 dB A sloped section rising by 1 dB/dB for Pin>-52 dBm See Figure D.2.3-1 | Option 1: P1=-63 dBm P2=-45 dBm F1=10 dB F2=19 dB Option 2: P1=-52 dBm F1=10 dB A sloped section rising by 1 dB/dB for Pin>-52 dBm See Figure D.2.3-1 | Option 1: P1=-63 dBm P2=-45 dBm F1=10 dB F2=19 dB Option 2: P1=-52 dBm F1=10 dB A sloped section rising by 1 dB/dB for Pin>-52 dBm See Figure D.2.3-1 |
| ACLR | 28 dBc | 28 dBc | 28 dBc |
| ACS | 24 dBc | 24 dBc | 24 dBc |
| BS self-interference receiver sensitivity degradation | 1 dB | 1 dB | 1 dB |

To capture receiver blocking aspects RAN4 have decided to adopt a piece wise linear model for the noise figure characteristics as shown in Figure D.2.3-1.

# Figure D.2.3-1: Noise figure model

If the total received power is larger than P2 the receiver will be blocked, and the throughput set to zero.

## D.2.4	UE RF characteristics

Assumptions relevant for modelling the UE RF characteristics are captured in Table D.2.4-1.

Table D.2.4-1: UE parameters

| Parameter | FR1 | FR2-1 |
| --- | --- | --- |
| Maximum transmitter power | 23 dBm | 22.4 dBm (EIRP) |
| Minimum transmitter power | -33 dBm (TS 38.101-1, Table 6.3.1-1) | -40 dBm |
| Antenna configuration | 0 dBi | (Mg,Ng,M,N,P) = (1,1,2,2,2) (dH,dV)=(0.5,0.5)λ (q3dB, j3dB)=(90,90) o, SLAv=25 dB Am=25 dB, GE,max=5.5 dBi |
| Receiver noise figure | 9 dB | 10 dB |
| ACLR | 30 dBc | 23 dBc |
| ACS | 33 dBc | 23 dBc |

# D.3	Simulation methodology

## D.3.1	Coexistance evaluation methodology

The coexistence evaluation methodology can be summarized as:

- Aggressor and victim network are generated. UEs are distributed as described by parameter assumptions.

- UEs are associated to BS based on coupling loss.

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS and UE. This is done for both victim and aggressor networks.

- Throughput is computed in the victim systems without considering Adjacent Channel Interference (ACI) as:

ThputNO ACI[bit/s/Hz]=f(SINRICI)=f(S/(N+IICI)), where IICI is the inter-cell interference.

- Throughput is computed considering ACI as:

ThputACI[bit/s/Hz]=f(SINRICI+ACI)=f(S/(N+IICI+IACI)), where IACI is the adjacent channel interference.

- RF parameters are determined based on the degradation caused by ACI as:

LossACI=1-ThputACI/ThputSINGLE

The simulation results should be in the form of the throughput with adjacent network with DTDD and without DTDD and the relative difference between the two can be compared at 50% and 5% points.

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput (Tp) in bps/Hz over a channel with a given SNIR, when using link adaptation:

Tp=0∝SSNIR∝SSNIRMAXfor SNIR≤SNIRMIN for SNIRMIN≤SNIR<SNIRMAXfor SNIR≥SNIRMAX

Where:

S(SNIR)   Shannon bound, S(SNIR) =log2(1+SNIR)  bps/Hz
			Attenuation factor, representing implementation losses
SNIRMIN  	Minimum SNIR of the code set, dB
SNIRMAX  Maximum SNIR of the code set, dB

The parameters can be chosen to represent different modem implementations and link conditions. A parameter set relevant for eMBB is listed in Table D.3.1-1.

Table D.3.1-1: Parameters describing baseline Link Level performance for NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| a, attenuation | 0.6 | 0.4 | Represent implementation losses |
| SNIRMIN | -10 | -10 | Based on QPSK, 1/8 rate (DL) and 1/5 rate (UL) |
| SNIRMAX | 30 | 22 | Based on 256QAM, 0.93 (DL) and 64QAM 0.93 (UL) |

For the case of SBFD the total throughput needs to be evaluated per slot since the SNIR will be different for different slots. The total throughput will be evaluated as the sum over all allocated UL slots.

## D.3.2	Received signal power model

The following model is applied:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

, where

RX_PWR is the received power.

TX_PWR is the transmitted power.

G_TX is the transmitter antenna gain (directional array gain).

G_RX is the receiver antenna gain (directional array gain).

## D.3.3	Network grid shift

In cases with grid shift >0%, the victim network base stations should be placed as shown in Figure D.3.3-1. The victim network grid is shifted along the line between BS and its closest 100%-grid-shift BS, where the distance from any BS in victim network to its second and third closest BS in the aggressor network is the same.

# Figure D.3.3-1: Network grid shift definition

## D.3.4	Coupling-loss

The Coupling Loss (CL) is defined as the loss in signal between BS-to-UE, UE-to-UE and BS-to-BS. CL is defined as the loss including propagation loss and antenna gains with BF weights applied measured between antenna connectors.

## D.3.5	Transmission power control

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in TR 36.942, subclause 9.1 is applied with following parameters.

-	CLx-ile = –SNR_target + UE_max_eirp– ThermalNoise – BS_NoiseFigure - 10*log10(BW)

-	γ = 1

Where, SNR_target for FR1 and FR2 are 15 dB.

In simulation, power control scheme is only used to compensate path loss. That’s the reason why final SINR for UL is less than assumed target SINR. But commercial UE UL SINR could meet target value according to power control scheme in TS 38.213.