TSG-RAN Working Group 4 (Radio) meeting #107	R4-2307702

Incheon, Korea, 22nd to 26th May 2023

Source:	Ericsson

Title:	Additional SBFD coexistence simulation results based on updated assumptions

Agenda item:	8.20.2.1

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#106-bis-E) simulation assumptions and simulation results were discussed. Results relevant for the calibration of simulators were collected in a common contribution [1]. It can be noticed that the calibration results converge among companies for FR1 and FR2 calibration scenarios. In [2] we also provided simulation results beyond the calibration assumptions to capture relevant deployment aspects. In this document we used an approach with an optimistic case and a realistic case to better understand the interference situation relevant for the SBFD coexistence evaluation. From the results it can clearly be noticed that coexistence between a legacy TDD network and an adjacent SBFD capable network will interfere each other, and when realistic isolation assumptions for e.g., inter-sector interference, and blocking model are considered, SBFD faces important challenges to operate properly.

In this contribution we provide additional simulation results based on updated simulation assumptions from last meeting. The simulation results are also presented in proposed template from last meeting [5].

# Discussion

In this contribution we follow the RAN4 coexistence evaluation methodology applied to the 4 coexistence cases identified for study during previous meeting. For each coexistence case we identify a “relative ACIR” value, which we sweep to achieve the RAN4 coexistence target of 5% degradation for mean user throughput and 5%-tile user throughput with respect to the baseline defined by the study in previous meetings. Specifically, for each identified coexistence case, the reference ACIR is derived from baseline assumptions for legacy TDD and SBFD BS and UE, as described in the following Table 2-1 and Table 2-2.

Table 2-1: FR1 Baseline ACIR derived from baseline assumptions for legacy TDD and SBFD BS and UE

| Coexistence Case | Baseline ACIR | ACLR | ACS |
| --- | --- | --- | --- |
| 1 | 28.23 dB | ACLR UE SBFD = 30 dB | ACS UE TDD = 33 dB |
| 2 | 42.46 dB | ACLR BS SBFD = 45 dB | ACS BS TDD = 46 dB |
| 3 | 42.46 dB | ACLR BS TDD = 45 dB | ACS BS SBFD = 46 dB |
| 4 | 28.23 dB | ACLR UE TDD = 30 dB | ACS UE SBFD = 33 dB |

Table 2-2: FR2 Baseline ACIR derived from baseline assumptions for legacy TDD and SBFD BS and UE

| Coexistence Case | Baseline ACIR | ACLR | ACS |
| --- | --- | --- | --- |
| 1 | 20.00 dB | ACLR UE SBFD = 23 dB | ACS UE TDD = 23 dB |
| 2 | 22.50 dB | ACLR BS SBFD = 28 dB | ACS BS TDD = 24 dB |
| 3 | 42.46 dB | ACLR BS TDD = 45 dB | ACS BS SBFD = 46 dB |
| 4 | 22.50 dB | ACLR UE TDD = 28 dB | ACS UE SBFD = 24 dB |

In the study that follows we will sweep the identified ACIR value, but we will also assume that ACS or ACLR of the legacy system cannot be modified.

The considered simulation assumptions in the study described in the following section, are those agreed in RAN4 #106-bis-E and detailed in documents [4] and [5], which are also summarized in the Annex at the end of the document. A consolidated summary of all simulation assumptions has been created in a companion contribution [3] with the intention to capture information in TR 38.858. If not explicitly mentioned otherwise, simulations assume the first priority assumptions, as they have been defined in the reference documents, and in the Annex. For the FR2 case we also present results with second priority assumptions, to provide a deeper analysis, since we had not done it in previous meetings.

Specifically, depending on the coexistence case and scenario, in this document the following simulation configurations will be considered:

- Configuration 1: first priority assumptions defined in [4, 5], and summarized in the Annex, and no blocking model. This configuration will be studied for FR1 and FR2

- Configuration 2 It is equal to configuration 1 (first priority assumptions in [4, 5] and included in Annex), but the blocking model is considered at the BS receivers. This configuration is studied for FR1 and FR2, and only for cases where UL is the victim, since those are the cases where blocking has a stronger impact.

- Configuration 3: Some assumptions are varied with respect to configuration 1:  BS transmission power is set to 40 dBm, instead of 30 dBm, and the BS-to-BS LoS (Line of Sight) model is the one considered in RAN1 studies (i.e., is 75% when the distance is below the inter-site distance (ISD) and is the default UMa LoS model otherwise). Grid-Shift (GS) is 100%. The blocking model is not considered. This configuration is only presented for FR2 (similar results were presented previously for FR1)

- Configuration 4: Is equal to configuration 3, BS transmission power is set to 40 dBm, the BS-to-BS LoS model is the one considered in RAN1 studies, and the grid-shift is 10%. This configuration is presented only for FR2.

This section is organized in two subsections, sub-section 2.1 will focus on coexistence results for FR1, while sub-section 2.2 will focus on FR2. Internally, each subsection is organized according to the identified coexistence cases described in Table 2-3.

Table 2-3: Coexistence cases

| Case | Aggressor | Victim | Slot allocation Aggressor Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | SBFD | TDD DL |  | High |
| 2 | SBFD | TDD UL |  | Low |
| 3 | TDD DL | SBFD |  | High |
| 4 | TDD UL | SBFD |  | Low |

In this contribution we provide simulation results for deployment scenario 1 (Urban Macro scenario) and 2 (Urban hotspot scenario with clustered UEs) for FR1, and deployment scenario 5 (Urban Macro scenario) and 6 (Urban hotspot scenario with clustered UEs) for FR2. All the cases studied will be presented for Urban macro scenario. For cases where the DL is the victim, also urban hotspot is discussed.

  - FR1

For FR1 deployment scenario 1 and 2 will be considered. In addition, results for cases with and without blocking model are also modelled discussed for coexistence cases where UL is the victim.

    - Case 1

Case 1 refers to the coexistence case where the aggressor is SBFD operator, and the victim is the DL (down-link) of legacy static TDD operator.

Since we are interested in observing the impact of UE-to-UE CLI (cross-link interference), for this case, performance of both Urban Macro and Urban Hotspot scenarios (deployment scenarios 1 and 2) are considered.

- Urban macro (uniform distribution of users, deployment scenario 1), referred to as configuration 1.

Figure 2.1.1-1 shows the DL mean and 5%-tile user throughput of TDD network, in urban macro scenario, and as can be observed, there is not coexistence impact on the DL TDD performance in urban scenario. The reason is that in a wide area scenario, if users are dropped uniformly, it is highly unlikely that they happen to be at a distance where they are subject to UE-to-UE CLI. For this reason, for this coexistence case it is of interest to also study urban hotspot deployment scenario, with clustered UEs.

Figure 2.1.1-1: DL mean and 5%-tile user throughput of TDD, Urban scenario, FR1

- Urban hotspot (clustered distribution of users, deployment scenario 2), referred to as configuration 1.

Figure 2.1.1-2 shows the DL mean and 5%-tile user throughput of TDD network, in Urban Hotspot scenario. It can be observed that DL mean user throughput is not impacted by coexistence with SBFD operator, but 5%-tile user throughput is. In particular, considering the reference ACIR values based on current assumptions, coexistence of DL TDD with SBFD network causes a 14.7% degradation in 5%-tile throughput performance, with respect to the baseline. In order to reduce the degradation to a level lower than 5%, it is required to increase the ACIR by 20 dB. So up to 40 dB ACIR is required. Assuming that the ACS of the TDD UEs is fixed and equal to 33 dB, it is not possible to achieve an ACIR of 40 dB by increasing the ACLR of SBFD UEs.

Figure 2.1.1-2: DL mean and 5%-tile user throughput of TDD, Urban Hotspot, FR1

Observation 1: Coexistence of an SBFD network with a DL legacy TDD network is possible in a scenario where users are uniformly distributed. In this case, the UE-to-UE CLI does not cause harmful impact against the DL of the legacy network. However, this scenario may hide coexistence issues, because if users are uniformly distributed over a wide area, the probability that two users active in UL and DL at the same time are dropped close enough to each other to generate UE-to-UE CLI is extremely low.

Observation 2: For coexistence Case 1, where DL TDD is the victim, it is of interest to study as well the Urban Hotspot scenario: when the users are clustered, and the distance among them is reduced, it is higher the probability that users can interfere among each other. In this case, we observe that the DL mean user throughput is not affected by the coexistence with SBFD, but the 5%-tile throughput instead is, with up to 15% degradation with respect to the baseline. It is necessary to increase the ACIR of by 20 dB with respect to the baseline, so up to 40 dB, in order to reduce the degradation to an acceptable value below 5%. Assuming the legacy TDD UE ACS cannot be changed, it is not possible to achieve such a high value of ACIR in reality.

Similar observations can be obtained when introducing the agreed blocking model at the SBFD aggressor BSs. With the current priority assumptions, we observe a blocking probability in order of 2-3%, for the UL SBFD, depending on the scenario, if urban macro or urban hotspot.

    - Case 2

Case 2 refers to the coexistence case where the aggressor is the SBFD operator, and the victim is the UL (up-link) of the legacy static TDD operator. In this case we discuss only the urban macro scenario, because since the UL is the victim, the clustering of users has a reduced impact compared to previous case. We also consider both cases when the blocking is not modelled at the BS, and when instead is modelled, by means of the agreed model.

- Blocking model is not considered (configuration 1, deployment scenario 1)

Figure 2.1.2-1 shows the UL mean and 5%-tile user throughout of the TDD network, when no blocking is considered at SBFD and TDD BSs. It can be observed a mean user and 5%-tile throughput degradation of 20% and 75%, respectively, when baseline ACIR values defined in the assumptions of this study are considered. In order to reduce this degradation, the ACIR has been gradually increased, up to achieving less than 5% degradation of the performance compared to the baseline. It requires to add 14 dB to achieve an acceptable degradation below 5% for mean user throughout and 24 dB for 5%-tile throughout. Assuming that the ACS of the legacy BS is fixed to 46 dB, it is not possible to achieve the ACIR of 56 or 66 dB required to reduce the degradation below 5%, for mean and 5%-tile user throughput, respectively.

Figure 2.1.2-1: UL mean and 5%-tile user throughput of TDD, Urban scenario (no blocking modelled), FR1

- Blocking model is considered (configuration 2, deployment scenario 1)

Figure 2.1.2-2 shows the UL mean and 5%-tile user throughput of TDD when blocking is modelled. Considering the reference ACIR values based on assumptions, it can be observed a degradation of 23% and 82% in terms of mean and 5%-tile user throughout, respectively. In this case, due to the blocking of the UL TDD, which happens with approximately 3% probability due to the DL SBFD activity, and to the increment of the resulting noise figure, it is not possible to reduce the degradation, beyond a certain level, even if the ACIR is increased to very high and not realistic values.

Figure 2.1.2-2: UL mean and 5%-tile user throughput of TDD, Urban scenario (with blocking modelled), FR1

Observation 3: The UL of a TDD network is highly impacted by the coexistence with an SBFD network. 20% degradation is observed in terms of mean user throughput, and 75% in terms of 5%-tile user throughput.

Observation 4: When blocking is not modelled at SBFD and TDD BS receivers, it is possible to find an ACIR value that allows to reduce the TDD UL mean user throughput and 5%-tile user throughput degradation. However, if the ACS of the legacy TDD BS is fixed at 46 dB, it is not possible in reality to achieve the needed ACIR values to reduce the degradation (i.e., 56 and 66 dB for mean and 5%-tile user throughput, respectively).

Observation 5: When blocking is modelled at SBFD and TDD BS, approximately a 3% of blocking probability is observed, in case of uniform UE distribution, due to the CLI generated by the DL of SBFD neighbour operator. This probability of blocking, together with the increase in noise figure, harmfully impacts the UL performance of the TDD network in such a way that even increasing the ACIR, it is not possible to reduce the degradation to an acceptable level, below 5%, with respect to the baseline.

    - Case 3

Case 3 refers to the coexistence case where the aggressor is the legacy TDD operator, and the victim is the UL of the SBFD operator. Similarly, to case 2, and differently from case 1, we discuss only the urban macro scenario, because since the UL is the victim, the clustering is less interesting than case 1 or case 4, where the DL is the victim instead. We also consider both cases when the blocking is modelled and when is not modelled, by means of the agreed model.

- Blocking model is not considered (configuration 1, deployment scenario 1)

Figure 2.1.3-1 shows the UL mean and 5%-tile user throughout of the SBFD network, when no blocking is considered at SBFD and TDD BSs. It can be observed a mean user and 5%-tile throughput degradation of 17.4 % and 72 %, respectively, when baseline ACIR values defined in the assumptions of this study are considered. In order to reduce this degradation, the ACIR has been gradually increased up to obtain less than 5% degradation of performance with respect to the defined baseline. It requires to add 14 dB to achieve an acceptable degradation below 5% for mean user throughout and 20 dB for 5%-tile throughout. Assuming that the ACLR of the legacy BS is fixed to 45 dB, it is not possible to achieve the ACIR of 54 or 66 dB required to reduce the degradation of the performance in relation to the baseline, below 5%.

Figure 2.1.3-1: UL mean and 5%-tile user throughput of SBFD, Urban scenario (no blocking modelled), FR1

- Blocking model is considered (configuration 2, deployment scenario 1)

Figure 2.1.3-2 shows the UL mean and 5%-tile user throughput of SBFD when blocking is modelled. Considering the reference ACIR assumptions, it can be observed a degradation with respect to the baseline of 20% and 83%, in terms of mean and 5%-tile user throughout, respectively. In this case, due to the blocking of the UL of SBFD network, which happens with approximately 3% probability, and increased noise figure, due to the DL TDD activity, it is not possible to reduce the degradation, beyond a certain level, even if the ACIR is increased to very high and not realistic values.

Figure 2.1.3-2: UL mean and 5%-tile user throughput of SBFD, Urban scenario (with blocking modelled), FR1

Observation 6: The UL of a SBFD network is highly impacted by the coexistence with a legacy TDD network. 17.4% degradation is observed in terms of mean user throughput, and 72% in terms of 5%-tile user throughput.

Observation 7: When blocking is not modelled at SBFD and TDD BS receivers, it is possible to find an ACIR value that allows to reduce the SBFD UL mean user throughput and 5%-tile user throughput degradation. However, if the ACLR of the legacy TDD BS is fixed at 45 dB, it is not possible to achieve the needed ACIR values to reduce the degradation (i.e., 56 and 66 dB for mean and 5%-tile user throughput, respectively).

Observation 8: When blocking is modelled at SBFD and TDD BS, approximately a 3% of blocking probability is observed, in case of uniform UE distribution, due to the CLI generated by the DL of legacy TDD neighbour operator. This probability of blocking, together with the increment in resulting noise figure as defined by the blocking model, harmfully impacts the UL performance of SBFD network in such a way that even increasing the ACIR, it is not possible to reduce the degradation to an acceptable level, below 5%, with respect to the baseline.

    - Case 4

Case 4 refers to the coexistence case where the aggressor is the UL of legacy TDD operator, and the victim is the DL (down-link) of the SBFD operator.

Since we are interested in observing the impact of UE-to-UE CLI (cross-link interference), similarly to what was done in Case 1, for this case, performance of both Urban Macro and Urban Hotspot scenarios are considered.

- Urban macro (configuration 1, deployment scenario 1)

Figure 2.1.4-1 shows the DL mean and 5%-tile user throughput of TDD network, in urban macro scenario, and as can be observed, there is not coexistence impact on the DL TDD performance in urban scenario. The reason is that in a wide area scenario, if users are dropped uniformly, it is highly unlikely that they happen to be at a distance where they are subject to UE-to-UE CLI. For this reason, it is of interest to study also clustered deployments.

Figure 2.1.4-1: DL mean and 5%-tile user throughput of SBFD network, Urban scenario, FR1

- Urban hotspot (configuration 1, deployment scenario 2)

Figure 2.1.4-2 shows the DL mean and 5%-tile user throughput of SBFD network, in Urban Hotspot scenario. It can be observed that DL mean user throughput is not impacted by coexistence with the legacy TDD operator, but 5%-tile user throughput is. In particular, considering the reference ACIR values based on current assumptions, coexistence of UL TDD with DL SBFD network causes a 12.2% degradation in 5%-tile throughput performance, with respect to the baseline. In order to reduce the degradation to a level lower than 5%, it is required to increase the ACIR by 6 dB. So ACIR up to 34 dB is required. Assuming that the ACLR of the TDD UEs is fixed and equal to 30 dB, it is not possible to achieve an ACIR of 34 dB by increasing the ACS of SBFD UEs.

Figure 2.1.4-2: DL mean and 5%-tile user throughput of TDD, Urban Hotspot, FR1

Observation 9: Coexistence of the UL of a TDD network with the DL an SBFD network is possible in a scenario where users are uniformly distributed. In this case, the UE-to-UE CLI does not cause harmful impact against the DL of the SBFD network. However, this scenario may hide coexistence issues, because if users are uniformly distributed over a wide area, the probability that two users active in UL and DL at the same time are dropped close enough to each other to generate UE-to-UE CLI is extremely low.

Observation 10: For coexistence Case 4 is of interest to study as well the Urban Hotspot scenario: when the users are clustered, and the distance among them is reduced, it is higher the probability that users can interfere among each other. In this case, we observe that the DL mean user throughput is not affected by the coexistence with SBFD, but the 5%-tile throughput instead is, with up to 12.2% degradation with respect to the baseline. It is necessary to increase the ACIR of 6 dB with respect to the baseline, so up to 34 dB, in order to reduce the degradation to an acceptable value below 5%. Assuming the legacy TDD UE ACLR cannot be changed, it is not possible to achieve such a high value of ACIR in reality.

  - FR2

For FR2 deployment scenario 5 and 6 will be considered when the DL is the victim. In addition, results for cases with and without blocking model are discussed, for the cases where the UL is the victim. In this contribution only case 1 and case 3 is considered.

    - Case 1

Case 1 refers to the coexistence case where the aggressor is SBFD operator, and the victim is the DL (down-link) of legacy static TDD operator. As done for FR1 results, since in this case we are interested in observing the impact of UE-to-UE CLI, it is important to analyse also the performance of urban hotspot scenario, besides the urban macro where users are uniformly distributed in the coverage area.

- Urban macro (configuration 1, deployment scenario 5)

Figure 2.2.1-1 shows the DL mean and 5%-tile user throughput of TDD network, in urban macro scenario, and as can be observed, there is not coexistence impact on the DL TDD performance in urban scenario. Similarly, to what has been observed for Case 1 FR1, when users are uniformly dropped in a wide area, it is unlikely that they interfere each other.

Figure 2.2.1-1: DL mean and 5%-tile user throughput of TDD, Urban scenario, FR2

- Urban hotspot (configuration 1, deployment scenario 2)

Figure 2.2.1-2 shows the DL mean and 5%-tile user throughput of TDD network, in Urban Hotspot scenario. It can be observed that DL mean user throughput is not impacted by coexistence with SBFD operator, but 5%-tile user throughput is impacted. In particular, considering the reference ACIR values based on current assumptions, coexistence of DL TDD with SBFD network causes a 11.2% degradation in 5%-tile throughput performance, with respect to the baseline. In order to reduce the degradation to a level lower than 5%, it is required to increase the ACIR by 6 dB. So up to 26 dB ACIR is required. Assuming that the ACS of the TDD UEs is fixed and equal to 23 dB, in reality an ACIR of 26 dB cannot be achieved.

Figure 2.2.1-2: DL mean and 5%-tile user throughput of TDD, Urban hotspot, FR2

Observation 11: In FR2 and coexistence case 1, similarly to what observed for FR1, it is important to study both urban macro and urban hotspot scenario. The uniform distribution of UEs may hide coexistence issues. Coexistence is possible when users are uniformly distributed, but when the users are clustered it is necessary to increase the ACIR to 26 dB, which is not feasible assuming ACS of legacy UE is 23 dB.

    - Case 2

Coexistence case 2 is a low priority case, simulation results for FR2 will be provided in an upcoming meeting, for completeness.

    - Case 3

Case 3 refers to the coexistence case where the aggressor is the legacy DL TDD operator, and the victim is the UL of the SBFD operator. In this section we consider different simulation configuration, and present results for additional options beyond those identified in [4] and [5] as high priority. Specifically, results obtained for configuration 1, 2, 3 and 4 as discussed in section 2 will be presented.

a) 		Blocking model is not considered (Configuration 1, deployment scenario 1)

In Figure 2.2.3-1 UE mean and 5%-tile user throughout of SBFD are shown for urban scenario. Differently from the same coexistence case in FR1 it can be observed that the mean user throughout of UL SBFD is not impacted in this case by the DL of the legacy TDD. The reason is that the communications are more directional than in FR1, the transmission power is lower, and the path loss offers a better attenuation. UL 5%-tile throughput is instead slightly impacted by the coexistence with DL TDD, with a degradation of 6.5% with respect to the baseline, when considering reference ACIR (red bar). ACIR needs to be increased by 4 dB to reduce this degradation below 5%, so up to 24 dB. Assuming that the ACLR of the legacy TDD BS is fixed and equal to 28 dB, the ACS of SBFD BS should be increased to e.g., 26 dB.

Figure 2.2.3-1: UL mean and 5%-tile user throughput of SBFD, Urban scenario, FR2 (no blocking modelled)

b) 		Blocking model is considered (Configuration 2, deployment scenario 1)

When the blocking is considered in SBFD and TDD BSs, UL 5%-tile throughput degradation of 9.1% with respect to the baseline can be observed, when assuming reference ACIR. It is necessary to increase the ACIR by 6 dB so that the degradation gets to less than 5%. This means that the ACIR should reach 28 dB. Assuming the ACLR of legacy TDD BS is fixed to 28 dB, the ACS of the SBFD BS should be much higher than the 24 dB that are currently assumed.

Figure 2.2.3-2: UL mean and 5%-tile user throughput of SBFD, Urban scenario (with blocking modelled)

c) 		Blocking model is not considered, and TRP, LoS assumptions are varied (Configuration 3, deployment scenario 1)

When increasing the BS transmission power and considering a less optimistic LoS model, Coexistence of SBFD UL with a neighbour TDD DL network becomes more challenging, compared to configuration 1, which considers 30 dBm transmission power and UMa LoS.

Figure 2.2.3-3 shows that UL mean and 5%-tile user throughput of SBFD suffer a degradation with respect to the baseline of 5.1% and 29% in terms of mean and 5%-tile user throughput, assuming reference ACIR. To correct the 5%-tile user throughput, it is necessary to improve ACIR by 20 dB. So up to 42 dB. Assuming that the BS ACLR of the legacy network is fixed to 28 dB, it is not possible to improve ACS of SBFD BS in order to reach such a level of ACIR.

Figure 2.2.3-3: UL mean and 5%-tile user throughput of SBFD, Urban scenario (no blocking, TRP 40 dBm, UMa LoS, GS 100%)

d) 		Blocking model is not considered and TRP, LoS and GS assumptions are (Configuration 4, scenario deployment 1)

Coexistence becomes even more challenging than configuration 3, if grid-shift (GS) is reduced to 10%, as it can be observed in Figure 2.2.3-4. The UL mean user throughput of SBFD suffers a degradation of 9% when coexisting with DL TDD and the UL 5%-tile throughput reduces by 54%. To reduce this degradation below a threshold of 5%, it is necessary to increase the ACIR by 10 dB for the mean user throughput and by even more than 36 dB for 5%-tile throughput. Assuming that the BS ACLR of the legacy network is fixed to 28 dB, it is not possible to improve ACS of SBFD BS in order to reach such high levels of ACIR.

Figure 2.2.3-4: UL mean and 5%-tile user throughput of SBFD, Urban scenario (no blocking, TRP 40 dBm, UMa LoS, GS 10%)

Observation 12: In FR2 scenarios, for coexistence case 3, when the UL of SBFD is the victim and the DL of a legacy TDD system, coexistence is extremely sensitive to the assumptions that are made. When optimistic simulation assumptions and low transmission power are considered (e.g., LoS UMa, GS 100%, TRP 30 dBm), coexistence may be possible if SBFD BS ACS can be increased with respect to current assumptions. On the other hand, when more realistic assumptions in terms of LoS and GS are considered, and a higher TRP (40 dBm) is considered, coexistence is challenging and is not possible in reality to achieve the ACIR needed to reach a tolerable degradation level.

    - Case 4

Coexistence case 4 is a low priority case, simulation results for FR2 will be provided in an upcoming meeting, for completeness.

  - Simulation result overview

In this section the simulation results are provided in a format discussed at last meeting. The format is not exact according to the template from last meeting, it is adopted to fit the outcome from the simulation campaign. Each coexistence case is different and exposed to a different level of CLI, this is why a different range of ACIR values of interest are considered in the plots presented in previous sections and in the following tables. In addition, for coexistence cases where the DL is the victim, both urban macro and urban hotspot deployments have been simulated, while for cases where the UL is the victim, conclusions when users are clustered are similar, to those when users are not clustered. As a result, only results for urban macro scenario are reported. Vice versa, the blocking model is highly relevant for cases where the UL is the victim, so that for cases where the DL is the victim results are not reported and the conclusion are similar to the case when blocking is not modelled.  Due to all that, in the following tables there are some columns and lines which are left empty and this happens for 3 reasons: 1) the ACIR column is not of interest for the coexistence case, 2) urban hotspot or blocking are not reported for lines corresponding to cases where they are not of primary interest, 3) for coexistence cases 2 and 4, in FR2 tables, considering that those are low priority scenarios, they will be discussed for completeness in future meetings.

In Table 2.3-1, the simulation results relevant for FR1 without blocking model included captured.

Table 2.3-1: FR1 simulation results without blocking

| Deployment scenario | Company | Case | Observation Point | Baseline (Mbps) | Throughput (Mbps) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |
|  |  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 24 |
| 1 | Ericsson | 1 | 5% | 98.3 | 97.3 | 98.1 | 98.2 |  |  |  |  |  |
|  |  |  | 50% | 314 | 314 | 314 | 314 |  |  |  |  |  |
|  |  | 2 | 5% | 16.4 | 3.97 | 5.5 | 7.3 | 9.1 | 12.9 | 13.4 | 14.9 | 16 |
|  |  |  | 50% | 33.6 | 26.9 | 27.8 | 28.8 | 29.8 | 31.8 | 32.1 |  |  |
|  |  | 3 | 5% | 10.1 | 2.77 | 3.59 | 4.58 | 5.62 | 8 | 8.8 | 9.57 | 9.87 |
|  |  |  | 50% | 27.87 | 23 | 23.7 | 24.5 | 25 | 26.4 | 26.7 |  |  |
|  |  | 4 | 5% | 109 | 109 | 109 | 109 |  |  |  |  |  |
|  |  |  | 50% | 332 | 331 | 331 | 331 |  |  |  |  |  |
| 2 | Ericsson | 1 | 5% | 101.8 | 86.9 | 87 | 87.5 | 90.4 | 94.8 | 95.8 | 97.3 |  |
|  |  |  | 50% | 314 | 310 | 311 | 311 | 312 | 312 | 313 | 314 |  |
|  |  | 2 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 3 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 4 | 5% | 77.8 | 68 | 68.3 | 73.2 | 74 |  |  |  |  |
|  |  |  | 50% | 326 | 322 | 323 | 324 | 325 |  |  |  |  |

In Table 2.3-2, the simulation results relevant for FR1 with blocking model included captured.

Table 2.3-2: FR1 simulation results with blocking

| Deployment scenario | Company | Case | Observation Point | Baseline (Mbps) | Throughput (Mbps) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |
|  |  |  |  |  | 0 | 2 | 4 | 6 | 12 | 16 | 26 | 36 |
| 1 | Ericsson | 1 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 2 | 5% | 16.4 | 2.8 | 3.1 | 4.7 | 5.1 | 9.9 | 11.1 | 12 | 12 |
|  |  |  | 50% | 33.6 | 25.7 | 26.7 | 27.6 | 28.2 | 30 | 30.5 | 31 | 31 |
|  |  | 3 | 5% | 9.8 | 1.6 | 1.6 | 2.2 | 2.4 | 3.8 | 4 | 5 | 5 |
|  |  |  | 50% | 27.6 | 22 | 22.4 | 23.1 | 23.6 | 24.7 | 25 | 25 | 25 |
|  |  | 4 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |

In Table 2.3-3, the simulation results relevant for FR2 without blocking model included captured.

Table 2.3-3: FR2 simulation results without blocking (configuration 1)

| Deployment scenario | Company | Case | Observation Point | Baseline (Mbps) | Throughput (Mbps) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |
|  |  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 24 |
| 5 | Ericsson | 1 | 5% | 223 | 214 | 214 | 217 |  |  |  |  |  |
|  |  |  | 50% | 689 | 680 | 682 | 684 |  |  |  |  |  |
|  |  | 2 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 3 | 5% | 45.6 | 42.6 | 42.8 | 44.5 |  |  |  |  |  |
|  |  |  | 50% | 62.2 | 61.6 | 61.6 | 61.8 |  |  |  |  |  |
|  |  | 4 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
| 6 | Ericsson | 1 | 5% | 213 | 188 | 189 | 195 | 206 |  |  |  |  |
|  |  |  | 50% | 689 | 681 | 681 | 683 | 685 |  |  |  |  |
|  |  | 2 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 3 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 4 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |

In Table 2.3-4, the simulation results relevant for FR2 with blocking model included captured.

Table 2.3-4: FR2 simulation results with blocking (configuration 2)

| Deployment scenario | Company | Case | Observation Point | Baseline (Mbps) | Throughput (Mbps) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |
|  |  |  |  |  | 0 | 2 | 4 | 6 | 12 | 16 | 26 | 36 |
| 5 | Ericsson | 1 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 2 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 3 | 5% | 45.6 | 41.4 | 42.7 | 43 | 45 |  |  |  |  |
|  |  |  | 50% | 62.2 | 61.4 | 61.4 | 61.8 | 61.8 |  |  |  |  |
|  |  | 4 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |

In Table 2.3-5, the simulation results relevant for FR2 without blocking model included captured and optional simulation assumptions.

Table 2.3-5: FR2 simulation results no blocking (configuration 3, TRP 40 dBm, RAN1 LoS, GS 100)

| Deployment scenario | Company | Case | Observation Point | Baseline (Mbps) | Throughput (Mbps) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |
|  |  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 16 | 20 |
| 5 | Ericsson | 1 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 2 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 3 | 5% | 40.3 | 28.6 | 28.6 | 31.5 | 32.4 | 36.4 | 37.2 | 37.8 | 38 |
|  |  |  | 50% | 60.77 | 57.6 | 57.8 | 58.6 | 59 | 60 | 60 | 60 | 60.3 |
|  |  | 4 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |

In Table 2.3-6, the simulation results relevant for FR2 without blocking model included captured and optional simulation assumptions.

Table 2.3-6: FR2 simulation results no blocking (configuration 3, TRP 40 dBm, RAN1 LoS, GS 10)

| Deployment scenario | Company | Case | Observation Point | Baseline (Mbps) | Throughput (Mbps) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |
|  |  |  |  |  | 0 | 2 | 4 | 6 | 12 | 20 | 26 | 36 |
| 5 | Ericsson | 1 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 2 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 3 | 5% | 40.3 | 17.6 | 21.8 | 21.9 | 22.5 | 30.6 | 32.9 | 37.3 | 37.8 |
|  |  |  | 50% | 60.7 | 55.4 | 56.4 | 56.9 | 57.3 | 59 | 59.6 | 60.1 | 60.4 |
|  |  | 4 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution we present simulation results based on assumptions agreed last meeting. From the simulation results following observations have been captured:

Observation 1: Coexistence of an SBFD network with a DL legacy TDD network is possible in a scenario where users are uniformly distributed. In this case, the UE-to-UE CLI does not cause harmful impact against the DL of the legacy network. However, this scenario may hide coexistence issues, because if users are uniformly distributed over a wide area, the probability that two users active in UL and DL at the same time are dropped close enough to each other to generate UE-to-UE CLI is extremely low.

Observation 2: For coexistence Case 1 is of interest to study as well the Urban Hotspot scenario: when the users are clustered, and the distance among them is reduced, it is higher the probability that users can interfere among each other. In this case, we observe that the DL mean user throughput is not affected by the coexistence with SBFD, but the 5%-tile throughput instead is, with up to 15% degradation with respect to the baseline. It is necessary to increase the ACIR of 20 dB with respect to the baseline, so up to 40 dB, in order to reduce the degradation to an acceptable value below 5%. Assuming the legacy TDD UE ACS cannot be changed, it is not possible to achieve such a high value of ACIR in reality.

Observation 3: The UL of a TDD network is highly impacted by the coexistence with an SBFD network. 20% degradation is observed in terms of mean user throughput, and 75% in terms of 5%-tile user throughput.

Observation 4: When blocking is not modelled at SBFD and TDD BS receivers, it is possible to find an ACIR value that allows to reduce the TDD UL mean user throughput and 5%-tile user throughput degradation. However, if the ACS of the legacy TDD BS is fixed at 46 dB, it is not possible to achieve the needed ACIR values to reduce the degradation (i.e., 56 and 66 dB for mean and 5%-tile user throughput, respectively).

Observation 5: When blocking is modelled at SBFD and TDD BS, approximately a 3% of blocking probability is observed, in case of uniform UE distribution, due to the CLI generated by the DL of SBFD neighbour operator. This probability of blocking harmfully impacts the UL performance of the TDD network in such a way that even increasing the ACIR, it is not possible to reduce the degradation to an acceptable level, below 5%, with respect to the baseline.

Observation 6: The UL of a SBFD network is highly impacted by the coexistence with a legacy TDD network. 17.4% degradation is observed in terms of mean user throughput, and 72% in terms of 5%-tile user throughput.

Observation 7: When blocking is not modelled at SBFD and TDD BS receivers, it is possible to find an ACIR value that allows to reduce the SBFD UL mean user throughput and 5%-tile user throughput degradation. However, if the ACLR of the legacy TDD BS is fixed at 45 dB, it is not possible to achieve the needed ACIR values to reduce the degradation (i.e., 56 and 66 dB for mean and 5%-tile user throughput, respectively).

Observation 8: When blocking is modelled at SBFD and TDD BS, approximately a 3% of blocking probability is observed, in case of uniform UE distribution, due to the CLI generated by the DL of legacy TDD neighbour operator. This probability of blocking harmfully impacts the UL performance of SBFD network in such a way that even increasing the ACIR, it is not possible to reduce the degradation to an acceptable level, below 5%, with respect to the baseline.

Observation 9: Coexistence of the UL of a TDD network with the DL an SBFD network is possible in a scenario where users are uniformly distributed. In this case, the UE-to-UE CLI does not cause harmful impact against the DL of the SBFD network. However, this scenario may hide coexistence issues, because if users are uniformly distributed over a wide area, the probability that two users active in UL and DL at the same time are dropped close enough to each other to generate UE-to-UE CLI is extremely low.

Observation 10: For coexistence Case 4 is of interest to study as well the Urban Hotspot scenario: when the users are clustered, and the distance among them is reduced, it is higher the probability that users can interfere among each other. In this case, we observe that the DL mean user throughput is not affected by the coexistence with SBFD, but the 5%-tile throughput instead is, with up to 12.2% degradation with respect to the baseline. It is necessary to increase the ACIR of 6 dB with respect to the baseline, so up to 34 dB, in order to reduce the degradation to an acceptable value below 5%. Assuming the legacy TDD UE ACLR cannot be changed, it is not possible to achieve such a high value of ACIR in reality.

Observation 11: In FR2 and coexistence case 1, similarly to what observed for FR1, it is important to study both urban macro and urban hotspot scenario. The uniform distribution of UEs may hide coexistence issues. Coexistence is possible when users are uniformly distributed, but when the users are clustered it is necessary to increase the ACIR to 24 dB, and consequently the ACLR of the SBFD UE.

Observation 12: In FR2 scenarios, for coexistence case 3, when the UL of SBFD is the victim and the DL of a legacy TDD system, coexistence is extremely sensitive to the assumptions that are made. When optimistic simulation assumptions and low transmission power are considered (e.g., LoS UMa, GS 100%, TRP 30 dBm), coexistence may be possible if SBFD BS ACS can be increased with respect to current assumptions. On the other hand, when more realistic assumptions in terms of LoS and GS are considered, and a higher TRP (40 dBm) is assumed, coexistence is challenging and is not possible in reality to achieve the ACIR needed to reach a tolerable degradation level.

Based on the detailed analysis of these results, we conclude the following:

- FR1 scenarios are more challenging than FR2 for coexistence when CLI is involved, due to higher transmission power, and reduced directionality of transmissions compared to FR2.

- FR1 scenarios where UL is the victim are impacted from both mean and 5%-tile throughput perspectives, since BS-to-BS CLI is the most limiting interference component.

- DL FR1 is less vulnerable to CLI than UL. Scenarios where users are far away do not suffer from degradation of performance due to UE-to-UE CLI. However, when users are closer due to deployments that cluster them, 5%-tile throughput is impacted by coexistence.

- FR2 coexistence results in both UL and DL are extremely sensitive to simulation assumptions: if the scenario considers users deployed far away from each other (uniform distribution), the GS is optimistically set to 100%, the transmission power is low, the LoS model tends to increase the isolation is BS-BS links, as is defined in the first priority assumptions, coexistence performance is quite optimistic. However, when less optimistic assumptions are considered, coexistence becomes more challenging, so that it is important to study more scenario configurations including different options, before drawing conclusions.

- Assuming clustered UEs (hot spot scenario) we see impact for both FR1 and FR2.

# References

[1]	R4-2305924, “Offline calibration data collection”, Samsung

[2]	R4-2304093, “SBFD coexistence simulation results”, Ericsson

[3] 	R4-2307701, “TP to TR 38.858: Addition of coexistence simulation assumptions to Annex D”, Ericsson

[4]	R4-2305923, “Summary of co-existence simulation parameters”, CMCC, CableLabs, Samsung

[5]	R4-2305921, “WF for SBFD adjacent channel co-existence study”, CMCC

# 5.           Annex: Parameters

In Table 5-1 and Table 5-2, parameters considered for simulation campaign relevant for urban macro and urban hotspot scenarios, FR1 and FR2 are listed.

Table 5-1: Urban Macro/Urban hotspot scenario FR1

|  | Parameters | Scenario |
| --- | --- | --- |
| System parameters | Scenario | UMa, Hexagonal layout, 19 BS per operator, 3 sectors per site, with wrapping |
|  | ISD | 500 m |
|  | Carrier Frequency | 4 GHz |
|  | Duplex Type | Static TDD (DDDDU), SBFD (XXXXX) |
|  | Base Static TDD pattern | 80:20 DL:UL |
|  | SBFD pattern | 100% SBFD slots |
|  | Channel bandwidth | 100 MHz for STDD 80:20 MHz (DU) for SBFD |
|  | Available resource blocks | 273 for STDD 218:55 (DU) for SBFD |
|  | Switching time | DL->UL: 2OS in the D slot |
|  | Sub-Carrier spacing | 30 kHz |
|  | Number of active UEs | 1 active users in UL or DL per cell at a time |
|  | Channel model | gNB-UE: UMa TR 38.803/38.828 gNB-gNB: UMa TR 38.803/38.828 UE-UE: UMi TR 38.803/38.828 (for d>10 m), FSPL (for d<10m) |
|  | UE to BS min 2D distance | 35 m |
|  | Grid-shift | 100% |
| BS | (Mg, Ng, M, N, P) | (1,1,8,8,2) (same antenna gain) (1,1,4,8,2) (same antenna area) |
|  | Sub-array configuration | 1x1 |
|  | Max gNB Tx Power (per polarization) | 49 dBm (same antenna gain) |
|  | (dv, dh) | (0.5λ, 0.8λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element | TR 38.803 |
|  | Subarray electrical down-tilt | N/A |
|  | Mechanical down-tilt | 6 deg |
|  | Beamforming method | Frequency domain |
|  | Noise figure | 5 dB |
|  | Link level model | As per TR 38.803 |
|  | BS height | 25 m |
|  | Panel HW assumptions | Same antenna gain, same antenna area |
| UE | UE antenna | 1TX 2RX |
|  | Antenna model | isotropic |
|  | Antenna element gain | 0 dBi |
|  | Max UE TX Power | 23 dBm |
|  | UE power control | Sec. 9.1 TR36.942 |
|  | SNR target | 15 dB |
|  | Noise figure | 9 dB |
|  | Link level model | As per TR 38.803 |
|  | UE distribution outdoor/indoor | 80:20 for uniform distribution 20: 80 for clustered distribution |
|  | Clusters | Circular zones with radius 25 m, 1 cluster per cell |

Table 5-2: Urban Macro/Urban hotspot scenario FR2

|  | Parameters | Scenario |
| --- | --- | --- |
| System parameters | Scenario | UMa, Hexagonal layout, 19 BS per operator, 3 sectors per site, with wrapping |
|  | ISD | 200 m |
|  | Carrier Frequency | 30 GHz |
|  | Duplex Type | Static TDD (DDDDU), SBFD (XXXXX) |
|  | Base Static TDD pattern | 80:20 DL:UL |
|  | SBFD pattern | 100% SBFD slots |
|  | Channel bandwidth | 200 MHz for STDD 80:20 MHz (DU) for SBFD |
|  | Available resource blocks | 132 for STDD 106:26 (DU) for SBFD |
|  | Switching time | DL->UL: 2OS in the D slot |
|  | Sub-Carrier spacing | 120 kHz |
|  | Number of active UEs | 1 active users in UL or DL per cell at a time |
|  | Channel model | gNB-UE: UMa TR 38.803/38.828 gNB-gNB: UMa TR 38.803/38.828 UE-UE: UMi TR 38.803/38.828 (for d>10 m), FSPL (for d<10m) |
|  | UE to BS min 2D distance | 35 m |
|  | Grid-shift | 100% |
| BS | (Mg, Ng, M, N, P) | (1,1,16,8,2) (same antenna gain) |
|  | Sub-array configuration | 1x1 |
|  | Max gNB Tx Power (per polarization) | 49 dBm (same antenna gain) 46 dBm (same antenna area) |
|  | (dv, dh) | (0.5λ, 0.5λ) |
|  | Antenna element gain | 3 dBi |
|  | Antenna element | TR 38.803 |
|  | Subarray electrical down-tilt | N/A |
|  | Mechanical down-tilt | 6 deg |
|  | Beamforming method | Frequency domain |
|  | Noise figure | 10 dB |
|  | Link level model | As per TR 38.803 |
|  | BS height | 25 m |
|  | Panel HW assumptions | Same antenna gain |
| UE | UE antenna | 1TX 2RX |
|  | Antenna model | (1,1,2,2,2) |
|  | Antenna element gain | 5.5 dBi |
|  | Max UE TX Power | 22.4 dBm EIRP |
|  | UE power control | Sec. 9.1 TR 36.942 |
|  | SNR target | 15 dB |
|  | Noise figure | 10 dB |
|  | Link level model | As per TR 38.803 |
|  | UE distribution outdoor/indoor | 100:0 for uniform distribution 100: 0 for clustered distribution |
|  | Clusters | Circular zones with radius 25 m, 1 cluster per cell |