3GPP TSG RAN WG4 Meeting #107	R4-2307726

Incheon, KR, May 22 – May 26, 2023

Agenda item:			8.11.2.2

Source:	vivo

Title:	On RRM impact of option B-1-1 for BWP operation without restriction

Document for:	Discussion

- Introduction

In the RAN4#106-bis-e meeting, the discussions started based a new WID to complete the specification support for BWP operation without restriction in NR. The agreements and open issues are captured in the WF [1].

## Issue 3-1: Applicable of existing RLM/BFD/BM requirements for supporting Option B-1-1 <Agreement >: - When CD-SSB is outside the active BWP, the existing requirements for SSB-based RLM/BFD/BM are applicable for UE supporting option B-1-1. - FFS details of applicable condition ## Issue 3-2: L1 measurement requirements to be made applicable for supporting option B-1-1 <Agreement >: - Existing SSB based RLM/BFD/CBD/BM measurement requirements in clause 8.1, 8.5, 9.5 and 9.8 are to be made applicable for UE supporting option B-1-1. ## Issue 3-3: Clarification on existing timing requirements for supporting Option B-1-1 <Way forward >: - Proposals - Option 1: (vivo, Intel) - It is clarified in the spec that existing timing requirements for non-RedCap UE are applicable regardless of whether SSB is within active BWP or not. - Option 2: (Ericsson, Apple, Nokia, Intel, Huawei, Spreadtrum, CATT, MediaTek, OPPO) - No clarification on existing timing requirements ## Issue 3-5: Clarification/modification on inter-frequency measurement requirements for supporting option B-1-1 <Way forward >: - The issue is postponed. ## Issue 3-6: Whether to define requirements for supporting multiple options <Way forward >: - Proposals - Option 1: - Requirements/UE behaviour for UE supporting both option B-1-1 and C - Option 2: - Requirements for UE supporting both option B-1-1 and A. ## Issue 3-7: Whether to define additional requirements for supporting option B-1-1 <Way forward >: - Proposals - Option 1: - Do not introduce new UE and gNB behaviour for option B-1-1 in RAN4 spec. It is up to UE and gNB implementation how option B-1-1 is supported. - Do not introduce a new section for RRM requirements for option B-1-1. - Option 2: - Introduce a section for RRM requirement for Option B-1-1 and describe the behaviour of UE and gNB like as below. - “The Option B-1-1 supporting UE can autonomously change carrier freq. and RF BW to access CD-SSB for BM/RLM/BFD without causing any interruption. The gNB is expected to configure UE specific active BWP considering the max. supportable CHBW of the UE in the specific band.” ## Issue 3-8: Assumption on RF/BB bandwidth for UE supporting option B-1-1) <Agreement >: - Do not specify assumptions on RF BW and BB BW for UE supporting option B-1-1.

In this contribution, we further provide our views on RRM impact of option B-1-1 for BWP operation without restriction.

- Discussion

- 2.1 RLM/BFD/BM Requirements for option B-1-1

It was agreed that when CD-SSB is outside the active BWP, the existing requirements for SSB-based RLM/BFD/BM are applicable for UE supporting option B-1-1 and applicable conditions are FFS.

In existing RLM/BFD/BM requirements, the applicable conditions of SSB are specified as follows.

8.1 Radio Link Monitoring 8.1.1 Introduction The UE shall monitor the downlink radio link quality based on the reference signal configured as RLM-RS resource(s) in order to detect the downlink radio link quality of the PCell, PSCell and deactivated PSCell if configured with bfd-and-RLM with value true as specified in TS 38.213 [3]. The configured RLM-RS resources can be all SSBs, or all CSI-RSs, or a mix of SSBs and CSI-RSs. UE is not required to perform RLM outside the active DL BWP. 8.1.2 Requirements for SSB based radio link monitoring 8.1.2.1 Introduction The requirements in this clause apply for each SSB based RLM-RS resource configured for PCell, PSCell or deactivated PSCell, provided that the SSB configured for RLM is actually transmitted within UE active DL BWP during the entire evaluation period specified in clause 8.1.2.2. 8.5 Link Recovery Procedures 8.5.1 Introduction The RS resource configurations in the set on PCell, PSCell or deactivated PSCell (if configured with bfd-and-RLM with value true) can be periodic CSI-RS resources and/or SSBs. RS resource configuration in the set on SCell shall be periodic CSI-RS. UE is not required to perform beam failure detection outside the active DL BWP. 8.5.2 Requirements for SSB based beam failure detection 8.5.2.1 Introduction The requirements in this clause apply for each SSB resource in the set configured for a serving cell, provided that the SSB configured for beam failure detection is actually transmitted within the UE active DL BWP during the entire evaluation period specified in clause 8.5.2.2. The requirements in this clause could not be applicable if UE is required to perform beam failure detection on more than 1 serving cell per band. 8.5.5 Requirements for SSB based candidate beam detection 8.5.5.1 Introduction The requirements in this clause apply for each SSB resource in the set configured for a serving cell, provided that the SSBs configured for candidate beam detection are actually transmitted within UE active DL BWP during the entire evaluation period specified in clause 8.5.5.2. The requirements in this clause apply when UE is required to perform beam failure detection on no more than 1 serving cell per band. 9.5 L1-RSRP measurements for Reporting 9.5.1 Introduction When configured by the network, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP. The measurements shall be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-RSRP measurements within the active BWP. 9.8 L1-SINR measurements for Reporting 9.8.1 Introduction When configured by the network, the UE shall be able to perform L1-SINR measurements with the measurement resources configured as the selection of: - CSI-RS based CMR and no dedicated IMR configured; - SSB based CMR and dedicated IMR configured; - CSI-RS based CMR and dedicated IMR configured. The measurements shall be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-SINR measurements within the active BWP.

It can be seen that in existing RLM/BFD/CBD/BM(L1-RSRP) requirements for non-RedCap UEs, it is specified explicitly that the requirements are applicable provided SSB is within active DL BWP. For UE supporting option B-1-1 for BWP operation without restriction, the SSB is outside active BWP and the UE is able to perform L1 measurements based on the SSB outside active BWP.

Since the applicable conditions with regard to whether L1 measurement resources in within active BWP are different for the L1 RLM/BFD/CBD/BM measurements, the exact wording can be discussed during CR phase. It would be good if high level principle can be agreed which could help CR discussions.

In the RAN1 discussions on feature list [2], following features were introduced for BWP operation without restriction.

| Features | Index | Feature group |
| --- | --- | --- |
| 53. NR_BWP_wor | 53-1 | Support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions |
| 53. NR_BWP_wor | 53-2 | Support RLM/BM/BFD measurements based on CD-SSB outside active BWP with interruptions |
| 53. NR_BWP_wor | 53-3 | Support RLM/BM/BFD measurements based on NCD-SSB within active BWP |

With the new UE capability, the applicable conditions for RLM/BM/BFD measurements can be updated. The UE is required to perform RLM/BM/BFD measurements based on CD-SSB outside active BWP if UE supporting FG 53-1 and FG 53-2. Otherwise RLM/BM/BFD measurements are performed based on SSB withing active BWP.

Proposal 1: UE is required to perform RLM/BM/BFD measurements based on CD-SSB outside active BWP if the UE supporting FG 53-1 and FG 53-2. Otherwise, RLM/BM/BFD measurements is performed based on SSB within active BWP.

- 2.2. Timing requirements for option B-1-1

In existing requirements, without considering FR2-2, the applicability of timing requirements for non-RedCap UE is based on availability of SSB during 160ms.

When the UL SCS is 120 kHz or smaller, the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. ……

For UE supporting option B-1-1 for BWP operation without restriction, the UE should be able to meet the timing requirements when SSB is outside active DL BWP without interruptions. However, for UE supporting option A, UE may need to be configured with measurement gap to meet the timing requirements. It is proposed in [3] that a note is added on the factors that may impact availability of SSB. For option B-1-1, it would be enough to just clarify that SSB can be either within or outside active BWP.

Proposal 2: For supporting option B-1-1, it is clarified in the spec that existing timing requirements for non-RedCap UE are applicable regardless of whether SSB is within active BWP or not.

- 2.3 L3 measurement requirements for option B-1-1

Regarding L3 measurement requirements for option B-1-1 and option B-1-2, following agreements were made in the last RAN4 meeting.

<Agreement >: - It is a common RAN4 understanding that L3 measurements requirements impact for options B-1-1 and B-1-2 are not explicitly included in the WI objectives. - RAN4 work will focus on enabling L1 measurements for options B-1-1 and B-1-2 in Q2’2023. - In RAN4#107 RAN4 can continue discussion on whether any further clarifications are needed on intra-frequency measurements for UEs supporting options B-1-1 and B-1-2.

In general, for a UE supporting option B-1-1 for BWP operation without restriction, it can perform intra-frequency measurement without gap based on SSB outside active DL BWP. It may need further discussion as this may have impact on UE power saving. Based on UE implementation for supporting option B-1-1, e.g., using larger BW, UE may take opportunities to adjust working bandwidth from large BW to active BWP without causing interruptions when the UE is not performing RLM/BFD/BM for power saving purpose.

Although power saving may be compromised due to reduced opportunities for BW change without interruptions for UE supporting option B-1-1 to support intra-frequency measurement without gap based on SSB outside active BWP, it could bring significant system benefit in terms of overhead reduction on measurement gap. Thus, it is beneficial to support intra-frequency measurement without gap for UE supporting FG 53-1.

Proposal 3: It is beneficial to support intra-frequency measurement without gap based on SSB outside active BWP for UE supporting option B-1-1.

In existing requirements, UE is only required to perform intra-frequency SSB based measurements without measurement gaps if the SSB is completely contained in the active BWP of the UE.

The UE can perform intra-frequency SSB based measurements without measurement gaps if - the UE indicates ‘no-gap’ via intraFreq-needForGap for intra-frequency measurement, or - the SSB is completely contained in the active BWP of the UE, or - the active downlink BWP is initial BWP[3].

For UE supporting option B-1-1, the UE can perform intra-frequency SSB based measurements without measurement gaps even if the SSB is outside active BWP of the UE. Following update on existing requirements is expected.

The UE can perform intra-frequency SSB based measurements without measurement gaps if - the UE indicates ‘no-gap’ via intraFreq-needForGap for intra-frequency measurement, or - the SSB is completely contained in the active BWP of the UE, or - the active downlink BWP is initial BWP[3], or - the SSB is outside the active BWP if the UE supports FG 53-1.

Observation 1: Update on existing intra-frequency measurement requirements is expected if intra-frequency measurement without gap based on CD-SSB outside active BWP is introduced for UE supporting option B-1-1.

As discussed in [3], in addition to L3 measurements, we also see the benefit to support handover to a BWP without CD-SSB. If it is not supported, then the active BWP without CD-SSB can only be activated and switched to after handover. There would be additional delay and signalling overhead being needed.

Proposal 4: It is beneficial to support handover to a BWP without CD-SSB directly for UE supporting option A, B-1-1 and option B-1-2. Whether to support the feature needs RAN plenary guidance.

- 2.4 Others

There were discussions on whether to define requirements for UE supporting multiple options for BWP operation without restriction.

## Issue 3-6: Whether to define requirements for supporting multiple options <Way forward >: - Proposals - Option 1: - Requirements/UE behaviour for UE supporting both option B-1-1 and C - Option 2: - Requirements for UE supporting both option B-1-1 and A.

It is highly likely that UE would support both option B-1-1 and C. In deployment where NCD-SSB is configured, the UE may operate as option C for power saving. In deployment without NCD-SSB, the UE operates as option B-1-1. There is no need to specify requirements for combination of option B-1-1 and option C, i.e., the case that UE works with larger BW when NCD-SSB is configured within active BWP does not make sense.

Then the UE behaviour for supporting both option B-1-1 and C should be clarified. One simple way is that UE works with option C whenever NCD-SSB is configured within active BWP and corresponding L1 measurements are configured. Otherwise, UE works with option B-1-1. In other words, if the UE is configured with NCD-SSB within the active DL BWP, the UE works and meets the requirements based on Option C i.e., NCD-SSB for BM/RLM/BFD; Otherwise, the UE works and meets the requirements based on Option B-1-1.

Proposal 5: If UE supports both option B-1-1 and C, the UE works with option B-1-1 or option C mutually exclusively, and the UE meets the requirements based on option B-1-1 or option C for BM/RLM/BFD, respectively.

Proposal 6: if the UE is configured with NCD-SSB within the active DL BWP, the UE works and meets the requirements based on Option C i.e., NCD-SSB for BM/RLM/BFD; Otherwise, the UE works and meets the requirements based on Option B-1-1.

Supporting both option B-1-1 and option A may be possible from UE implementation perspective, and depends on measurement RS configuration (e.g., SSB or CSI-RS), UE can perform L1 measurement based on Option A or Option B-1-1.

Based on current UE feature for BWP without restriction, there is no new UE capability for option A. For legacy UE, it needs to report supporting FG 1-7, 2-24 and 2-31, which is mandatory feature with capability. Then intra-frequency measurement with gap should also be supported by the UE, which should be supported by default in our understanding. So, it seems option A can be supported by the UE by existing mandatory UE capabilities. It needs further discussion on how the UE can meet timing requirements based on CD-SSB outside active BWP.

With this understanding, it seems if a UE supports option B-1-1 by indicating FG53-1, then the UE supports both option A and B-1-1 as option A is indicated by mandatory features FG 1-7, 2-24 and 2-31. However, it is not desirable that when option A is supported in practical network, the UE still needs to work with option B-1-1 together. It brings no obvious gain by operating with the two options simultaneously.

If option A and option B-1-1 are supported simultaneously, the requirements for supporting the two options for RLM/BM/BFD measurements should be defined, i.e., applicable conditions should be clarified for the case when both CSI-RS and SSB outside active BWP are configured for RLM/BM/BFD measurements. If UE supports option A only, it only measures CSI-RSs within active BWP. If UE supports option B-1-1 only, it only measures SSB outside active BWP. If UE supports both option A and option B-1-1, then the UE measures both CSI-RS and SSB outside active BWP. The UE behavior or conditions should be defined if option C + option B-1-1 are supported simultaneously if it is supported.

Proposal 7: If UE working with option A and option B-1-1 simultaneously is supported, UE behaviour/conditions for RLM/BM/BFD measurements should be defined.

In addition, UE may support both option A and option C simultaneously. The CSI-RS and NCD-SSB within active BWP can be configured for RLM/BM/BFD. This should be reasonable configuration, which is similar to allowed configuration of CSI-RS and CD-SSB within active BWP for RLM/BM/BFD by existing specification. Thus, UE should be allowed to work with option A and option C simultaneously.

Proposal 8: UE working with option A and option C simultaneously should be supported and UE behaviour/conditions for RLM/BM/BFD measurements should be defined.

## Issue 3-7: Whether to define additional requirements for supporting option B-1-1 <Way forward >: - Proposals - Option 1: - Do not introduce new UE and gNB behaviour for option B-1-1 in RAN4 spec. It is up to UE and gNB implementation how option B-1-1 is supported. - Do not introduce a new section for RRM requirements for option B-1-1. - Option 2: - Introduce a section for RRM requirement for Option B-1-1 and describe the behaviour of UE and gNB like as below. - “The Option B-1-1 supporting UE can autonomously change carrier freq. and RF BW to access CD-SSB for BM/RLM/BFD without causing any interruption. The gNB is expected to configure UE specific active BWP considering the max. supportable CHBW of the UE in the specific band.”

Based on discussions in the last RAN4 meeting, companies agree that there is no need to introduce new UE and gNB behaviour for option B-1-1 in RAN4 spec. It is up to UE and gNB implementation how option B-1-1 is supported as long as requirements being defined for option B-1-1 is satisfied by the UE. If any specific restriction is needed, it can be discussed with corresponding requirements.

Proposal 9: Do not introduce new UE and gNB behaviour for option B-1-1 in RAN4 spec. It is up to UE and gNB implementation how option B-1-1 is supported as long as requirements defined for option B-1-1 is satisfied.

- Summary

In this contribution, we further provided our views on RRM impact of option B-1-1 for BWP operation without restriction. Based on above analysis, following proposals are present.

Proposal 1: UE is required to perform RLM/BM/BFD measurements based on CD-SSB outside active BWP if the UE supporting FG 53-1 and FG 53-2. Otherwise, RLM/BM/BFD measurements is performed based on SSB within active BWP.

Proposal 2: For supporting option B-1-1, it is clarified in the spec that existing timing requirements for non-RedCap UE are applicable regardless of whether SSB is within active BWP or not.

Proposal 3: It is beneficial to support intra-frequency measurement without gap based on SSB outside active BWP for UE supporting option B-1-1.

Observation 1: Update on existing intra-frequency measurement requirements is expected if intra-frequency measurement without gap based on CD-SSB outside active BWP is introduced for UE supporting option B-1-1.

Proposal 4: It is beneficial to support handover to a BWP without CD-SSB directly for UE supporting option A, B-1-1 and option B-1-2.

Proposal 5: If UE supports both option B-1-1 and C, the UE works with option B-1-1 or option C mutually exclusively, and the UE meets the requirements based on option B-1-1 or option C for BM/RLM/BFD, respectively.

Proposal 6: If the UE is configured with NCD-SSB within the active DL BWP, the UE works and meets the requirements based on Option C i.e., NCD-SSB for BM/RLM/BFD; Otherwise, the UE works and meets the requirements based on Option B-1-1.

Proposal 7: If UE working with option A and option B-1-1 simultaneously is supported, UE behaviour/conditions for RLM/BM/BFD measurements should be defined.

Proposal 8: UE working with option A and option C simultaneously should be supported and UE behaviour/conditions for RLM/BM/BFD measurements should be defined.

Proposal 9: Do not introduce new UE and gNB behaviour for option B-1-1 in RAN4 spec. It is up to UE and gNB implementation how option B-1-1 is supported as long as requirements defined for option B-1-1 is satisfied.

- References

- R4-2306397	WF on completion of specification support for BWP operation without restriction, vivo

- R1-2304282	Initial RAN1 UE features list for Rel-18 NR after RAN1#112bis-e	Moderators, AT&T, NTT DOCOMO, INC.

- R4-2307724	On RRM impact of option A for BWP operation without restriction, vivo