3GPP TSG-RAN WG4 Meeting#107	R4-2307739

# Incheon, South Korea, 22 – 26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | CRNum | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the channel raster |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_channel_raster_enh-Core |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Enhance the channel raster by defining what has be on the raster to allow configuration of a UE-specific channel bandwidth within a wider cell-specific channel raster for capable UEs. (The capability only needed for bands with a 100k channel raster in FR1.) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.3.1: clarify that the UE CHBW only supports part of the carrier if this CHBW is configured within a wider carrier. Clause 5.4.2.2: the carrier resource grid for a carrier supported by the RF channel and as advertised in SIB1 has to be on channel raster for at least one supported numerology for compliance with minimum requirements in 38.101-2. The configuration of a UE-specific channel bandwidth within the RF channel is described. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Still unclear as to what has to be on the channel raster: the BS bandwidth, a UE-specific bandwidth or both? |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1, 5.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

### 5.3.1	General

The UE channel bandwidth supports a single NR RF carrier or parts thereof in the uplink or downlink at the UE. From a BS perspective, different UE channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs connected to the BS. Transmission of multiple carriers to the same UE (CA) or multiple carriers to different UEs within the BS channel bandwidth can be supported.

From a UE perspective, the UE is configured with one or more BWP / carriers, each with its own UE channel bandwidth. The UE does not need to be aware of the BS channel bandwidth or how the BS allocates bandwidth to different UEs.

The placement of the UE channel bandwidth for each UE carrier is flexible but can only be completely within the BS channel bandwidth.

The relationship between the channel bandwidth, the guardband and the transmission bandwidth configuration is shown in Figure 5.3.1-1.

Figure 5.3.1-1: Definition of channel bandwidth and transmission bandwidth configuration for one NR channel

### 5.3.2	Maximum transmission bandwidth configuration

< text omitted >

#### 5.4.2.2	Channel raster to resource element mapping

The mapping between the RF reference frequency on channel raster and the frequency location of the subcarrier of the corresponding resource element of a resource grid of a carrier supported by the RF channel is given in Table 5.4.2.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL. The mapping must apply to at least one numerology supported by the UE.

Table 5.4.2.2-1: Channel raster to resource element mapping

|  | NRB mod 2 = 0 | NRB mod 2 = 1 |
| --- | --- | --- |
| Resource element index k | 0 | 6 |
| Physical resource block number nPRB |  |  |

nRB is the PRB index, k the resournce element index within this PRB and NRB ≤ 275 PRB the transmission bandwidth configuration specified in sub-clause 5.3.1 and given by carrierBandwidth indicated in downlinkConfigCommon or uplinkConfigCommon or supplementaryUplink for the relevant transmission direction and numerology as specified in [13].

A UE-specific channel bandwidth can be configured within the resource grid of a carrier supported by the RF channel in the UL and DL. For each numerology supported and transmission direction, the center frequency of a UE specific channel bandwidth is the frequency location of the subcarrier of the resource element determined according to Table 5.4.2.2-1 but with NRB the maximum transmission bandwidth configuration indicated by carrierBandwidth in ServingCellConfig.

#### 5.4.2.3	Channel raster entries for each operating band

< end of changes >