3GPP TSG-RAN WG4 Meeting #107		R4- 2307806

Incheon, Korea, 22nd – 26th May, 2023

Agenda item:	8.10.3.2

Source:	Intel Corporation

Title:	Discussion on inter-RAT measurement without gaps

Document for:	Discussion

# Introduction

Up to this meeting, all agreed using scenarios for inter-RAT NR/LTE measurements without gap can summarized as:

- the inter-RAT NR measurements without gap in Rel18 includes the two scenarios below.

  - Case a-1: UE performing the measurements without gap in NR carriers as there is vacant RF chains for UE measurements

  - Case a-2: NR reference signal to be measured are fully contained within UE’s LTE channel bandwidth

- the inter-RAT LTE measurements without gap in Rel18 includes the two scenarios below.

  - Case b-1: UE performing the measurements without gap in LTE carriers as there is vacant RF chains for UE measurements

  - Case b-2: LTE CRS are fully contained within UE’s active BWP

In this paper we provide our views on how to specify the requirements for each of the cases.

# Discussion

## RRM requirements

General

In the last meeting, we have agreed on the list of RRM impacts from the cases included in the scope as agreed further before. The agreement below is captured in the approved WF [1].

< Agreement >:

| Using scenarios | Capability indications | New RRM requirements needed | Notes |
| --- | --- | --- | --- |
| Case a-1: Inter-RAT NR wo gap because of the vacant RF chain available | “gap” | No | The existing requirements in TS36.133 8.1.2.4.21&22 can be applied |
|  | “no gap but interruption allowed” | Yes | To be defined in TS36.133 |
|  | “no gap no interruption” | Yes. | To be defined in TS36.133 |
| Case b-1: Inter-RAT LTE wo gap because of the vacant RF chain available | “gap” | No | The existing requirements in TS38.133 9.4.2&9.4.3 can be applied |
|  | “ncsg” | No. | the existing requirements in TS38.133 9.4.2&9.4.3 can be reused. |
|  | “nogap-noncsg” | Yes | To be defined in TS38.133 |
| Case b-2: Inter-RAT LTE wo gap because the measurement reference signal can be contained within UE’s active BWP | “gap”[TBD] | No | The existing requirements in TS38.133 9.4.2&9.4.3 can be applied |
|  | “no gap but interruption allowed” [TBD] | TBC (Depending on issue 2-2-2) |  |
|  | “no gap” [TBD] | Yes | To be defined in TS38.133 |

In our views, UE’s behaviour for the inter-RAT NR measurement is similar as these for UE support ‘NeedForGaps’. For an example, when UE performs the inter-RAT NR measurement without gap, the following two cases are considered:

- No gap and with interruption (nogap-withint)

- Neither gap nor interruption (nogap-noint)

Hereby we need to clarify for inter-RAT NR measurement case a-1, whether does RAN4 define the requirements for both of them above?

Observation 1: For the inter-RAT NR measurement case a-1 the requirements can be different when UE reports different capabilities among the two (nogap-withint and nogap-noint).

In current stage, we believe the case when UE supports ‘nogap-noint’ shall be defined at least. Thus, we can propose that:

Proposal 1: Prioritize the requirement for case a-1 without gap and no allowed interruption.

## Case a-1 requirements

Case a-1 requirements

Regarding the requirements of inter-RAT NR measurements without gap (case a-1), the frameworks of LTE TDD and FDD clauses can be reused while the requirements should be based on NR measurement periods. For example,

- The requirements shall be differentiated for TDD and FDD

- And the requirements can be

  - Tidentify_irat_without_index = (TPSS/SSS_sync_irat + TSSB_measurement_period_irat) ms

  - Tidentify_irat_with_index = (TPSS/SSS_sync_irat + TSSB_measurement_period_irat + TSSB_time_index_irat) ms

- Plus, we remove “MGRP” from the formulation of the measurement cycle.

Therefore, we propose that:

Proposal 2: For the inter-RAT NR measurements without gap (case a-1), the requirements can be defined as:

- The requirements shall be differentiated for TDD and FDD.

- And the requirements can be

  - Tidentify_irat_without_index = (TPSS/SSS_sync_irat + TSSB_measurement_period_irat) ms

  - Tidentify_irat_with_index = (TPSS/SSS_sync_irat + TSSB_measurement_period_irat + TSSB_time_index_irat) ms

- For the specific the requirement, e.g.

Table x-1: Time period for PSS/SSS detection (Frequency range FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_irat |
| --- | --- |
| No DRX | Max(600ms, 8 SMTC period) Nfreq |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(81.5) Max(SMTC period, DRX cycle)) Nfreq |
| DRX cycle > 320ms | 8 DRX cycle Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 5. |  |

Table x-5: Measurement period for inter-RAT measurements (Frequency range FR1)

| Condition NOTE1,2 | TSSB_measurement_period_irat |
| --- | --- |
| No DRX | Max(200ms, 8 SMTC period) Nfreq |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5) Max(SMTC period, DRX cycle)) Nfreq |
| DRX cycle > 320ms | 8 DRX cycle Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 5. |  |

## LS to RAN2

In the last meeting, a draft LS was generated and discussed but it was not sent since the consensus in the group was to send the LS in this meeting and include mature agreements to guide RAN2 on signalling design for inter-RAT measurement without gap.

In the annex of this paper we pasted the draft version we had in the last meeting. Let’s use it as the baseline in this meeting to generate a finer version for RAN2.

Proposal 3: Send an LS to RAN2 about signalling design for inter-RAT measurement without gap case a-1, b-1 and b-2.

# Conclusion

Observation 1: For the inter-RAT NR measurement case a-1 the requirements can be different when UE reports different capabilities among the two (nogap-withint and nogap-noint).

Proposal 1: Prioritize the requirement for case a-1 without gap and no allowed interruption.

Proposal 2: For the inter-RAT NR measurements without gap (case a-1), the requirements can be defined as:

- The requirements shall be differentiated for TDD and FDD.

- And the requirements can be

  - Tidentify_irat_without_index = (TPSS/SSS_sync_irat + TSSB_measurement_period_irat) ms

  - Tidentify_irat_with_index = (TPSS/SSS_sync_irat + TSSB_measurement_period_irat + TSSB_time_index_irat) ms

- For the specific the requirement, e.g.

Table x-1: Time period for PSS/SSS detection (Frequency range FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_irat |
| --- | --- |
| No DRX | Max(600ms, 8 SMTC period) Nfreq |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(81.5) Max(SMTC period, DRX cycle)) Nfreq |
| DRX cycle > 320ms | 8 DRX cycle Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 5. |  |

Table x-5: Measurement period for inter-RAT measurements (Frequency range FR1)

| Condition NOTE1,2 | TSSB_measurement_period_irat |
| --- | --- |
| No DRX | Max(200ms, 8 SMTC period) Nfreq |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5) Max(SMTC period, DRX cycle)) Nfreq |
| DRX cycle > 320ms | 8 DRX cycle Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 5. |  |

Proposal 3: Send an LS to RAN2 about signalling design for inter-RAT measurement without gap case a-1, b-1 and b-2.

# References

- R4-2306331 WF on NR_MG_enh2 Part 2, Intel Corporation

# Annex

- Overall description

In R18 measurement gap enhancement WI (NR_MG_enh2-Core), RAN4 discussed RRM requirements for measurement without gaps for the following cases:

- Inter-RAT measurements without gaps [RAN4] - Inter-RAT NR measurements - Inter-RAT LTE measurement

RAN4 classified inter-RAT measurements without gaps to the three subcases below.

  - Case a-1: inter-RAT NR measurements without gap as there is vacant RF chains for UE measurements

  - Case b-1: inter-RAT LTE measurements without gap as there is vacant RF chains for UE measurements

  - Case b-2: inter-RAT LTE measurements without gap when LTE CRS to be measured are fully contained within UE’s active BWP

RAN4 has reached the following agreements below.

< Agreements in R4#106bis-e>: - For case b-1 - Reuse NeedForNCSG-InfoEUTRA-r17 to support Case b-1 - Define requirements for case when UE reports “nogap-noncsg” in NeedForNCSG-InfoEUTRA-r17 for indicating no-gap without interruption. - For case b-2 - A new per-UE capability to support Case b-2 should be defined.

< Agreements in R4#107>: TBA if any

For Case b-1, RAN4 agreed no need to introduce any new Rel-18 signalling from UE but reuse to existing NeedForNCSG-InfoEUTRAN in TS38.331 v17.1.0 support Case b-1.

For Case b-2, RAN4 agreed to introduce new Rel-18 signalling from UE to indicate whether UE can perform the inter-RAT LTE measurements without gap if LTE CRS is completely located in active BWP or not.

RAN4 would like to respectively ask RAN2 to take the above information into account when designing corresponding RRC signalling in R18.

- Actions

To RAN WG2

ACTION: 	RAN4 kindly asks RAN2 to take the above information into account and design Rel18 signalling for measurements without gap.