3GPP TSG-RAN WG4 Meeting #107	R4-2307823

Incheon, KR, May 22 – May 26, 2023

Source:	LG Electronics

Title:	Discussion on Rel-18 RF requirement for NES

Agenda item:	8.35.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on Rel-18 RF requirement for NES

Discussion

In the last meeting following WFs were agreed [1] in RF session.

Issue 2-1-1: Priorities for feasibility study - Agreement： - RAN4 prioritize the discussion on SSB-less Scell operation for inter-band CA for FR1 and co-located cell. Issue 2-2-6: How to define the RF requirements for inter-band CA for SSB-less Scell operation - Agreement: - No impact to the legacy requirements of inter-band CA TAE defined in TS38.104. Whether or not to define feature-specific TAE requirements for SSB-less operation (NW enabling energy savings) for FR1 co-located inter-band CA is FFS.

In the last meeting following WFs were agreed [2] in RRM session.

< Agreement >Issue 2-1-2: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-1? Online session (Monday April 24, 2023) Agreements - Conditions for SSB-less inter-band CA SCell operation for scenario-1 - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell - Option 1-1: 260ns - Option 1-2: 3us - Option 1-3: smaller than 3us - Other values are not precluded - The difference of the reception power with the FR1 inter-band active serving cell - Option 2-1: 6dB - Option 2-2: 6dB+offset - Other values are not precluded - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell) - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations. - FFS if requirements will be defined subject to feasibility analysis - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a < Agreement >Issue 2-1-3: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-2? Agreements - Conditions for SSB-less inter-band CA SCell operation for scenario-2 - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell - Option 1-1: 260ns - Option 1-2: 3us - Option 1-3: smaller than 3us - Other values are not precluded - The difference of the reception power with the FR1 inter-band active serving cell - Option 2-1: 6dB - Option 2-2: 6dB+offset - Other values are not precluded - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell) - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations. - FFS if requirements will be defined subject to feasibility analysis - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a < Agreement >Issue 2-1-4: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-2a? Agreements - Conditions for SSB-less inter-band CA SCell operation for scenario-2a - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell - Option 1-1: 260ns - Option 1-2: 3us - Option 1-3: smaller than 3us - Other values are not precluded - The difference of the reception power with the FR1 inter-band active serving cell - Option 2-1: 6dB - Option 2-2: 6dB+offset - Other values are not precluded - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell) - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations. - FFS if requirements will be defined subject to feasibility analysis - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a

As listed issues in above, inter-band SSB-less SCell operation was discussed in RRM sessions. Some companies think that feasible frequency bands for inter-band CA with respect to frequency separation can be discussed in RF sessions. We also think this issue can be discussed in the RF session. Also, the issue on the difference of the reception power listed in above can be discussed together with frequency separation since pathloss difference between FR1 PCell and SSB-less SCell due to center frequency difference can affect the reception power difference

Proposal 1: Frequency separation between carriers in inter-band CA can be discussed together with reception power difference in RF session.

Conclusion

We provide our views on Rel-18 RF requirement for NES, we propose

Proposal 1: Frequency separation between carriers in inter-band CA can be discussed together with reception power difference in RF session.

Reference

[1] R4-2306639, “WF on RF feasibility study of NR Network Energy Saving”

[2] R4-2306369, “WF on RRM requirements for NR network energy saving”