3GPP TSG-RAN WG4 Meeting #107                                                            R4-2307835

Incheon, Korea, 22 – 26 May 2023

Agenda Item:	8.14.5.2

Source:	Ericsson

Title:	Simulation results for ATG PDSCH demodulation requirements

Document for:	Information

# 1	Introduction

After last RAN4 #106bis-e meeting’s discussion online, there were several parameter configurations and assumptions agreed. A way forward was approved to carry all the agreements and open issues [1].

In this contribution, we shared our simulation results based on the agreed parameter configurations and assumptions for information.

# 2	Discussion

## 2.1 Assumptions

In this section, we summarized the simulation assumptions that we used for our evaluations.

- AWGN + Doppler for the channel model

- koffset value = 2ms

- rank = 1

- DMRS: 1 front loaded + 1 additional

- TDD pattern:

  - For test 2-9, new TDD pattern: 30D4S6U

  - For rest of other tests: Normal TDD pattern: 7D1S2U

## 2.2 Simulation result

We’ve evaluated the following test cases and summarized the results.

Table 2.2-1 Simulation cases and results

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| FDD |  |  |  |  |  |  |
| 1-1 | 10 / 15 | 13 (Table-1) | AWGN + 220Hz Doppler | 2x2 | 70 | 0.36 |
| 1-2 | 20 / 15 | 13 (Table-1) | AWGN + 220Hz Doppler | 2x2 | 70 | 0.39 |
| 1-3 | 10 / 15 | 19 (Table-1) | AWGN + 220Hz Doppler | 2x2 | 70 | 5.00 |
| 1-4 | 20 / 15 | 19 (Table-1) | AWGN + 220Hz Doppler | 2x2 | 70 | 4.56 |
| 1-5 | 10 / 15 | 20 (Table-2) | AWGN + 220Hz Doppler | 2x2 | 70 | 12.49 |
| 1-6 | 20 / 15 | 20 (Table-2) | AWGN + 220Hz Doppler | 2x2 | 70 | 12.41 |
| 1-7 | 10 / 15 | 27 (Table-2) | AWGN + 220Hz Doppler | 2x2 | 70 | 19.51 |
| 1-8 | 20 / 15 | 27 (Table-2) | AWGN + 220Hz Doppler | 2x2 | 70 | 19.53 |
| TDD |  |  |  |  |  |  |
| 2-1 | 40 / 30 | 13 (Table-1) | AWGN + 500Hz Doppler | 2x2 | 70 | 0.39 |
| 2-2 | 100 / 30 | 13 (Table-1) | AWGN + 500Hz Doppler | 2x2 | 70 | 0.41 |
| 2-3 | 40 / 30 | 19 (Table-1) | AWGN + 500Hz Doppler | 2x2 | 70 | 4.57 |
| 2-4 | 100 / 30 | 19 (Table-1) | AWGN + 500Hz Doppler | 2x2 | 70 | 4.51 |
| 2-5 | 40 / 30 | 20 (Table-2) | AWGN + 500Hz Doppler | 2x2 | 70 | 12.41 |
| 2-6 | 100 / 30 | 20 (Table-2) | AWGN + 500Hz Doppler | 2x2 | 70 | 12.44 |
| 2-7 | 40 / 30 | 27 (Table-2) | AWGN + 500Hz Doppler | 2x2 | 70 | 19.53 |
| 2-8 | 100 / 30 | 27 (Table-2) | AWGN + 500Hz Doppler | 2x2 | 70 | 19.68 |
| Verifying the new TDD pattern |  |  |  |  |  |  |
| 2-9 | 40 / 30 | 13 (Table-1) | AWGN + 500Hz Doppler | 2x2 | 70 | 0.41 |

Note: New TDD pattern is configured for case 2-9 to compare with the performance of 2-1

# 3	Summary

In this contribution, we shared our simulation results based on the parameter configurations and assumptions agreed in the last RAN4 #106bis-e meeting.

# References

[1]	R4-2305899: WF for ATG demodulation requirements, CMCC, RAN4 #106bis-e