# 3GPP TSG- RAN WG4 Meeting # 107	R4-2307845

# Incheon, Korea, 22th May–26th May, 2023

# Source: 	ZTE Corporation

# Title: 	Simulation results for FR1 dynamic range and ICS requirements

# Agenda item:	8.15.4

# Document for:	Approval

# 1 Introduction

In the RAN4 #106bis meeting, there are some further discussion on added FRC parameters for REFSENS, ICS and dynamic range and some updates are made for BS RF FRC after discussion [1]. In this contribution, we want to share some simulation results for FR1 dynamic range and ICS requirements.

# 2 Discussion

  - 2.1 Simulation assumptions

Following FRC assumptions were agreed in last meeting.

Table 2.1: Added FRC parameters for FR1 in-channel selectivity

| Reference channel | G-FR1-A1-20 |
| --- | --- |
| Subcarrier spacing (kHz) | 15 |
| Allocated resource blocks | 9 |
| CP-OFDM Symbols per slot (Note 1) | 12 |
| Modulation | QPSK |
| Code rate (Note 2) | 1/3 |
| Payload size (bits) | [808] |
| Transport block CRC (bits) | 16 |
| Code block CRC size (bits) | - |
| Number of code blocks - C | 1 |
| Code block size including CRC (bits) (Note 3) | [824] |
| Total number of bits per slot | [2592] |
| Total symbols per slot | [1296] |
| NOTE 1: UL-DMRS-config-type = 1 with UL-DMRS-max-len = 1, UL-DMRS-add-pos = 1 with l0 = 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity and in-channel selectivity. NOTE 3: Code block size including CRC (bits) equals to K’ in sub-clause 5.2.2 of TS 38.212 |  |

Table 2.2: Added FRC parameters for FR1 dynamic range

| Reference channel | G-FR1-A2-15 |
| --- | --- |
| Subcarrier spacing (kHz) | 15 |
| Allocated resource blocks | 15 |
| CP-OFDM Symbols per slot (Note 1) | 12 |
| Modulation | 16QAM |
| Code rate (Note 2) | 2/3 |
| Payload size (bits) | [5504] |
| Transport block CRC (bits) | [24] |
| Code block CRC size (bits) | - |
| Number of code blocks - C | 1 |
| Code block size including CRC (bits) (Note 3) | [5528] |
| Total number of bits per slot | [8640] |
| Total symbols per slot | [2160] |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS, additional DM-RS position = pos1 with l0 = 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211. NOTE 2: MCS index 16 and target coding rate = 658/1024 are adopted to calculate payload size. NOTE 3: Code block size including CRC (bits) equals to K’ in sub-clause 5.2.2 of TS 38.212 . |  |

  - 2.2 Simulation results

Following Figure 2.1 shows simulation results for FR1 ICS SNR, and the simulation results are summarized in the Table 2.3.

Figure 2.1: Simulation results for G-FR1-A1-20

Table2.3 Summarized simulation results for FR1 ICS SNR

|  | PRBs | SCS (kHz) | SNR@95% throughput (dB) |
| --- | --- | --- | --- |
| G-FR1-A1-20 | 9 | 15 | -0.8 |

Following Figure 2.2 shows simulation results for FR1 dynamic range SNR, and the simulation results are summarized in the Table 2.4.

Figure 2.2: Simulation results for G-FR1-A2-15

Table 2.4 Summarized simulation results for FR1 dynamic range SNR

|  | CBW (MHz) | PRBs | SCS (kHz) | SNR@95% throughput (dB) |
| --- | --- | --- | --- | --- |
| G-FR1-A2-15 | 3 | 15 | 15 | 8.8 |

# 3 Conclusion

In this contribution, we share some updated simulation results for FR1 ICS and dynamic range requirement and the proposals are made as following.

Observation 1: the updated simulation results for ICS requirement is shown as following table:

|  | PRBs | SCS (kHz) | SNR@95% throughput (dB) |
| --- | --- | --- | --- |
| G-FR1-A1-20 | 9 | 15 | -0.8 |

Observation 2: the updated simulation results for dynamic range requirement is shown as following table:

|  | CBW (MHz) | PRBs | SCS (kHz) | SNR@95% throughput (dB) |
| --- | --- | --- | --- | --- |
| G-FR1-A2-15 | 3 | 15 | 15 | 8.8 |

# 4 Reference

- R4-2305900, “WF on FRC parameters for link level simulation for added FRC”, ZTE Corporation