3GPP TSG-RAN WG4 # 107	R4-2307852

Incheon, South Korea, May 22 – May 26, 2023

Agenda item:	8.8.4.2

Source:	MediaTek Inc.

Title:	Discussion on PDSCH requirements of FR2 Multi-Rx DL Demod

Document for:	Discussion

# Introduction

In RAN4#106-bis-e, the Way Forward document was approved [1]. We provide discussion of PDSCH requirements with proposals for the Rel-18 work in this contribution.

# Discussion

In this chapter we will discuss remaining open items of PDSCH requirements in Chapter 2.1 and PDSCH rate matching RAN1 specification clarification in Chapter 2.2.

# 2.1 Open items

In this chapter we discuss remaining open items of PDSCH requirements of FR2 Multi-Rx DL Demod.

Issue 2-2-1: Channel model

<Way forward >

- Proposals

  - Option 1: Consider including TDLA30-300 for 120kHz/200MHz requirements, if introduced

  - Option 2: Consider TDLA30-300 for 100 MHz/120 kHz.

  - Option 3: Consider TDLA30-75 for 100MHz/120KHz

We think lower doppler scenarios are more practical for FR2. Therefore, we support Option 3.

Proposal #1: We support Option 3.

Issue 2-1-5: PTRS Port

<Way forward >

- Proposals

  - Option 1: One PTRS port per TRP for both sDCI and mDCI schemes

  - Option 2: One port PTRS for s-DCI SDM scheme

As already mentioned by other companies in the previous meeting, in TS38.306 UE capability specification [2] there is parameter called supportTwoPortDL-PTRS-r16 that indicates whether UE supports 2-port DL PT-RS in Single-DCI based multi-TRP SDM transmission scheme. To make requirements applicable for most of the UEs we should not use test configuration dependent on optional UE capability. Therefore, we support Option 2 for minimum requirements.

Observation #1: Two port PTRS for Single-DCI SDM scheme is optional feature for UE.

Proposal #2: We support Option 2. Two port PTRS for Single-DCI SDM scheme is optional feature for UE and we should define minimum requirements.

Issue 2-2-7: Time offset between TRPs

<Way forward >

- Proposals

  - Option 1: Use the time offset between the two TRPs transmission points as {0.25us, -0.0625us}.

  - Option 2: Use time offset of TRP1 with respect to TRP2 as {0.2us, -0.05us} for different test cases.

  - Option 3: For NR FR2-1 multi-Rx chain DL reception, consider sDCI and mDCI based SDM scheme with timing offset between TRxPs as follow

    - Timing offset of the second TRxP from the first TRxP [us] as -0.0625 or 0.25.

  - Option 4: Select following value for timing offset of the second TRxP from the first TRxP for demodulation cases.

|  |  | Timing offset[us] |
| --- | --- | --- |
| Multi-DCI based multi-TRP | non-overlapping | -0.0625 |
|  | full-overlapping | -0.0625 |
| Single-DCI based multi-TRP | SDM | 0.25 |

Proposed options are almost the same. We support Option 1 and are open to merge options for compromise if needed.

Proposal #3: We support Option 1.

Issue 2-2-8: Frequency offset between TRPs

<Way forward >

- Proposals

- Option 1: Select 3000Hz frequency offset for all demodulation cases as starting point.

- Option 2: 600Hz

- Option 3: 400Hz

We propose using smaller frequency offset as a starting point. We can check later if larger offset would be really needed for any reason.

Proposal #4: We support Option 2 and Option 3.

# 2.2 PDSCH rate matching

In this chapter we are discussing PDSCH rate matching in mTRP transmission scheme with PT-RS. We want to check if RAN1 specification is clear enough for aligned implementation assumptions. Is it clear that the PTRS from one TRP should be non-overlapping with the PDSCH data from the other TRP?

In TS 38.214 [3] DM-RS port allocation in mTRP multi-DCI of fully or partially overlapping PDSCHs is defined as

If a UE is configured by the higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, the UE may be scheduled with fully or partially overlapping PDSCHs in the time and frequency domain by multiple PDCCHs with the following restrictions,

-	the UE is not expected to assume different DM-RS configuration with respect to the actual number of front-loaded DM-RS symbol(s), the actual number of additional DM-RS symbol(s), the actual DM-RS symbol location, and DM-RS configuration type.

-	the UE is not expected to assume DM-RS ports in a CDM group indicated by two TCI states.

Observation #1: In mTRP multi-DCI fully and partial overlapping transmission schemes TRPs are using different CDM group for PDSCH and DM-RS.

In TS 38.211 [4] PDSCH RE allocation is defined as

The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria:

-	they are in the virtual resource blocks assigned for transmission;

-	the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214];

-	the corresponding resource elements in the corresponding physical resource blocks are

-	not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2;

-	not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by TRS-ResourceSet IE, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource;

-	not used for PT-RS according to clause 7.4.1.2;

-	not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214].

Observation #2: PDSCH RE allocation is not overlapping with PT-RS allocation.

In TS 38.211 [4] PT-RS RE allocation is defined as

For the purpose of PT-RS mapping, the resource blocks allocated for PDSCH transmission are numbered from 0 to  from the lowest scheduled resource block to the highest. The corresponding subcarriers in this set of resource blocks are numbered in increasing order starting from the lowest frequency from 0 to . The subcarriers to which the UE shall assume the PT-RS is mapped are given by

where

-	i=0,1,2,…

-	 is given by Table 7.4.1.2.2-1 for the DM-RS port associated with the PT-RS port according to clause 5.1.6.3 in [6, TS 38.214]. If the higher-layer parameter resourceElementOffset in the PTRS-DownlinkConfig IE is not configured, the column corresponding to 'offset00' shall be used.

-	 is the RNTI associated with the DCI scheduling the transmission

-	 is the number of resource blocks scheduled

-	KPT-RS∈2,4 is given by [6, TS 38.214].

Table 7.4.1.2.2-1: The parameter .

| DM-RS antenna port |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DM-RS Configuration type 1 |  |  |  | DM-RS Configuration type 2 |  |  |  |
|  | resourceElementOffset |  |  |  | resourceElementOffset |  |  |  |
|  | offset00 | offset01 | offset10 | offset11 | offset00 | offset01 | offset10 | offset11 |
| 1000 | 0 | 2 | 6 | 8 | 0 | 1 | 6 | 7 |
| 1001 | 2 | 4 | 8 | 10 | 1 | 6 | 7 | 0 |
| 1002 | 1 | 3 | 7 | 9 | 2 | 3 | 8 | 9 |
| 1003 | 3 | 5 | 9 | 11 | 3 | 8 | 9 | 2 |
| 1004 | - | - | - | - | 4 | 5 | 10 | 11 |
| 1005 | - | - | - | - | 5 | 10 | 11 | 4 |

Observation #3: In DM-RS Configuration type 1 PT-RS RE allocation in CDM group 0 uses even RE offsets, and in CDM group 1 uses odd RE offsets.

From previous observations we can see that in mTRP transmission scheme PT-RS is allocated in different frequency domain locations per TRP. Also, it is defined that PT-RS allocation does not overlap with PDSCH allocation. Now we are wondering if definition of “PT-RS allocation does not overlap with PDSCH allocation” in mTRP transmission scheme is valid per TRP or for all TRPs. We would like companies to confirm their understanding in Proposal #5. Option 1 and Option 2 are also illustrated in Table 2-1.

Proposal #5: Companies confirm their understanding of PDSCH rate matching in mTRP transmission scheme with PT-RS

- Option 1: PT-RS allocation does not overlap with PDSCH allocation per TRP

- Option 2: PT-RS allocation does not overlap with any PDSCH allocation

Table 2-1: PDSCH data matching options for mTRP with PT-RS

|  | Option 1 |  | Option 2 |  |
| --- | --- | --- | --- | --- |
| Freq. alloc. | TRP#1 | TRP#2 | TRP#1 | TRP#2 |
| RE11 | PDSCH#1 | PDSCH#2 | PDSCH#1 | PDSCH#2 |
| RE10 | PDSCH#1 | PDSCH#2 | PDSCH#1 | PDSCH#2 |
| RE9 | PDSCH#1 | PDSCH#2 | PDSCH#1 | PDSCH#2 |
| RE8 | PDSCH#1 | PDSCH#2 | PDSCH#1 | PDSCH#2 |
| RE7 | PDSCH#1 | PDSCH#2 | PDSCH#1 | PDSCH#2 |
| RE6 | PDSCH#1 | PDSCH#2 | PDSCH#1 | PDSCH#2 |
| RE5 | PDSCH#1 | PDSCH#2 | PDSCH#1 | PDSCH#2 |
| RE4 | PDSCH#1 | PDSCH#2 | PDSCH#1 | PDSCH#2 |
| RE3 | PDSCH#1 | PTRS#2 | DTX | PTRS#2 |
| RE2 | PTRS#1 | PDSCH#2 | PTRS#1 | DTX |
| RE1 | PDSCH#1 | PDSCH#2 | PDSCH#1 | PDSCH#2 |
| RE0 | PDSCH#1 | PDSCH#2 | PDSCH#1 | PDSCH#2 |

We think that if there are any controversial views on Proposal #5 or the RAN1 specification is ambiguous, RAN4 should consider sending LS to RAN1 for specification clarification request to guarantee interoperability.

Proposal #6: If there are any controversial views on Proposal #5 or the RAN1 specification is ambiguous, consider sending LS to RAN1 for specification clarification request

# Conclusion

In this paper we provided the view on the PDSCH requirements of FR2 multipanel RX downlink demodulation requirements. The following observations and proposals are made:

Proposal #1: We support Option 3.

Observation #1: Two port PTRS for Single-DCI SDM scheme is optional feature for UE.

Proposal #2: We support Option 2. Two port PTRS for Single-DCI SDM scheme is optional feature for UE and we should define minimum requirements.

Proposal #3: We support Option 1.

Proposal #4: We support Option 2 and Option 3.

Observation #1: In mTRP multi-DCI fully and partial overlapping transmission schemes TRPs are using different CDM group for PDSCH and DM-RS.

Observation #2: PDSCH RE allocation is not overlapping with PT-RS allocation.

Observation #3: PT-RS RE allocation in CDM group 0 uses even RE offsets, and in CDM group 1 uses odd RE offsets.

Proposal #5: Companies confirm their understanding of PDSCH rate matching in mTRP transmission scheme with PT-RS

Proposal #6: If there are any controversial views on Proposal #5 or the RAN1 specification is ambiguous, consider sending LS to RAN1 for specification clarification request

# Reference

- R4-2306000, “WF for UE demodulation and CSI requirements of Rel-18 NR FR2 multi-Rx chain DL reception”, Qualcomm

- 3GPP TS 38.306 V17.4.0 (2023-03), “User Equipment (UE) radio access capabilities (Release 17)”

- 3GPP TS 38.214 V17.5.0 (2023-03), “Physical layer procedures for data (Release 17)”

- 3GPP TS 38.211 V17.4.0 (2022-12), “Physical channels and modulation (Release 17)”