3GPP TSG-RAN WG4 # 107	R4-2307854

Incheon, South Korea, May 22 – May 26, 2023

Agenda item:	8.12.4

Source:	MediaTek Inc.

Title:	Discussion on UE demodulation requirements for intra-band non-collocated EN-DC/NR-CA deployment scenario

Document for:	Discussion

# Introduction

In RAN4#106-bis-e, the Way Forward document for demodulation requirements was approved [1]. Demodulation discussions are dependent on RF agreements. Also, in RAN4#106-bis-e, the Way Forward document for RF requirements was approved [2]. We provide discussion with proposals for the Rel-18 work in this contribution.

# Discussion

In this chapter we discuss some remaining open items.

Issue 1-2-1: UE type assumption

Agreement:

- Discuss the UE demodulation requirements for Type 2 UE first.

  - Test scope covers both EN-DC and NR-CA

- Not define the UE demodulation requirements for Type 3 UE.

- For UE demodulation requirements for Type 4 UE, depending on the progress of RF discussion.

The Way Forward document for RF requirements from RAN4#106-bis-e has following agreements

- RAN4 suggests to postpone Type 3a/3b to future release.

- RAN4 suggests to postpone Type 4a/4b to future release.

Therefore, it is not feasible to define demodulation requirements to Type 3a/3b and Type 4a/4b UEs.

Proposal #1: Do not define demodulation requirements to Type 3a/3b and Type 4a/4b UEs.

Issue 1-2-2: Test scope

Whether to define the corresponding UE demodulation requirements for Type 2 UE

Way forward:

- Option 1: Define new UE demodulation requirements for Type 2 UE for intra-band non-contiguous EN-DC/NR-CA deployment scenario.

  - Interested companies assume the maximum received time difference value (33us) and maximum power imbalance assumption (25dB).

- Option 2: Not define new UE demodulation requirements for Type 2 UE for intra-band non-contiguous EN-DC/NR-CA deployment scenario.

In 3GPP TS 38.101-3 V17.9.0 (2023-03) in Chapter 7.6B.2.6 there is RF in-band blocking test for Type 2 UE called “Power imbalance for type 2 UE of inter-band EN-DC with overlapping DL bands”. This test has 25dB power imbalance between carriers and test metric is data throughput. To pass the test UE must achieve ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes. [3]

Observation #1: There exist RF tests for Type2 UE of EN-DC where data throughput is measured 1dB above reference sensitivity level with another carrier having 25dB higher power.

Observation #2: There does not exist RF tests for Type2 UE of NR-CA.

Proposal #1: We support not to define new UE demodulation requirements for Type 2 UE for intra-band non-contiguous EN-DC deployment scenario. There already exist RF tests for Type2 UE of EN-DC with data throughput test metric.

Proposal #2: We support to define new UE demodulation requirements for Type 2 UE for intra-band non-contiguous NR-CA deployment scenario.

# Conclusion

In this paper we provided the view on the UE demodulation requirements for intra-band non-collocated EN-DC/NR-CA deployment scenario. The following observations and proposals are made:

Observation #1: There exist RF tests for Type2 UE of EN-DC where data throughput is measured 1dB above reference sensitivity level with another carrier having 25dB higher power.

Observation #2: There does not exist RF tests for Type2 UE of NR-CA.

Proposal #1: We support not to define new UE demodulation requirements for Type 2 UE for intra-band non-contiguous EN-DC deployment scenario. There already exist RF tests for Type2 UE of EN-DC with data throughput test metric.

Proposal #2: We support to define new UE demodulation requirements for Type 2 UE for intra-band non-contiguous NR-CA deployment scenario.

# Reference

- R4-2305891, “WF on UE demodulation requirements for intra-band non-collocated EN-DC/NR-CA deployment scenario”, Ericsson

- R4-2306605, “WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment”, KDDI

- 3GPP TS 38.101-3 V17.9.0 (2023-03), “User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios (Release 17)”