3GPP TSG-RAN WG4 # 107	R4-2307855

Incheon, South Korea, May 22 – May 26, 2023

Agenda item:	8.19.1.1

Source:	MediaTek Inc.

Title:	Discussion on Advanced receiver to cancel inter-user interference for MU-MIMO

Document for:	Discussion

# Introduction

In RAN#99, the status report of Rel-18 WI of NR demodulation performance evolution, the work objective for advanced receiver to cancel inter-user interference for MU-MIMO is defined [1].  The work objective is to evaluate and specify advanced receiver to cancel inter-user interference for MU-MIMO. This work is split into two phases where the first phase studies the performance gain, reference receiver assumption, interference modelling, testability, required signalling overhead, as well as impact on other WGs. The initial receiver candidates are E-MMSE-IRC and R-ML. In the second phase it is expected to specify PDSCH demodulation requirements under MU-MIMO scenario with advanced receiver.

# Discussion

# 2.1 Background

New test cases of PDSCH with intra-cell inter-user interference were introduced in Rel-17 test specification [2]. These requirements were defined assuming MMSE-IRC receiver to mitigate co-scheduled UE interference. For Rel-18 the work objective is to evaluate and specify advanced receiver to cancel inter-user interference for MU-MIMO for improved performance over Rel-17 baseline. In the previous meeting some agreements were already achieved to initiate the study phase simulations. In the following Chapter 2.2 we will discuss agreed receiver assumptions for study phase. Finally in the Chapter 2.3 we will discuss assistant information related issues.

# 2.2 Advanced receiver options

To improve receiver performance from Rel-17 MMSE-IRC solution, new advanced receiver to cancel inter-user interference for MU-MIMO is proposed to be introduced in Rel-18. The initial receiver candidates are E-MMSE-IRC and R-ML.

Reduced complexity Maximum Likelihood detection (R-ML) receiver assumption was discussed in the previous meeting and the following 4 options were notified as listed in Issue 1-1-1.

Issue 1-1-1: Reference receiver assumption for R-ML

Candidate options:

  - Option 1: UE perform RML algorithm for serving and all co-scheduled UEs in the cell

  - Option 2: R-ML receiver in terms of total layer (serving + interfering) and modulation order

  - Option 3: UE performs joint detection on layers of one additional co-scheduled UE in addition to its own layers on the same frequency and time resource as its own allocation

  - Option 4: Limit the number of co-scheduled UE is no more than 1 and the number of interference layers are no more than 2

Way forward

  - This issue is highly related to how UE could obtain each requirement information and how NWA is designed.

  - Discuss how to obtain each of the needed parameters.

We think it would be good starting point to assume UE can perform R-ML for serving and all co-scheduled UE layers. However, as will be discussed later in Issues 1-2-3-1 and 1-2-3-2 the modulation order of co-scheduled UE needs to be known by UE by assistant information. For simplicity, it would be better in a case of multiple co-scheduled UEs, the same modulation order would be used to limit signalling overhead, otherwise UE may need to fallback to E-MMSE-IRC receiver assumption.

Proposal #1: UE perform R-ML algorithm for serving and all co-scheduled UEs with known modulation order in the cell.

Issue 1-1-2: Reference receiver

Candidate options:

  - Option 1: Down-select R-ML as a candidate reference receiver to define phase II requirements

  - Option 2: To be decided later

Way forward

  - To be discussed in the next meeting.

We think this is related to modulation order of co-scheduled UE network assistant signalling agreements. Therefore, it is better to make this decision later after NWA has been agreed.

Proposal #2: We support Option 2. A candidate reference receiver to define phase II requirements can be decided later when NWA has been agreed.

# 2.3 Assistant information discussion

One goal in work item description is to find if any assistant information is needed for advanced receiver. There are several open issues related to assistant information identified in the last meeting listed in WF [3]. These issues are discussed in this chapter.

Issue 1-2-2-1: The presence of co-scheduled UE

- Proposals on how this information could be obtained by the UE:

  - Option 1: Blind detection should be considered

  - Option 2: By assistant information signalling

  - Option 3: Through signalling of other parameters or by blind detection to the DMRS port information to be discussed

- Proposals on how the NWA is signalled (if introduced):

  - Option 1: DCI-based signalling jointly with modulation order and DMRS port

  - Option 2: DCI-based signalling jointly with only modulation order

  - Option 3: 1-bit RRC signalling to indicate whether the DMRS port is used for the co-scheduled UE

Way forward

  - Postpone this issue after the agreements of other information are reached.

Related to Issue 1-2-2-3 of co-scheduled UE DMRS port allocation and Issue 1-2-2-8 of co-scheduled UE frequency domain allocation, we believe the blind detection of those properties is possible with reasonable accuracy of ports and PRBs with notable signal power, also indicating the presence of co-scheduled UEs. However, the big problem of using only blind detection are scenarios when any co-scheduled UEs are not present, and all blind detection would cause unnecessary UE processing and power consumption. Therefore, some indication of the presence of co-scheduled UEs needs to be signalled to UE that can be implicit or explicit depending on other assistant information.

Observation #1: Blind detection of the presence of MU-MIMO transmission would cause unnecessary UE processing and power consumption in scenarios when MU-MIMO transmission is not used.

Proposal #3: We support Option 2. UE needs to know the presence of MU-MIMO transmission by assistant information signalling as in Proposal 2 in Issue 1-2-3-2.

Issue 1-2-2-2: The DMRS sequence information for the co-scheduled UE

Way forward

  - For the DMRS configuration parameter including: DMRS type, DMRS additional position, maximum length:

    - Restriction already exists in RAN1 specification (TS38.214), thus signaling is not needed.

  - For the scrambling ID and nSCID information:

    - Assume same as that of the target UE agreed as RAN4 default assumption

    - FFS on the signaling to inform UE if RAN4 default assumption not valid for any of the co-scheduled UE:

- Option 1: 1-bit RRC signaling

- Option 2: Implied by DCI signaling on modulation order (if introduced)

- Option 2A: If “no co-schedule UEs are presented in the allocated resource to the target UE” is signaled in DCI, combine this information in the same signaling without additional bits

In Issue 1-2-2-2 we think DMRS sequence of co-scheduled UEs needs to be known by UE and it cannot be blindly detected. Therefore, we see only Options 1 and 2 as applicable in practice. We consider Option 2 as more practical deployment.

Proposal #4: We support Option 2. UE assumes the DMRS sequences for all co-scheduled UEs are always the same with that of the target UE when presence of co-scheduled UE is signalled.

Issue 1-2-2-3: The DMRS port information for the co-scheduled UE

GTW agreements on Apr 17th:

  - Dedicated DCI signaling is not preferred for the DMRS port information

  - FFS whether assistant RRC signalling can be introduced to reduce the BD complexity and/or maintain reasonable CE performance for target UE

  - Companies are encouraged to further evaluate BD performance including the detailed assumption:

    - Number of co-scheduled UE for BD

    - Time/frequency location of co-scheduled UE

Way forward

  - Study the BD performance together with FDRA information as recommended in Issue 1-2-2-8.

  - FFS whether to introduce additional assistant RRC signalling to restrict the BD complexity.

UE needs know the DMRS port information of the co-scheduled UEs to perform channel estimation and interference cancellation for used DMRS ports. We believe the blind detection of this information is possible with reasonable accuracy of ports with notable signal power. However, as discussed in Issue 1-2-2-1 it is important to inform UE the presence of co-scheduled UE to avoid unnecessary UE processing and power consumption.

Proposal #5: UE is assumed to perform blind detection of DMRS port information of the co-scheduled UEs when UE gets indication of the presence of MU-MIMO transmission.

Proposal #6: Introduce additional and optional assistant RRC signalling to restrict the BD complexity by limiting search space of DMRS ports.

Issue 1-2-2-4: PRB bundling size for the co-scheduled UE

Way forward

  - UE needs to know the PRB bundling size of co-scheduled UEs if different from target UE

  - How could be obtained

    - Assume the PRB bundling size of co-scheduled UEs is same with that of target UE

    - FFS on the signaling to inform UE if RAN4 default assumption not valid:

- Option 1: 1-bit RRC signaling

- Option 2: No signaling is required.

- Option 3: Implied by DCI signaling on modulation order (if introduced)

For optimal channel estimation performance, it would be beneficial for UE to know precoding granularity of the co-scheduled UE. Also, we are bit suspicious if pre-coding granularity can be reliably blindly detected. Therefore, we suggest UE can assume the precoding granularity to be the same as own granularity. Alternatively, UE may need to fallback to assume the smallest granularity with some performance penalty.

Proposal #7: We support Option 3. If UE cannot assume the pre-coding granularity of co-scheduled UEs to be the same as own granularity, then fallback to regular MMSE-IRC.

Issue 1-2-2-5: DMRS power boosting for the co-scheduled UE

Way forward

  - DMRS power boosting should be the same for both target and the co-scheduled UE.

  - FFS on the signaling to inform UE if RAN4 default assumption not valid:

    - Option 1: 1-bit RRC signaling

    - Option 2: Implied by DCI signaling on MO (if introduced)

- Option 2A: If “no co-schedule UEs are presented in the allocated resource to the target UE” is signaled in DCI, combine this information in the same signaling without additional bits

    - Option 3: No signaling is required.

We think DMRS power boosting should be the same for both target and the co-scheduled UE and this can be implicitly confirmed with DCI signalling.

Proposal #8: We support Option 2. If UE cannot assume DMRS power boosting should be the same for both target and the co-scheduled UE, then fallback to regular MMSE-IRC.

Issue 1-2-2-7: Time domain resource allocation information of the co-scheduled UE

Way forward

  - UE assumes the same PDSCH symbols are allocated to the target and the co-scheduled UEs

  - FFS on the signaling to inform UE if RAN4 default assumption not valid, by:

    - Option 1: 1-bit RRC signalling

    - Option 2: Implied by DCI signaling on MO (if introduced)

- Option 2A: If “no co-schedule UEs are presented in the allocated resource to the target UE” is signaled in DCI, combine this information in the same signaling without additional bits

    - Option 3: No signaling is required

It would make possible scenarios complicated if different time domain resource allocation is allowed for co-scheduled UEs. Therefore, we clearly prefer Option 2 to let UE assume the same time domain allocation.

Proposal #9: We support Option 2. If UE cannot assume the same PDSCH symbols are allocated to the target and the co-scheduled UEs, then fallback to regular MMSE-IRC.

Issue 1-2-2-8: Frequency domain resource allocation information of the co-scheduled UE

Way forward

  - UE should know the FDRA information of the co-scheduled UE

  - Study the R-ML performance with BD to the DMRS port information as well as the FDRA information of the Co-UE (Low priority for the May meeting). Evaluation assumptions are as below:

    - 1 Co-UE

    - Detection granularity – up to UE implementation

    - Select the following two cases based on the agreed simulation assumption:

- Case 2 (rank 1+1 2T2R QPSK interference TDLC300-100 random precoding)

- Case 9 (rank 2+2 4T4R 64QAM interference TDLA30-10 orthogonal precoding)

    - Two cases for the FDRA of the co-UE:

- Full CHBW allocation (52PRBs)

- Partial CHBW allocation (0~24 PRBs)

    - Note: Assume that the R-ML has known all the other required information and all the agreed default assumptions are valid

  - Companies are encouraged to bring simulation results.

UE needs know the frequency domain resource allocation of the co-scheduled UEs to perform channel estimation and interference cancellation for used resource in frequency domain. We believe the blind detection of this information is possible with reasonable accuracy of resources in frequency domain with notable signal power. However, as discussed in Issue 1-2-2-1 it is important to inform UE the presence of co-scheduled UE to avoid unnecessary UE processing and power consumption.

Proposal #10: UE is assumed to perform blind detection of frequency domain resource allocation of the co-scheduled UEs when UE gets indication of the presence of MU-MIMO transmission.

Issue 1-2-3-1: The modulation order information of the co-scheduled UE

Way forward

- The following additional assumptions to the R-ML receiver can be agreed:

  - Within each PRB/PRG, UE applies R-ML to all interference layers with prior information that all interference layers have same modulation order

  - FFS whether to consider the case with interference layers have different modulation orders within one or more PRBs.

- Evaluation assumptions of the MO BD study:

  - 1 Co-UE

  - Detection granularity – up to UE implementation

  - Following cases:

    - Rank 1+1, 2T2R, MCS 13 for the target UE, QPSK interference, TDLC300-100, random precoding

    - Rank 2+2, 4T4R, MCS 17 for the target UE, 16QAM interference, TDLA30-10, orthogonal precoding

    - Rank 1+1, 2T2R, MCS 13 for the target UE, 16QAM interference TDLC300-100 random precoding (Optional)

  - Full CHBW allocation (52PRBs) FDRA of the co-UE:

  - Note: Assume that the R-ML also needs to perform DMRS port and FDRA information BD and all the agreed default assumptions are valid.

  - Companies are encouraged to bring simulation results for the next meeting.

- With this MO BD study, the following is not precluded:

  - The possibility of full signalling of modulation order and/or other information.

  - The possibility of non-dynamic NWA signalling (i.e., non-DCI) solutions.

- For this MO BD study, companies are encouraged to take all proposals from Issue 1-2-3-2 into consideration.

UE needs to know the modulation order information of the co-scheduled UE to perform the joint demodulation of own and co-scheduled UE PDSCH layers. We believe the blind detection of modulation order of co-scheduled UE would be unreliable in many cases and the wrong assumption of modulation order would cause performance loss. Also, modulation order blind detection increases UE processing. Therefore, to guarantee robustness and reasonable complexity of advanced receiver operation we should introduce assistant information for co-scheduled UE modulation order. UE would need to fallback to E-MMSE-IRC type of receiver if there is not information available for modulation order of co-scheduled UE.

Proposal #11: UE with R-ML needs to know the modulation order information for each co-scheduled layer by assistant information signalling.

Issue 1-2-3-2: Content of the network signalling on modulation order (for information)

- Proposal 1:

| Signalling Overhead |  | DCI signalling Info. | Antenna Port = 1000 + DMRS Port (P) |
| --- | --- | --- | --- |
| 2 bits | maxMIMO-Layers = 2 | 00: No interference presence 01: Interference with QPSK 10: Interference with 16QAM 11: Interference with 64QAM or 256 QAM | P = {0, 1} |
| 6 bits | maxMIMO-Layers = 4 | For 3 interfering ports in cyclic order from desired layer, each 2 bits represent as 00: No interference presence 01: Interference with QPSK 10: Interference with 16QAM 11: Interference with 64QAM or 256 QAM | P = {0, 1, 2, 3} |

- Proposal 2: Use examples listed in below as start point

  - Example 1:

| Bitfield | coUE modulation order | UE behavior |
| --- | --- | --- |
| 000 | No UE which has same DMRS sequence with target paired | SU-MIMO (MMSE-IRC) |
| 001 | PRB aligned (Single coUE modulation per PRB) | UE fallback to MMSE-IRC or R-ML with blind detection with only 4 hypotheses (UE dependent) |
| 010 | Bandwidth aligned, QPSK | R-ML without modulation order detection |
| 011 | Bandwidth aligned, 16-QAM | R-ML without modulation order detection |
| 100 | Bandwidth aligned, 64-QAM | R-ML without modulation order detection |
| 101 | Bandwidth aligned, 256-QAM | R-ML without modulation order detection |
| 110 | Bandwidth aligned, 1024-QAM | R-ML without modulation order detection |
| 111 | PRB not aligned (Multiple coUE modulations per PRB) | UE fallback to MMSE-IRC or R-ML with blind detection (UE dependent) |

- Proposal 3: Introduce the RRC based signaling to inform UE the MCS index table(s) used for PDSCH of the co-scheduled UEs, or signal the maximum MCS index table (64QAM, 256QAM or 1024QAM table) used among all the (potential) co-scheduled UEs.

- Proposal 4: Include blind detection opportunities for modulation order detection using semi-static DUT resource allocation, where DUT is not scheduled with PDSCH, but co-scheduled UEs are scheduled with PDSCH.

- Proposal 5: Introduce DCI signaling to assist blind detection, such as 1 bit modulation order set information for each co-scheduled UE to reduce search space, for example, when gNB decide 16QAM for a co-scheduled UE, then gNB transmit 1 bit to indicate the modulation order of this co-scheduled UE is in the set {16QAM, 64QAM}

Way forward

  - To be discussed in the next meeting if introduced

As we support Proposal 2, we would like to clarify description and terminology of this proposal.

“Bandwidth aligned” means that all co-scheduled UEs overlapping in frequency domain over the whole allocated bandwidth with target UE are using the same modulation order. In this case this modulation order is signalled, and no blind detection of modulation order is needed.

“PRB aligned” means that all co-scheduled UEs overlapping in frequency domain per PRB with target UE are using the same modulation order. In this case this modulation order is not signalled, and blind detection of modulation order is needed if UE wants to use R-ML receiver. In this case blind detection of modulation order per PRB is less complex because the same modulation order can be assumed to all allocated ports.

“PRB not aligned” means that all co-scheduled UEs overlapping in frequency domain per PRB with target UE are not using the same modulation order. In this case this modulation order is not signalled, and blind detection of modulation order is needed if UE wants to use R-ML receiver. In this case blind detection of modulation order per PRB is full complex because the same modulation order cannot be assumed to all allocated ports.

As already explained in Issue 1-2-3-1, we think that modulation order needs to be signalled to UE to avoid unnecessary blind detection in most of the cases. This requirement is best fulfilled by Proposal 2.

Proposal #12: We support Proposal 2. This is the only proposal to help UE to avoid unnecessary blind detection in most of the cases.

Issue 1-2-3-3: RS location information of the co-scheduled UE

Way forward

- UE can assume the target PDSCH is not overlapped with the CSI-RS of the co-scheduled UE

- FFS whether to consider RRC signalling to inform UE whether the default assumption is needed

We believe it is practical deployment to configure NZP-CSI-RS and ZP-CSI-RS in a cell such a way that NZP-CSI-RS is not overlapping and being interfered by PDSCH either way.

Proposal #13: We support implicit signalling in DCI with MO. If UE cannot assume the target PDSCH is not overlapped with the CSI-RS of the co-scheduled UE, then fallback to regular MMSE-IRC.

Issue 1-2-4-1: Signalling for the network assistant information (If introduced)

Candidate options:

- Option 1: Only consider RRC or MAC-CE based network assistance signalling

- Option 2: DCI

- Option 3: FFS once it is agreed which information is to be signalled

- Option 4: Some of the information could be carried by DCI and others carried by higher layer

Way forward

- Discuss separately for each parameter

The UE needs to know the presence of the co-scheduled UE and the modulation order information of the co-scheduled UE to perform the joint demodulation of own and co-scheduled UE PDSCH layers. This information can change from slot to slot to allow network flexibility in scheduling. We see it difficult for network to use slow signalling options listed in Option 1 to guarantee up to date information for UE. The network assistant information for modulation order and advanced receiver activation needs to be done with DCI. In addition, RRC is needed to activate DCI signalling and possibly help to limit DMRS port allocation blind detection complexity. Furthermore, DCI signalling needs to be carefully designed to minimize number of bits needed.

Proposal #14: We support Option 4. The network assistant information for modulation order and advanced receiver activation needs to be done with DCI. In addition, RRC is needed to activate DCI signalling and possibly help to limit DMRS port allocation blind detection complexity.

Issue 1-2-4-2: Granularity of the network assistant signalling (If introduced)

Candidate options:

- Option 1: The granularity of the network assistant signalling should be the wideband

- Option 2: FFS until it is agreed which information is to be signalled

Way forward

- Further discuss.

As we in Issue 1-2-4-1 are suggesting using DCI for assistant information we must minimize number of bits needed. Therefore, we see information granularity would be better to limit to whole bandwidth.

Proposal #15: Granularity of the network assistant signalling should be for the whole bandwidth of serving UE considering the overhead limitation.

# Conclusion

In this paper we provided the view on the advanced receiver to cancel inter-user interference for MU-MIMO. The following observations and proposals are made:

Proposal #1: UE perform R-ML algorithm for serving and all co-scheduled UEs with known modulation order in the cell.

Proposal #2: We support Option 2. A candidate reference receiver to define phase II requirements can be decided later when NWA has been agreed.

Observation #1: Blind detection of the presence of MU-MIMO transmission would cause unnecessary UE processing and power consumption in scenarios when MU-MIMO transmission is not used.

Proposal #3: We support Option 2. UE needs to know the presence of MU-MIMO transmission by assistant information signalling as in Proposal 2 in Issue 1-2-3-2.

Proposal #4: We support Option 2. UE assumes the DMRS sequences for all co-scheduled UEs are always the same with that of the target UE when presence of co-scheduled UE is signalled.

Proposal #5: UE is assumed to perform blind detection of DMRS port information of the co-scheduled UEs when UE gets indication of the presence of MU-MIMO transmission.

Proposal #6: Introduce additional and optional assistant RRC signalling to restrict the BD complexity by limiting search space of DMRS ports.

Proposal #7: We support Option 3. If UE cannot assume the pre-coding granularity of co-scheduled UEs to be the same as own granularity, then fallback to regular MMSE-IRC.

Proposal #8: We support Option 2. If UE cannot assume DMRS power boosting should be the same for both target and the co-scheduled UE, then fallback to regular MMSE-IRC.

Proposal #9: We support Option 2. If UE cannot assume the same PDSCH symbols are allocated to the target and the co-scheduled UEs, then fallback to regular MMSE-IRC.

Proposal #10: UE is assumed to perform blind detection of frequency domain resource allocation of the co-scheduled UEs when UE gets indication of the presence of MU-MIMO transmission.

Proposal #11: UE with R-ML needs to know the modulation order information for each co-scheduled layer by assistant information signalling.

Proposal #12: We support Proposal 2. This is the only proposal to help UE to avoid unnecessary blind detection in most of the cases.

Proposal #13: We support implicit signalling in DCI with MO. If UE cannot assume the target PDSCH is not overlapped with the CSI-RS of the co-scheduled UE, then fallback to regular MMSE-IRC.

Proposal #14: We support Option 4. The network assistant information for modulation order and advanced receiver activation needs to be done with DCI. In addition, RRC is needed to activate DCI signalling and possibly help to limit DMRS port allocation blind detection complexity.

Proposal #15: Granularity of the network assistant signalling should be for the whole bandwidth of serving UE considering the overhead limitation.

# Reference

- RP-230313, “Status report for NR demodulation requirement evolution”, China Telecom, Intel Corporation

- 3GPP TS 38.101-4 V17.8.0 (2023-03), “User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 17)”

- R4-2305914, “WF on advanced receiver for MU-MIMO scenario”, China Telecom