3GPP TSG-RAN WG4 # 107	R4-2307856

Incheon, South Korea, May 22 – May 26, 2023

Agenda item:	8.19.1.2

Source:	MediaTek Inc.

Title:	Simulation results of Advanced receiver to cancel inter-user interference for MU-MIMO

Document for:	Discussion

# Introduction

In RAN#99, the status report of Rel-18 WI of NR demodulation performance evolution, the work objective for advanced receiver to cancel inter-user interference for MU-MIMO is defined [1].  The work objective is to evaluate and specify advanced receiver to cancel inter-user interference for MU-MIMO. This work is split into two phases where the first phase studies the performance gain, reference receiver assumption, interference modelling, testability, required signalling overhead, as well as impact on other WGs. The initial receiver candidates are E-MMSE-IRC and R-ML. In the second phase it is expected to specify PDSCH demodulation requirements under MU-MIMO scenario with advanced receiver.

# Simulation parameters and results

In this chapter we are going to discuss simulation parameters in Chapter 2.1, simulation results in Chapter 2.2 and simulation analysis in Chapter 2.3.

# 2.1 Simulation parameters

In this chapter we are going to discuss simulation assumptions from WF [2] and show simulation results with short analysis.

Issue 2-1-1: The number of co-scheduled UEs

Candidate options:

  - Option 1: 1 co-scheduled UE as higher priority

  - Option 2: Cover 2 co-scheduled UEs at least in phase I study

Way forward

  - Keep the previous meeting agreement and continue discussion in the next meeting.

Issue 2-1-2: Rank allocation for the target and co-scheduled UEs, with 1 co-scheduled UE

Way forward

  - For 2Rx: rank 1+1

  - For 4Rx: Only consider rank 2+2

Issue 2-1-3: Rank allocation for the target and co-scheduled UEs, with 2 co-scheduled UEs

Candidate options:

  - Option 1: Rank 1+1+1

Way forward

  - Maintain the last meeting agreement: Option 1 for interested companies to bring evaluation results in initial study stage

In Issues 2-1-1, 2-1-2 and 2-1-3 it was agreed that companies bring simulation results with 1 co-scheduled UE. In addition, it was agreed that interested companies can bring evaluation results with more than 1 co-scheduled UE. In NWA discussion we are suggesting limiting to single modulation order in assistant information for simplified signalling, and in a case of multiple co-scheduled UEs with different modulation orders, UE could fallback to E-MMSE-IRC type of receiver. We would like to have this discussion before running simulations for this configuration.

Issue 2-1-4: MCS for the target UE

Way forward

  - Cover MCS 13 for rank 1 and rank 2 for initial simulation

  - Cover MCS 17 for rank 2 for initial simulation

  - Further discuss whether to cover MCS 4 for rank 1 in the next meeting

  - The assumption can be updated later based on available results.

Issue 2-1-5: Modulation order for the co-scheduled UE

Way forward

  - Maintain last meeting status from the modulation order perspective:

For R-ML, E-IRC and IRC (baseline in Rel-17, for performance comparison purpose) for initial simulation

    - For rank 1+1: QPSK (high priority)

    - For rank 2+2: 64QAM (high priority)

    - For rank 2+2: QPSK (high priority)

    - For rank 1+1 (64QAM) +1 (QPSK) (lower priority)

    - For rank 1+1 (64QAM) +1 (16QAM) (lowest priority)

    - Other options on the modulation order for co-scheduled UE are not precluded.

    - These assumptions can be updated in the next meeting based on available simulation results.

As agreed in Issue 2-1-4, we will provide simulation results of 2Rx UE in MCS13 1+1 scenario and 4Rx UE in MCS13 2+2 and MCS17 2+2 scenarios. Also, we are assuming the same scrambling ID for DMRS sequence for all UEs.

For Modulation order of the co-scheduled UE, we have simulated all options of QPSK, 16-QAM and 64-QAM to give more complete summary of results, including combinations listed in Issue 2-1-5. As mentioned earlier, we have simulated only single co-scheduled UE scenarios for this meeting.

Issue 2-1-6: Channel model

Way forward

  - For initial study

    - TDLC300-100 when the rank of the target UE is 1

    - TDLA30-10 when the rank of the target UE is 2

Issue 2-1-7: Antenna correlation

Way forward

  - Rank 1+1: ULA medium

  - Rank 2+2: ULA Low

  - Rank 1+1+1: XPL medium (Interested companies can bring simulation results)

Issue 2-1-8: Precoder selection target and co-scheduled UEs

Way forward

  - For phase I study

    - For rank 1+1: Random PMI selection

    - For rank 2+2: Orthogonal PMI selection

Channel model and antenna correlation follow Issue 2-1-6 and 2-1-7 guidance except we extended simulation scope of rank 1+1 and 2+2 scenarios to include also TDLC300-100 with low correlation. For precoder selection guidance in Issue 2-1-8, we extended simulation scope to do both random and orthogonal precoding options for all scenarios to give more complete summary of results.

Issue 2-1-9: PDSCH resource allocation for the target and co-scheduled UE

Candidate options:

  - Candidate scenarios:

    - Scenario 1: Maximum transmission bandwidth configuration for all UEs.

    - Scenario 2: Maximum transmission bandwidth configuration for the target UE and partial transmission bandwidth configuration for the co-scheduled UEs.

    - Full OFDM symbol allocation for both scenarios.

  - Option 1: Cover both scenario 1 and scenario 2 in the phase I study

    - Option 1A: Consider the following scenario for the study of R-ML and E-IRC with blind detection to the required information

- Target UE with frequency domain full PRB allocation (52PRBs)

- Co-scheduled UE1 with PRB0~19 allocation with 16QAM transmission.

- Co-scheduled UE2 with PRB40~51 allocation with QPSK transmission.

- Full OFDM symbol allocation.

  - Option 1B:

| Allocation | Layer 1 | Layer 2 | Layer 3 | Layer 4 |
| --- | --- | --- | --- | --- |
| PRB 0 ~ PRB19 | Target UE |  | UE3 64QAM | UE1 16QAM |
| PRB20 ~ PRB39 |  |  |  | None |
| PRB40 ~ PRB51 |  |  | UE2 QPSK |  |

  - Option 1C

- Target UE with frequency domain full PRB allocation (52PRBs)

- Co-scheduled UE1 with PRB0~26 allocation.

  - Option 2: Only consider scenario 1

  - Option 3: FFS based on the conclusion for Issue 1-2-2-8

Way forward

  - For initial simulation in phase I, cover scenario 1

  - FFS whether to cover scenario 2, based on the discussion outcome in the NWA part.

As proposed in Option 2 in Issue 2-1-9, we have assumed full bandwidth allocation of all UEs as defined in Scenario 1.

We have assumed the same QCL assumption in all DMRS ports. We have assumed all needed co-scheduled UE information as known. The evaluation metric is SNR at 70% of maximum throughput.

In this document we compare performance of 3 receiver types as agreed in WF [2], namely

- MMSE-IRC (Rel-17 reference receiver)

- E-MMSE-IRC (Rel-18 advanced receiver)

- R-ML (Rel-18 advanced receiver)

# 2.2 Simulation results

Simulation configuration of target UE rank 1 using MCS13 with co-scheduled UE rank 1 simulations in 2RX are shown in Table 2-1. The corresponding simulation results with random orthogonal precoding are shown in Table 2-2 and Figure 2-1. Furthermore, simulation results with random non-orthogonal precoding are shown in Table 2-3 and Figure 2-2.

Simulation configuration of target UE rank 2 using MCS13 with co-scheduled UE rank 2 simulations in 4RX are shown in Table 2-4. The corresponding simulation results with random orthogonal precoding are shown in Table 2-5 and Figure 2-3. Furthermore, simulation results with random non-orthogonal precoding are shown in Table 2-6 and Figure 2-4.

Simulation configuration of target UE rank 2 using MCS17 with co-scheduled UE rank 2 simulations in 4RX are shown in Table 2-7. The corresponding simulation results with random orthogonal precoding are shown in Table 2-8 and Figure 2-5. Furthermore, simulation results with random non-orthogonal precoding are shown in Table 2-9 and Figure 2-6.

Table 2-1: Test configurations for MU-MIMO with Rank1 target UE using MCS13 and Rank1 co-scheduled UE in 2RX.

| Test | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Co-scheduled UE precoder | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Target UE Rank1 CoUE Rank1 | FDD: 10/15 | 16QAM, 0.48 | Random QPSK, 16QAM or 64QAM symbols | Orthogonal/ Non-orthogonal | TDLC300-100 | 2 x 2 ULA Low or ULA Medium | 70 | Test dependent |

Table 2-2: Simulation results for MU-MIMO with Rank1 target UE using MCS13 and Rank1 co-scheduled UE with orthogonal precoding.

| Orthogonal precoding | R-ML | E-MMSE-IRC | MMSE-IRC | Gain of R-ML | Gain of E-MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| coUE:QPSK TDLC-Low | 11.0dB | 12.7dB | 13.4dB | 2.4dB | 0.7dB |
| coUE:16-QAM TDLC-Low | 12.3dB | 12.7dB | 13.4dB | 1.1dB | 0.7dB |
| coUE:64-QAM TDLC-Low | 12.5dB | 12.7dB | 13.4dB | 0.9dB | 0.7dB |
| coUE:QPSK TDLC-Medium | 14.8dB | 20.4dB | 20.8dB | 6.0dB | 0.4dB |
| coUE:16-QAM TDLC-Medium | 17.7dB | 20.4dB | 20.8dB | 3.1dB | 0.4dB |
| coUE:64-QAM TDLC-Medium | 19.1dB | 20.4dB | 20.8dB | 1.7dB | 0.4dB |

Figure 2-1: Simulation results for MU-MIMO with Rank1 target UE using MCS13 and Rank1 co-scheduled UE with orthogonal precoding.

| TDLC300-100 Low correlation | TDLC300-100 Medium correlation |
| --- | --- |

Table 2-3: Simulation results for MU-MIMO with Rank1 target UE using MCS13 and Rank1 co-scheduled UE with non-orthogonal precoding.

| Non-orthogonal precoding | R-ML | E-MMSE-IRC | MMSE-IRC | Gain of R-ML | Gain of E-MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| coUE:QPSK TDLC-Low | 12.0dB | 14.8dB | 15.9dB | 3.9dB | 1.1dB |
| coUE:16-QAM TDLC-Low | 14.0dB | 14.8dB | 15.9dB | 1.9dB | 1.1dB |
| coUE:64-QAM TDLC-Low | 14.3dB | 14.8dB | 15.9dB | 1.6dB | 1.1dB |
| coUE:QPSK TDLC-Medium | 15.9dB | 23.1dB | 23.8dB | 7.9dB | 0.7dB |
| coUE:16-QAM TDLC-Medium | 19.5dB | 23.1dB | 23.8dB | 4.3dB | 0.7dB |
| coUE:64-QAM TDLC-Medium | 21.3dB | 23.1dB | 23.8dB | 2.5dB | 0.7dB |

Figure 2-2: Simulation results for MU-MIMO with Rank1 target UE using MCS13 and Rank1 co-scheduled UE with non-orthogonal precoding.

| TDLC300-100 ULA Low correlation | TDLC300-100 ULA Medium correlation |
| --- | --- |

Table 2-4: Test configurations for MU-MIMO with Rank2 target UE using MCS13 and Rank2 co-scheduled UE in 4RX.

| Test | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Co-scheduled UE precoder | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Target UE Rank2 + CoUE Rank2 | FDD: 10/15 | 16QAM, 0.48 | Random QPSK, 16QAM or 64QAM symbols | Orthogonal/ Non-orthogonal | TDLA30-10 or TDLC300-100 | 4 x 4 ULA Low | 70 | Test dependent |

Table 2-5: Simulation results for MU-MIMO with Rank2 target UE and Rank2 co-scheduled UE with orthogonal precoding.

| Orthogonal precoding | R-ML | E-MMSE-IRC | MMSE-IRC | Gain of R-ML | Gain of E-MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| coUE:QPSK TDLA-Low | 11.0dB | 12.4dB | 12.4dB | 1.4dB | 0.0dB |
| coUE:16-QAM TDLA-Low | 12.1dB | 12.4dB | 12.4dB | 0.3dB | 0.0dB |
| coUE:64-QAM TDLA-Low | 12.3dB | 12.4dB | 12.4dB | 0.1dB | 0.0dB |
| coUE:QPSK TDLC-Low | 13.1dB | 14.5dB | 16.0dB | 2.9dB | 1.5dB |
| coUE:16-QAM TDLC-Low | 14.3dB | 14.5dB | 16.0dB | 1.7dB | 1.5dB |
| coUE:64-QAM TDLC-Low | 14.4dB | 14.5dB | 16.0dB | 1.6dB | 1.5dB |

Figure 2-3: Simulation results for MU-MIMO with Rank2 target UE using MCS13 and Rank2 co-scheduled UE with orthogonal precoding.

| TDLA30-10 ULA Low correlation | TDLC300-100 ULA Low correlation |
| --- | --- |

Table 2-6: Simulation results for MU-MIMO with Rank2 target UE using MCS13 and Rank2 co-scheduled UE with non-orthogonal precoding.

| Non-orthogonal precoding | R-ML | E-MMSE-IRC | MMSE-IRC | Gain of R-ML | Gain of E-MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| coUE:QPSK TDLA-Low | 13.5dB | 16.1dB | 16.1dB | 3.0dB | 0.0dB |
| coUE:16-QAM TDLA-Low | 15.3dB | 16.1dB | 16.1dB | 1.1dB | 0.0dB |
| coUE:64-QAM TDLA-Low | 15.9dB | 16.1dB | 16.1dB | 0.3dB | 0.0dB |
| coUE:QPSK TDLC-Low | 15.8dB | 19.3dB | 26.6dB | 10.8dB | 7.3dB |
| coUE:16-QAM TDLC-Low | 18.0dB | 19.3dB | 26.6dB | 8.6dB | 7.3dB |
| coUE:64-QAM TDLC-Low | 18.7dB | 19.3dB | 26.6dB | 7.9dB | 7.3dB |

Figure 2-4: Simulation results for MU-MIMO with Rank2 target UE using MCS13 and Rank2 co-scheduled UE with non-orthogonal precoding.

| TDLA30-10 ULA Low correlation | TDLC300-100 ULA Low correlation |
| --- | --- |

Table 2-7: Test configurations for MU-MIMO with Rank2 target UE using MCS17 and Rank2 co-scheduled UE in 4RX.

| Test | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Co-scheduled UE precoder | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Target UE Rank2 + CoUE Rank2 | FDD: 10/15 | 64QAM, 0.43 | Random QPSK, 16QAM or 64QAM symbols | Orthogonal/ Non-orthogonal | TDLA30-10 or TDLC300-100 | 4 x 4 ULA Low | 70 | Test dependent |

Table 2-8: Simulation results for MU-MIMO with Rank2 target UE using MCS17 and Rank2 co-scheduled UE with orthogonal precoding.

| Orthogonal precoding | R-ML | E-MMSE-IRC | MMSE-IRC | Gain of R-ML | Gain of E-MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| coUE:QPSK TDLA-Low | 15.2dB | 16.6dB | 16.6dB | 1.4dB | 0.0dB |
| coUE:16-QAM TDLA-Low | 16.5dB | 16.6dB | 16.6dB | 0.1dB | 0.0dB |
| coUE:64-QAM TDLA-Low | 17.0dB | 16.6dB | 16.6dB | -0.4dB | 0.0dB |
| coUE:QPSK TDLC-Low | 18.0dB | 19.9dB | 31.8dB | 13.8dB | 11.9dB |
| coUE:16-QAM TDLC-Low | 19.7dB | 19.9dB | 31.8dB | 12.1dB | 11.9dB |
| coUE:64-QAM TDLC-Low | 20.4dB | 19.9dB | 31.8dB | 11.4dB | 11.9dB |

Figure 2-5: Simulation results for MU-MIMO with Rank2 target UE using MCS17 and Rank2 co-scheduled UE with orthogonal precoding.

| TDLA30-10 ULA Low correlation | TDLC300-100 ULA Low correlation |
| --- | --- |

Table 2-9: Simulation results for MU-MIMO with Rank2 target UE using MCS17 and Rank2 co-scheduled UE with non-orthogonal precoding.

| Non-orthogonal precoding | R-ML | E-MMSE-IRC | MMSE-IRC | Gain of R-ML | Gain of E-MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| coUE:QPSK TDLA-Low | 18.7dB | 21.2dB | 21.2dB | 2.5dB | 0.0dB |
| coUE:16-QAM TDLA-Low | 20.8dB | 21.2dB | 21.2dB | 0.4dB | 0.0dB |
| coUE:64-QAM TDLA-Low | 21.6dB | 21.2dB | 21.2dB | -0.4dB | 0.0dB |
| coUE:QPSK TDLC-Low | 22.5dB | 26.3dB | N/A | N/A | N/A |
| coUE:16-QAM TDLC-Low | 25.0dB | 26.3dB | N/A | N/A | N/A |
| coUE:64-QAM TDLC-Low | 25.9dB | 26.3dB | N/A | N/A | N/A |

Figure 2-6: Simulation results for MU-MIMO with Rank2 target UE using MCS17 and Rank2 co-scheduled UE with non-orthogonal precoding.

| TDLA30-10 ULA Low correlation | TDLC300-100 ULA Low correlation |
| --- | --- |

# 2.3 Simulation analysis

We can do several observations from these simulations.

Observations from MCS13 1+1 scenario:

Observation #1: In MCS13 1+1 scenario with orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.7dB in TDLC300-100 low correlation and 0.4dB in TDLC300-100 medium correlation.

Observation #2: In MCS13 1+1 scenario with non-orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 1.1dB in TDLC300-100 low correlation and 0.7dB in TDLC300-100 medium correlation.

Observation #3: In MCS13 1+1 scenario with orthogonal random precoding R-ML gain over MMSE-IRC varies between 0.9dB and 2.4dB in TDLC300-100 low correlation and between 1.7dB and 6.0dB in TDLC300-100 medium correlation depending on co-scheduled UE modulation order.

Observation #4: In MCS13 1+1 scenario with non-orthogonal random precoding R-ML gain over MMSE-IRC varies between 1.6dB and 3.9dB in TDLC300-100 low correlation and between 2.5dB and 7.9dB in TDLC300-100 medium correlation depending on co-scheduled UE modulation order.

Observations from MCS13 2+2 scenario:

Observation #5: In MCS13 2+2 scenario with orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.0dB in TDLA30-10 low correlation and 1.5dB in TDLC300-100 low correlation.

Observation #6: In MCS13 2+2 scenario with non-orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.0dB in TDLA30-10 low correlation and 7.3dB in TDLC300-100 low correlation.

Observation #7: In MCS13 2+2 scenario with orthogonal random precoding R-ML gain over MMSE-IRC varies between 0.1dB and 1.4dB in TDLA30-10 low correlation and between 1.6dB and 2.9dB in TDLC300-100 low correlation depending on co-scheduled UE modulation order.

Observation #8: In MCS13 2+2 scenario with non-orthogonal random precoding R-ML gain over MMSE-IRC varies between 0.3dB and 3.0dB in TDLA30-10 low correlation and between 7.9dB and 10.8dB in TDLC300-100 low correlation depending on co-scheduled UE modulation order.

Observations from MCS17 2+2 scenario:

Observation #9: In MCS17 2+2 scenario with orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.0dB in TDLA30-10 low correlation and 11.9dB in TDLC300-100 low correlation.

Observation #10: In MCS17 2+2 scenario with non-orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.0dB in TDLA30-10 low correlation and undefined in TDLC300-100 low correlation.

Observation #11: In MCS17 2+2 scenario with orthogonal random precoding R-ML gain over MMSE-IRC varies between -0.4dB and 1.4dB in TDLA30-10 low correlation and between 11.4dB and 13.8dB in TDLC300-100 low correlation depending on co-scheduled UE modulation order.

Observation #12: In MCS17 2+2 scenario with non-orthogonal random precoding R-ML gain over MMSE-IRC varies between -0.4dB and 2.5dB in TDLA30-10 low correlation and undefined in TDLC300-100 low correlation depending on co-scheduled UE modulation order.

Common observations:

Observation #13: E-MMSE-IRC gain over MMSE-IRC do not depend on modulation order of co-scheduled UE.

Observation #14: R-ML gain over MMSE-IRC increases when modulation order of co-scheduled UE decrease.

Observation #15: In TDLA30-10, MMSE-IRC and E-MMSE-IRC are practically equivalent.

# Conclusion

In this paper we provided the view on the advanced receiver to cancel inter-user interference for MU-MIMO. The following observations and proposals are made:

Observation #1: In MCS13 1+1 scenario with orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.7dB in TDLC300-100 low correlation and 0.4dB in TDLC300-100 medium correlation.

Observation #2: In MCS13 1+1 scenario with non-orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 1.1dB in TDLC300-100 low correlation and 0.7dB in TDLC300-100 medium correlation.

Observation #3: In MCS13 1+1 scenario with orthogonal random precoding R-ML gain over MMSE-IRC varies between 0.9dB and 2.4dB in TDLC300-100 low correlation and between 1.7dB and 6.0dB in TDLC300-100 medium correlation depending on co-scheduled UE modulation order.

Observation #4: In MCS13 1+1 scenario with non-orthogonal random precoding R-ML gain over MMSE-IRC varies between 1.6dB and 3.9dB in TDLC300-100 low correlation and between 2.5dB and 7.9dB in TDLC300-100 medium correlation depending on co-scheduled UE modulation order.

Observation #5: In MCS13 2+2 scenario with orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.0dB in TDLA30-10 low correlation and 1.5dB in TDLC300-100 low correlation.

Observation #6: In MCS13 2+2 scenario with non-orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.0dB in TDLA30-10 low correlation and 7.3dB in TDLC300-100 low correlation.

Observation #7: In MCS13 2+2 scenario with orthogonal random precoding R-ML gain over MMSE-IRC varies between 0.1dB and 1.4dB in TDLA30-10 low correlation and between 1.6dB and 2.9dB in TDLC300-100 low correlation depending on co-scheduled UE modulation order.

Observation #8: In MCS13 2+2 scenario with non-orthogonal random precoding R-ML gain over MMSE-IRC varies between 0.3dB and 3.0dB in TDLA30-10 low correlation and between 7.9dB and 10.8dB in TDLC300-100 low correlation depending on co-scheduled UE modulation order.

Observation #9: In MCS17 2+2 scenario with orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.0dB in TDLA30-10 low correlation and 11.9dB in TDLC300-100 low correlation.

Observation #10: In MCS17 2+2 scenario with non-orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.0dB in TDLA30-10 low correlation and undefined in TDLC300-100 low correlation.

Observation #11: In MCS17 2+2 scenario with orthogonal random precoding R-ML gain over MMSE-IRC varies between -0.4dB and 1.4dB in TDLA30-10 low correlation and between 11.4dB and 13.8dB in TDLC300-100 low correlation depending on co-scheduled UE modulation order.

Observation #12: In MCS17 2+2 scenario with non-orthogonal random precoding R-ML gain over MMSE-IRC varies between -0.4dB and 2.5dB in TDLA30-10 low correlation and undefined in TDLC300-100 low correlation depending on co-scheduled UE modulation order.

Observation #13: E-MMSE-IRC gain over MMSE-IRC does not depend on modulation order of co-scheduled UE.

Observation #14: R-ML gain over MMSE-IRC increases when modulation order of co-scheduled UE decrease.

Observation #15: In TDLA30-10, MMSE-IRC and E-MMSE-IRC are practically equivalent.

# Reference

- RP-230313, “Status report for NR demodulation requirement evolution”, China Telecom, Intel Corporation

- R4-2305914, “WF on advanced receiver for MU-MIMO scenario”, China Telecom