3GPP TSG-RAN WG4 Meeting # 107		R4-2307891

# Incheon, KR, May 22 – May 26, 2023

Agenda item:			8.12.3

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for FR1 non-co-located EN-DC and NR-CA

Document for:	Discussion

# Introduction

In this contribution, our views on RRM requirements for FR1 non-collocated EN-DC and CA are provided.

# For Type 2 UE for 2 layer MIMO case

It has agreed that the existing activation delay requirements (clause 8.3.2) for an unknown FR1 SCell for non-contiguous intra-band CA can be re-used for Type 2 UE [1]. The existing unkwown requirement is copied below and the reuirements can apply to non-contiguous intra-band CA is highlghted. If it is the common understanding in RAN4, then it may not need to be clarified in the CR.

If the SCell is unknown and belongs to FR1, and if one of the following conditions is met

-	 ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or

-	 ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation,

provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is:

-	TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms, if the following conditions are met,

-	the SCell is contiguous to an active serving cell in the same band, and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB;

-	TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms, otherwise. // TCI is provided

otherwise, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is: // TCI not provided

-	6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting,

-	3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

Proposal 1: For an unknown FR1 intra-band non-collocated SCell, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is:

- Tactivation_time is TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms, if one of the following conditions is met

-	 ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or

-	 ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation,

- Otherwise Tactivation_time is:

  - 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting,

  - 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

# Summary

The observations and proposals in this contribution are summarized:

Proposal 1: For an unknown FR1 intra-band non-collocated SCell, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is:

- Tactivation_time is TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms, if one of the following conditions is met

-	 ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or

-	 ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation,

- Otherwise Tactivation_time is:

  - 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting,

  - 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

# Reference

- R4-2306335, WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA, RAN4#106b, Apr. 2023.