3GPP TSG-RAN WG4 Meeting # 107		R4-2307896

# Incheon, KR, May 22 – May 26, 2023

Agenda item:			9.7.6

Source:	MediaTek Inc.

Title:	Discussion on RRM performance requirement for IoT NTN

Document for:	Discussion

# Introduction

In this contribution, our views are provided for RRM performance requirement for IoT NTN, including test cases of measurement procedure and measurement performance for intra-frequency in GEO for Cat-M1. The overall test cases are listed in section 3 for reference.

# Tests case list

Test cases for intra-frequency in GEO are agreed in WF [1]. In addition, for Cat-M1, test cases of measurement procedure and measurement performance can also be introduced.

Proposal 1: For Cat-M1, introduce test cases of measurement procedure and measurement performance for intra-frequency in GEO, including the following test cases:

| Measurement Procedure | Event triggered reporting | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | M1 6-1 |
| --- | --- | --- | --- |
|  |  | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | M1 6-2 |
|  |  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | M1 6-3 |
|  |  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | M1 6-4 |
| Measurement Performance Requirements | RSRP accuracy | FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | M1 7-1 |
|  |  | HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | M1 7-2 |

# Test case summary

#### List of Tests for IoT NTN for NB-IoT

Table I. Test case for IoT NTN for NB-IoT

|  |  |  | # | Agreed? | Volunteer |
| --- | --- | --- | --- | --- | --- |
| RRC_IDLE state | Cell Re-Selection | HD – FDD Intra frequency case for UE Category NB1 in normal coverage | NB 1-1 | Yes fo GEO | MTK |
|  |  | HD – FDD Inter frequency case for UE Category NB1 in normal coverage | NB 1-2 | No |  |
|  |  | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation | NB 1-3 | Yes for GEO | MTK |
|  |  | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX | NB 1-4 | Yes for GEO |  |
| RRC Connection Control | RRC Re-establishment | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage | NB 2-1 | Yes for GEO | Huawei |
|  |  | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage | NB 2-2 | No |  |
|  | Random Access | Contention Based Random Access Test for UE category NB1 UEs Standalone mode in normal coverage | NB 2-3 | Yes | Nokia |
|  |  | Contention Based Random Access Test for UE category NB1 UEs Standalone mode in Enhanced Coverage | NB 2-4 | Yes |  |
|  |  | Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage | NB 2-5 | Yes |  |
| Timing | UE Transmit Timing | HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage | NB 3-1 | Yes | Huawei |
|  |  | HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage | NB 3-2 | Yes |  |
|  | UE Timing Advance | HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage | NB 3-3 | Yes |  |
|  |  | HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage | NB 3-4 | Yes |  |
| Signaling characteristic | RLM | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage | NB 4-1 | Yes | MTK |
|  |  | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage | NB 4-2 | Yes |  |
|  |  | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Enhanced Coverage | NB 4-3 | Yes |  |
|  |  | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage | NB 4-4 | Yes |  |
|  |  | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | NB 4-5 | Yes |  |
|  |  | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage | NB 4-6 | Yes |  |
|  |  | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | NB 4-7 | Yes |  |
|  |  | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage | NB 4-8 | Yes |  |
| Measurement Performance Requirements | channel quality reporting accuracy | HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage | NB 5-1 | Yes | Qualcomm |
|  |  | HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage | NB 5-2 | Yes |  |
|  |  | HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage | NB 5-3 | Yes |  |
|  |  | HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage | NB 5-4 | Yes |  |
|  |  | HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage | NB 5-5 | Yes |  |
|  |  | HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage | NB 5-6 | Yes |  |

#### List of Tests for IoT NTN for Cat-M1

Table II. Test case for IoT NTN for eMTC

|  |  |  | # | Agreed? | Volunteer |
| --- | --- | --- | --- | --- | --- |
| RRC_IDLE state | Cell Re-Selection | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage | M1 1-1 | Yes fo GEO | Nokia |
|  |  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage | M1 1-2 | Yes fo GEO |  |
|  |  | E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage | M1 1-3 | No |  |
|  |  | E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage | M1 1-4 | No |  |
|  |  | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation | M1 1-5 | Yes fo GEO | Nokia |
|  |  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation | M1 1-6 | Yes fo GEO |  |
| RRC CONNECTED Mode Mobility | Handover and Conditional Handover | E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition | M1 2-1 | Yes fo GEO | CMCC |
|  |  | E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition | M1 2-2 | Yes fo GEO |  |
|  |  | E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA | M1 2-3 | No |  |
|  |  | E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA | M1 2-4 | No |  |
|  |  | E-UTRAN FDD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-5 | Yes fo GEO | CMCC |
|  |  | E-UTRAN HD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-6 | Yes fo GEO |  |
|  |  | E-UTRAN FDD inter frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-7 | No |  |
|  |  | E-UTRAN HD-FDD inter frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-8 | No |  |
| RRC Connection Control | RRC Re-establishment | E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-1 | Yes fo GEO | Ericsson |
|  |  | E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-2 | Yes fo GEO |  |
|  |  | E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-3 | No |  |
|  |  | E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-4 | No |  |
|  | Random Access | E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | M1 3-5 | Yes | Huawei |
|  |  | E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | M1 3-6 | Yes |  |
|  |  | E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | M1 3-7 | Yes |  |
|  |  | E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | M1 3-8 | Yes |  |
| Timing | UE Transmit Timing | E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | M1 4-1 | Yes | Nokia |
|  |  | E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | M1 4-2 | Yes |  |
|  |  | E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | M1 4-3 | Yes |  |
|  |  | E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | M1 4-4 | Yes |  |
|  | UE Timing Advance | E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | M1 4-5 | Yes |  |
|  |  | E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | M1 4-6 | Yes |  |
|  |  | E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB | M1 4-7 | Yes |  |
|  |  | E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB | M1 4-8 | Yes |  |
| Signaling characteristic | RLM | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | M1 5-1 | Yes | Ericsson |
|  |  | E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | M1 5-2 | Yes |  |
|  |  | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | M1 5-3 | Yes |  |
|  |  | E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | M1 5-4 | Yes |  |
|  |  | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for Cat-M1 UE in CEMode A | M1 5-5 | Yes | CMCC |
|  |  | E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync in DRX for Cat-M1 UE in CEMode A | M1 5-6 | Yes |  |
|  |  | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for Cat-M1 UE in CEMode A | M1 5-7 | Yes |  |
|  |  | E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync in DRX for Cat-M1 UE in CEMode A | M1 5-8 | Yes |  |
| Measurement Procedure | Event triggered reporting | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | M1 6-1 | suspended |  |
|  |  | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | M1 6-2 | suspended |  |
|  |  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | M1 6-3 | suspended |  |
|  |  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | M1 6-4 | suspended |  |
|  |  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA | M1 6-5 | No |  |
|  |  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used | M1 6-6 | No |  |
|  |  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA | M1 6-7 | No |  |
|  |  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used | M1 6-8 | No |  |
| Measurement Performance Requirements | RSRP accuracy | FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | M1 7-1 | suspended |  |
|  |  | HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | M1 7-2 | suspended |  |
|  |  | FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA | M1 7-3 | No |  |
|  |  | HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA | M1 7-4 | No |  |
|  | channel quality reporting accuracy | E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A | M1 7-5 | Yes | Ericsson |
|  |  | E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A | M1 7-6 | Yes |  |
|  |  | E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B | M1 7-7 | Yes |  |
|  |  | E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B | M1 7-8 | Yes |  |

# Summary

The observations and proposals in this contribution are summarized:

Proposal 1: For Cat-M1, introduce test cases of measurement procedure and measurement performance for intra-frequency in GEO, including the following test cases:

| Measurement Procedure | Event triggered reporting | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | M1 6-1 |
| --- | --- | --- | --- |
|  |  | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | M1 6-2 |
|  |  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | M1 6-3 |
|  |  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | M1 6-4 |
| Measurement Performance Requirements | RSRP accuracy | FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | M1 7-1 |
|  |  | HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | M1 7-2 |

# Reference

- R4-2306370, WF on NB-IoT/eMTC RRM requirements for NTN, MediaTek Inc., RAN4#106bis-e, Apr. 2023.