3GPP TSG-RAN WG4 Meeting # 107                                    R4-2307925

Incheon, Korea, 22th - 26th May 2023

Agenda item:	8.32.4

Source: 	ZTE Corporation

Title: 	Discussion on impacts to RRM core requirements for Enhanced RedCap

Document for:	Approval

Introduction

In RAN#98e meeting, the revised WID on Enhanced support of reduced capability NR devices has been approved. Core part objectives are shown below for reference:

## The objective is to specify support for the following enhancements: ## Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2, CT1 and CT4 involvement ## Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. ## Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. ## Check in RAN#99 regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

In this contribution, we provide our views on the RRM impacts for enhanced Redcap UE about the UE BB bandwidth reduction.

Discussion

At the last meeting, RAN4 has discussed enhanced support for reduced capability devices and has achieved some agreements[1]. We can divide the discussion into two main parts, complexity/cost reduction and power saving/energy efficiency enhancements, respectively.

Regarding energy-saving/energy efficiency enhancement, compared to R17 RedCap UE, R18 eRedCap UE supports the eDRX>10.24s in RRC_INACTIVE state to obtain extra power saving gain and new UE requirements will be defined. According to what we know, the maximum value of RRC_IDLE eDRX cycle is larger than 10.24s in existing results. Regarding specific requirement definitions, RAN4 needs to wait for more input from other working groups, such as delay requirements, which require waiting for RAN2's limitations on PTW and definitions for T.

Regarding complexity/cost reduction, it can be achieved by UE BB bandwidth reduction and UE peak data rate reduction. Meanwhile, we have reached a consensus that there is no impact due to peak data rate reduction objective in FR1 and no impact on paging reception requirements and THARQ due to baseband BW reduction. Therefore, the remaining issue that urgently needs to be discussed is whether the reduction of BB will affect the CGI reading requirements.

Impact on CGI reading requirements - FFS whether to define CGI reading requirements for Rel-18 eRedCap UEs - If CGI reading requirements for Rel-18 eRedCap UEs are introduced - If SIB1 is scheduled within 5MHz BW - Reuse existing requirements for SIB1 reading for the case - FFS for the requirements for the case of SIB1 scheduled with BW > 5MHz

Based on the conclusion of the last meeting, we need to discuss two issues here: whether it is necessary to define and how to define CGI reading requirements for Rel-18 eRedCap UEs.

Some companies believe that it is not necessary to define it because NW can perform CGI reading through normal UE with normal BB bandwidth instead of R18 Redcap UE with reduced BB bandwidth. However, we do not believe it is necessary to exclude CGI reading. RAN1 has agreed that eRedCap can be configured to BW>5 MHz, so eRedCap UE can support CGI reading [2].

SIB1 bandwidth Agreement: (replaced by later agreement) For UE BB bandwidth reduction, for SIB1 (PDSCH) to Rel-18 RedCap UEs, down-select between the following options: - Option 1: Restrict the scheduling of SIB1 to be within 5 MHz - Option 2: Allow the scheduling of SIB1 to be larger than 5 MHz (as in legacy operation) - FFS: whether 5MHz is assumed to be physically contiguous Agreement: Replace the agreement on SIB1(PDSCH) for UE BB bandwidth reduction with the following: For UE BB bandwidth reduction, for SIB1 (PDSCH), - Allow the scheduling of SIB1 to be larger than 5 MHz (as in legacy operation) - FFS: UE post-FFT buffering “assumption” (replaced by later agreement)

Observation 1: Rel-18 bandwidth-reduced RedCap UE needs to read SIB1 which is transmitted larger than 5MHz.

Proposal 1: Define CGI reading requirements for Rel-18 eRedCap UEs.

These complexity reduction focus on reduce UE BB bandwidth for PDSCH (for both unicast and broadcast) and PUSCH to 5MHz while keeping 20 MHz RF bandwidth. Other physical channels and reference signals still use a bandwidth up to 20 MHz as for Rel-17 RedCap. In our understanding, there is no reduction on SSB bandwidth since the RF BW is still 20MHz and it has very limited impact on RRM requirements. However, the requirements for SIB1 reading might be impacted due to UE BB bandwidth reduction.

According to RAN1’s agreement, allow the scheduling of SIB1 to be larger than 5 MHz for UE BB bandwidth reduction. In our understanding, Rel-18 UE is unable to receive the entire PDSCH using a BW greater than 5 MHz and UE needs to combine multiple SIB1 transmissions and process SIB1 in multiple slots. For instance, for a 5MHz BB bandwidth that supports SIB1 with BW=X MHz, where X>5, eRedCap UE expects to repeatedly combine ceil(X/5) times to obtain the entire SIB1. Therefore, the SIB1 decoding delay of Rel-18 eRedCap requires additional samples. In the worst-case scenario, 4 extensions are required.

In Clause 9.11A.2 of TS 38.133, there are the following CGI reading requirements. From the calculation formula, it can be found that when the decoding time of SIB1 increases due to a decrease in UE BB bandwidth for PDSCH, the interruption duration on the serving cell may be larger accordingly. Therefore, RAN4 should discuss whether an extended CGI reporting period is needed for the requirements for the case of SIB1 scheduled with BW > 5MHz.

Clause 9.11A.2 of TS 38.133 Tidentify_CGI_redcap = (TMIB_redcap + T SIB1_redcap) ms Where: TMIB_redcap is the time period used to acquire MIB message. TMIB_redcap = 6 * TSMTC ms for target cell carrier frequency on FR1 and TMIB_redcap = 25 * TSMTC ms for target cell carrier frequency on FR2 TSIB1_redcap is the time period used to acquire SIB1 message. - TSIB1_redcap = 6 * TRMSI-scheduling ms for 2 Rx RedCap. - TSIB1_redcap = 12 * TRMSI-scheduling ms for 1 Rx RedCap.

Proposal 2: For Rel-18 eRedCap UEs, RAN4 should discuss whether an extended CGI reporting period is needed for the requirements for the case of SIB1 scheduled with BW > 5MHz.

Conclusions

In this contribution, we provide our views on the RRM impacts for enhanced Redcap UE and the proposals are:

Observation 1: Rel-18 bandwidth-reduced RedCap UE needs to read SIB1 which is transmitted larger than 5MHz.

Proposal 1: Define CGI reading requirements for Rel-18 eRedCap UEs.

Proposal 2: For Rel-18 eRedCap UEs, RAN4 should discuss whether an extended CGI reporting period is needed for the requirements for the case of SIB1 scheduled with BW > 5MHz.

References

- R4-2306365, WF on R18 RedCap RRM requirements, Ericsson.

- R4-2301885, RAN1 agreements for Rel-18 NR RedCap, Rapporteur (Ericsson).