3GPP TSG-RAN4 Meeting #107		R4-2307950

Incheon, KR, 22th May – 26th May, 2023

Title:	Discussion on L1/L2 based inter-cell mobility delay requirement

Source:	Xiaomi

Agenda Item:	8.25.2.3

Document for:	Discussion

- Introduction

In RAN4#106bis-e meeting, RAN4 had some discussion on the delay of L1/L2 based inter-cell mobility, and the related WF was approved in [1]. In this contribution, we would like to discuss L1/L2 based inter-cell mobility delay requirements and provide our proposals.

- Discussion

General and Principles

Issue 3-1-5: SpCell cell switch delay requirements for SpCell change with SCell change < Way forward >: FFS the following options - Option 1: For the scenario “PCell change with SCell change”, PCell switch delay is not extended by SCell changes, i.e., UE is supposed to perform SCell change after cell switch of PCell is finished. - Option 2: FFS Issue 3-1-6: LTM delay requirements < Way forward >: FFS the following options - Option 1: Not define the LTM delay requirement which starts from UE receives RRC configuration on candidate cell(s). - Option 2: FFS

For SpCell cell switch delay requirements for SpCell change with SCell change, we think the SpCell switch delay is not extended by SCell change and the SCell change should be done after SpCell cell switch is complete, as the SMTC window of target SCell is based on the target SpCell’s timing.

Proposal 1: For the scenario “SpCell change with SCell change”, SpCell switch delay is not extended by SCell changes, i.e., UE is supposed to perform SCell change after cell switch of SpCell is finished.

For LTM delay requirements, RAN4 does not need to define the LTM delay requirement which starts from UE receives RRC configuration on candidate cell(s), as according to signaling design in RAN2, the LTM procedure starts when the UE receives the cell switch command.

Proposal 2: RAN4 not to define the LTM delay requirement which starts from UE receives RRC configuration on candidate cell(s).

Timeline of cell swith delay for Pcell/PSCell

Issue 3-2-1: Ending point of RACH-less cell switch delay for PCell/PSCell < Agreement>: - Wait for more progress in RAN2 Issue 3-2-2: Procedure of cell switch < Way forward >: FFS the following options - Option 1: Further discuss whether UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch. - Option 2: Under the condition that target cell is known, UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch. - Option 3: if T/F fine tracking (TΔ) is needed after receiving cell switch command, UE is not required to perform it before L1/L2/L3 processing (Tprocessing,2). - Option 4: UE cannot perform T/F tracking before necessary L1 reconfiguration, which get the UE's L1 ready to receive DL of the target cell. If needed, some other L1 configuration, L2/L3 reconfiguration and L2 reset can be performed during cell switch, i.e., in some cases it can be performed after the T/F tracking.

For RACH-based cell switch, it was agreed that the ending point can be defined as the time when UE starts the transmission of new PRACH on the target cell. For RACH-less cell switch, as UE doesn’t need to acquire TA during the cell switch. And according to RAN1’s agreement, 1 joint or 1 pair of UL/DL unified TCI state index for the target cell is conveyed in MAC CE cell switch command, thus, after the cell switch, the UE is ready to perform DL/UL reception/transmission on the indicated beam of target cell.

RAN1 Agreement From RAN1 point of view, at least the following information can be included in the cell switch command, which is conveyed by MAC CE - Information to identify the target cell(s) - The details including bit number are designed by RAN2 - TA related information (details up to the discussion in A.I. 9.10.2) - 1 joint or 1 pair of UL and DL unified TCI State index for the target Cell - Note: discussion on target SpCell is not precluded - Active DL and UL BWPs for the target cell - FFS: Triggering of aperiodic TRS transmitted from the target cell - FFS: Triggering the CSI acquisition of the target cell and reporting to the target cell - FFS: Triggering of aperiodic SRS transmission to the target cell - FFS: C-RNTI - FFS: the presence of each field (i.e. always present or configurable)

Proposal 3: For RACH-less cell switch, the ending point of cell switch delay is the time when UE performs the first DL/UL reception/transmission on the indicated beam of the target cell.

According to RAN1 agreements, the TCI state activation of a candidate cell is received before the reception of beam indication of the candidate cell. Thus, the UE needs to perform T/F fine time tracking before receiving cell switch command. And RAN1 is discussing whether to support the TCI state activation of a candidate cell is received together with the reception of beam indication of the candidate cell, if supported, UE needs to perform T/F fine time tracing during cell switch procedure and the UE is not required to perform T/F fine time tracing before L1/L2/L3 processing (Tprocessing).

RAN1 Agreement For the Rel-17 unified TCI based beam indication in Rel-18 LTM, at least Alt 1 is supported: - Alt 1: TCI state activation of a candidate cell is received before the reception of beam indication of the candidate cell, - Alt 2: TCI state activation of a candidate cell is received together with the reception of beam indication of the candidate cell - FFS: signalling details for TCI state indication, if both activation and indication are done in the same MAC CE message carrying switch command - Alt 3: Alt 1 and/or Alt 2 can be supported based on the UE capability FFS: signalling details for TCI state activation FFS: For Alt 1, whether/how TCI state activation for candidate cell(s) is allowed Note: If scenarios 1 and 3 are to be supported other beam indication/TCI activation timing relationships are not precluded.

Proposal 4: If T/F fine tracking (TΔ) is needed after receiving cell switch command, UE is not required to perform it before L1/L2/L3 processing (Tprocessing,2).

Detail of cell switch delay requirements for PCell/PSCell

Issue 3-3-1: RACH-based Cell switch delay for Pcell/PSCell < Agreement>: - The baseline of RACH-based cell switch delay requirements is Tdelay = Tcmd + Tprocessing / TLTM-processing + Tsearch + T∆ + Tmargin + Tuncertainity /TIU, where Tuncertainity /TIU is the uncertainty in acquiring the first available PRACH occasion in the new cell. - FFS: the exact value of each component. Some components can be 0 in certain cases, if agreed. - FFS: add/remove/modify other component(s). Note: ‘/’ means ‘or’ here. Issue 3-3-2: RACH-less Cell switch delay for Pcell/PSCell < Agreement>: - The baseline of RACH-less cell switch delay requirements is Tdelay = Tcmd + Tprocessing,2 / TLTM-processing + Tsearch + T∆ + Tmargin + Tuncertainity/TIU, - FFS: the ending point - FFS: the exact value of each component. Some components can be 0 in certain cases, if agreed. - FFS: add/remove/modify other component(s). Note: ‘/’ means ‘or’ here.

We support to define cell switch delay requirement provided that the target cell is known to UE, in other words, the subframe/slot/symbol level synchronization of target cell should be guaranteed during cell switch procedure. Thus, the cell search delay of target cell should not be accounted in cell switch delay requirement.

Observation 1: The cell search delay of target cell should not be accounted in cell switch delay requirement.

In legacy HO requirement, the UE processing time is the time for UE software processing and RF warm-up. Since UE has already performed L1 measurement before receiving cell switch command, the RF warm-up delay may not be needed. However, the software processing time and the baseband preparation time are still needed for target cell, RAN4 needs to discuss whether to reduce the delay of Tprocessing.

Observation 2: The UE processing time in term of software processing time and baseband preparation time are considered in cell switch delay requirement.

If RAN1 supports the TCI state activation of a candidate cell is received together with the reception of beam indication of the candidate cell, the UE needs to perform T/F fine time tracing during cell switch procedure.

Observation 3: If T/F fine tracking (TΔ) is needed after receiving cell switch command, the T/F fine timing tracking delay is considered in cell switching delay.

For RACH-based cell switch, the RACH uncertainty delay need to be considered in cell switch delay. However, for RACH-less cell switch, the RACH uncertainty delay is not needed, instead, the uncertainty of the first PUSCH transmission on target cell should be considered.

Observation 4:  For RACH-based cell switch, the RACH uncertainty delay is considered in cell switching delay.

Observation 5: For RACH-less cell switch, the uncertainty delay of the first DL/UL reception/transmission on target cell is considered in cell switching delay.

In summary, the delay formula for RACH-based cell switch can be expressed as:

Tdelay = Tcmd + Tprocessing + T∆ + Tmargin + TIU

And the delay formula for RACH-less cell switch can be expressed as:

Tdelay = Tcmd + Tprocessing + T∆ + Tmargin + TIU

Where TUn is the time uncertainty of the first PUSCH transmission on the target cell.

Proposal 5: For L1/L2 based inter-cell mobility, the cell switch delay is defined as follows:

- For RACH-based cell switch

Tdelay = Tcmd + Tprocessing + T∆ + Tmargin + TIU

Where,

Tcmd is the MAC-CE decoding time;

Tprocessing is the SW processing and baseband preparation time

T∆ is the T/F fine timing tracking delay

TIU is the uncertainty in acquiring the first available PRACH occasion

Tmargin is time for SSB post-processing

- For RACH-less cell switch

Tdelay = Tcmd + Tprocessing + T∆ + Tmargin + TIU

Where,

Tcmd is the MAC-CE decoding time;

Tprocessing is the SW processing and baseband preparation time

T∆ is the T/F fine timing tracking delay

TIU is the time uncertainty of the first DL/UL reception/transmission on the target cell

Tmargin is time for SSB post-processing

- Conclusion

In this contribution, we discuss the L1/L2 based inter-cell mobility delay requirements and provide our proposals as follows.

Proposal 1: For the scenario “SpCell change with SCell change”, SpCell switch delay is not extended by SCell changes, i.e., UE is supposed to perform SCell change after cell switch of SpCell is finished.

Proposal 2: RAN4 not to define the LTM delay requirement which starts from UE receives RRC configuration on candidate cell(s).

Proposal 3: For RACH-less cell switch, the ending point of cell switch delay is the time when UE performs the first DL/UL reception/transmission on the indicated beam of the target cell.

Proposal 4: If T/F fine tracking (TΔ) is needed after receiving cell switch command, UE is not required to perform it before L1/L2/L3 processing (Tprocessing,2).

Observation 1: The cell search delay of target cell should not be accounted in cell switch delay requirement.

Observation 2: The UE processing time in term of software processing time and baseband preparation time are considered in cell switch delay requirement.

Observation 3: If T/F fine tracking (TΔ) is needed after receiving cell switch command, the T/F fine timing tracking delay is considered in cell switching delay.

Observation 4:  For RACH-based cell switch, the RACH uncertainty delay is considered in cell switching delay.

Observation 5: For RACH-less cell switch, the uncertainty delay of the first DL/UL reception/transmission on target cell is considered in cell switching delay.

Proposal 5: For L1/L2 based inter-cell mobility, the cell switch delay is defined as follows:

- For RACH-based cell switch

Tdelay = Tcmd + Tprocessing + T∆ + Tmargin + TIU

Where,

Tcmd equals to THARQ+3ms;

Tprocessing is the SW processing and baseband preparation time

T∆ is the fine timing tracking delay

TIU is the uncertainty in acquiring the first available PRACH occasion

Tmargin is time for SSB post-processing

- For RACH-less cell switch

Tdelay = Tcmd + Tprocessing + T∆ + Tmargin + TIU

Where,

Tcmd equals to THARQ+3ms;

Tprocessing is the SW processing and baseband preparation time

T∆ is the fine timing tracking delay

TIU is the time uncertainty of the first DL/UL reception/transmission on the target cell

Tmargin is time for SSB post-processing

- Reference

[1]	R4-2306395	WF on NR Mobility Enhancements RRM requirements (part 1),	MediaTek