3GPP TSG-RAN WG4 Meeting #107 	R4-2308024

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.8.3.2

Source: 	Ericsson

Title:	On L1-RSRP measurements

Document for:	Discussion

# Introduction

The agreements from RAN4#106bis-e are summarized in [12] and [13] for the 1st and 2nd threads, respectively. And the discussion summaries are available in [14] and [15] for the 1st and 2nd threads, respectively.

# Discussion

## Overlapping and partially overlapping time resources

The WI scope is simultaneous RS reception of two RSs (with different IDs) from different directions, i.e., the RSs occasions must overlap in time in at least one measurement occasion during the measurement period.

- Proposal 1: For simultaneous reception, the two different RSs (with different IDs) from different directions must be received in at least one overlapping measurement occasion during their respective measurement periods.

However, there can be many scenarios when not all RS occasions overlap in time or the RS occasions not fully overlap, e.g., due to:

- Different RS periodicities (in which case the overlapping occasions where simultaneous reception is feasible will occur with a period which is LCM of the two periodicities),

- Different RS occasion duration (in which case simultaneous reception is possible only on a common part of the respective time resources),

- The RSs may be configured with the same periodicity but one of the RSs may be muted in some occasions, etc.

In all these cases, simultaneous RS reception is only relevant for a common set of time resources for the two RSs.

So, RAN4 needs to also discuss whether partial overlap in time is acceptable, and, if so, what is the minimum amount overlap is needed for the simultaneous reception.

- Proposal 2: The minimum amount of overlap needed for multi-rx requirements to apply is TBD (e.g., full overlap, partial overlap, at least X% of overlapping RS occasions during the measurement period, etc.).

- Proposal 3: When two RSs are configured in time resources which are not fully overlapping, the simultaneous reception requirements are defined with respect to the common subset of time resources for the two RSs (which may result in a longer measurement period for a partially overlapping case than for a fully overlapping case with the same periodicity).

## Reconfiguration to/from multi-rx

When the UE is configured with multi-rx operation, it has to switch from its previous operation mode to the multi-rx operation mode, with different requirements, etc. Also, if it is configured with non-multi-rx operation, it has to switch back from multi-rx to the non-multi-rx operation mode. There is no UE behaviour defined for UE measurements when the switching to/from multi-rx operation occurs during the UE measurement period.

- Proposal 4: UE shall continue measurements when switching to/from multi-rx operation occurs during the measurement period, but the more relaxed requirements (between the corresponding legacy and multi-rx requirements) shall apply for the impacted measurement period.

However, if the UE needs to drop/restart the measurement upon the switching, then switching should not be more frequent than at least one measurement period.

- Proposal 5: If the UE needs to drop/restart the measurement upon the switching, then switching should not be more frequent than at least one measurement period.

## Measurement period

The following was agreed in RAN4#106bis-e:

Agreement from GTW:

- Introduce enhanced RRM requirements based on faster beam sweeping with multi-Rx chains based on UE capabilities

  - Option 1: Introduce beam sweeping factor reduction

  - Other options not precluded

  - FFS on specific conditions when enhancement apply

  - FFS if any signalling is needed

  - FFS on details of UE capabilities

- Proposal 6: Since the reduction of the beam sweeping factor N is due to simultaneous reception attempts for the same RS, while the scope of the WI is simultaneous reception of different RSs, another reduction is necessary to fulfil the WI target.

To enhance the L1-RSRP measurement requirement based on simultaneous reception of different RSs, another scaling factor can be justified. This approach can be implemented in the specification, e.g., as shown in the table below. The new parameter (e.g., “L”) could take values 1 for non-simultaneous reception and L<1 for simultaneous reception with multi-rx.

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N*L)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N*L)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N*L)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

- Proposal 7: For multi-rx operation, the measurement period is enhanced by a new scaling parameter L, to account for simultaneous reception of different RSs. L=1 for non-simultaneous reception, L=TBD<1 (e.g., L=½ ) when multi-rx operation is activated and the necessary conditions for multi-rx operation are met.

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N*L)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N*L)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N*L)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

## Conditions in L1-RSRP requirements enhanced for multi-rx operation

The enhanced L1-RSRP requirements for simultaneous reception shall apply under certain necessary conditions.

- Proposal 8: The following is a necessary condition for simultaneous reception:

  - UE has the multi-rx operation capability (to be replaced with the exact capability name, with a relevant reference in the specification).

- Proposal 9: The following is a necessary condition for simultaneous reception:

  - UE is configured with dual TCI.

- Proposal 10: The following is a necessary condition for simultaneous reception:

  - UE is not configured with CA or DC.

- Proposal 11: The following is a necessary condition for simultaneous reception:

  - The simultaneously received RSs are in PCell only.

- Proposal 12: The following is a necessary condition for simultaneous reception:

  - Both RSs and their associated signals in the QCL type D infos are detectable during the entire measurement period.

For two RSs, interference levels may differ a lot in overlapping resources and non-overlapping resources, i.e., much higher in the overlapping resources. Hence, the side conditions have to be applied in the overlapping resources, which is the worst case. Otherwise, more advanced receivers may be needed to deal with the high interference in the overlapping resources.

- Proposal 13: The following is a necessary condition in multi-rx requirements:

  - The applicable side conditions are the conditions during the overlapping RS occasions (the conditions can be better in the non-overlapping occasions).

- Proposal 14: The following is a necessary condition in multi-rx requirements:

  - The measured RS is being received simultaneously with another RS, where the two RSs have QCL-TypeD with different references, in at least one overlapping measurement occasion during the measurement period.

# Summary

The following have been observed and proposed in this contribution.

- Proposal 1: For simultaneous reception, the two different RSs (with different IDs) from different directions must be received in at least one overlapping measurement occasion during their respective measurement periods.

- Proposal 2: The minimum amount of overlap needed for multi-rx requirements to apply is TBD (e.g., full overlap, partial overlap, at least X% of overlapping RS occasions during the measurement period, etc.).

- Proposal 3: When two RSs are configured in time resources which are not fully overlapping, the simultaneous reception requirements are defined with respect to the common subset of time resources for the two RSs (which may result in a longer measurement period for a partially overlapping case than for a fully overlapping case with the same periodicity).

- Proposal 4: UE shall continue measurements when switching to/from multi-rx operation occurs during the measurement period, but the more relaxed requirements (between the corresponding legacy and multi-rx requirements) shall apply for the impacted measurement period.

- Proposal 5: If the UE needs to drop/restart the measurement upon the switching, then switching should not be more frequent than at least one measurement period.

- Proposal 6: Since the reduction of the beam sweeping factor N is due to simultaneous reception attempts for the same RS, while the scope of the WI is simultaneous reception of different RSs, another reduction is necessary to fulfil the WI target.

- Proposal 7: For multi-rx operation, the measurement period is enhanced by a new scaling parameter L, to account for simultaneous reception of different RSs. L=1 for non-simultaneous reception, L=TBD<1 (e.g., L=½ ) when multi-rx operation is activated and the necessary conditions for multi-rx operation are met.

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N*L)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N*L)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N*L)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

- Proposal 8: The following is a necessary condition for simultaneous reception:

  - UE has the multi-rx operation capability (to be replaced with the exact capability name, with a relevant reference in the specification).

- Proposal 9: The following is a necessary condition for simultaneous reception:

  - UE is configured with dual TCI.

- Proposal 10: The following is a necessary condition for simultaneous reception:

  - UE is not configured with CA or DC.

- Proposal 11: The following is a necessary condition for simultaneous reception:

  - The simultaneously received RSs are in PCell only.

- Proposal 12: The following is a necessary condition for simultaneous reception:

  - Both RSs and their associated signals in the QCL type D infos are detectable during the entire measurement period.

- Proposal 13: The following is a necessary condition in multi-rx requirements:

  - The applicable side conditions are the conditions during the overlapping RS occasions (the conditions can be better in the non-overlapping occasions).

- Proposal 14: The following is a necessary condition in multi-rx requirements:

  - The measured RS is being received simultaneously with another RS, where the two RSs have QCL-TypeD with different references, in at least one overlapping measurement occasion during the measurement period.

# References

[1] RP-221753, WI description, Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Inc., June 2022.

[2] R4-2214344, WF on FR2 multi-Rx RRM, vivo, Aug. 2022.

[3] R4-2217246, WF on RRM impacts and general aspects for multi-Rx, vivo, Oct. 2022.

[4] R4-2217587, WF on L1 measurements, beam sweeping factors and simultaneous reception, Qualcomm, Oct. 2022.

[5] R4-2217248, WF on TCI state switching for multi-RX chain DL reception, Ericsson, Oct. 2022.

[6] R4-2220742, WF on NR FR2 multi-Rx chain DL reception – Part 1, Nov. 2022.

[7] R4-2220743, WF on NR FR2 multi-Rx chain DL reception – Part 2, Nov. 2022.

[8] R4-2220402, WF on NR FR2 multi-Rx chain DL reception – Part 3, Nov. 2022.

[9] R4-2303193, Ad-hoc minutes for NR FR2 multi-Rx chain DL reception RRM, vivo, March 2023.

[10] R4-2303302, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, March 2023.

[11] R4-2303317, WF on NR FR2 multi-Rx chain DL reception RRM requirements(part2), Qualcomm, March 2023.

[12] R4-2306318, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, Apr. 2023.

[13] R4- 2306319, WF on FR2_multiRx_part2, Ericsson, Apr. 2023.

[14] R4-2306392, Topic summary for [106bis-e][202] FR2_multiRx_part1, vivo, Apr. 2023.

[15] R4-2306234, Topic summary for [106bis-e][203] FR2_multiRx_part2, Ericsson, Apr. 2023.