3GPP TSG-RAN WG4 Meeting #107 	R4-2308030

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On General Aspects of AI/ML for NR Air Interface

Agenda item:	8.22.1

Document for:	Discussion

Introduction

At RAN4#106-bis-e, the first RAN4 discussion on AI/ML for NR air interface took place. The outcomes of the meeting are captured in the WF [1]. In addition to organizational agreements on the working plan, terminology, etc., some of the general issues require further discussion, as follows:

- Requirements on data collection and training dataset

- Using legacy performance requirements as a reference

- High-level testing framework

In this paper, we provide some additional views on the topics listed above.

More detailed analysis of Use-case specific aspects and Interoperability and testing aspects are provided in our accompanying papers [3] and [4], correspondingly.

Discussion

On Training and Data collection

At the RAN4#106-bis-e meeting, the following way forward on the requirements on data collection and testing dataset definition were captured:

2.1.2 RAN4 Scope and baseline performance Agreement: - General aspects - ... - RAN4 does not need to study requirements/tests for training - If other WG defines the training procedure, RAN4 may need study to define the requirements for it. - ... - FFS if requirements for data collection (in particular for training) could/need be defined 2.1.3 Training dataset definition Agreement: - Dataset to be used for the device model training is left to implementation - If a specific test for training is defined, RAN4 might have to introduce some conditions and/or accuracy requirements for the training dataset or training data generation 2.3.2 Online training procedures and testing Study of tests for online training procedures are de-prioritized - This can be re-visited if any online training procedure is introduced

There are several aspects of model training that can be related to the RAN4 requirements. At least the following directions can be listed:

- Assumptions on model training procedures

- Generation/collection of training data/dataset for model training

- Data collection for LCM, including (offline) training, from in the real network

Regarding, model training procedures and related datasets, on the one hand, for UE-based or NW-based models, we can follow the RAN4 agreement above that no requirements are needed. Indeed, in most cases, especially when online training is not considered in Release 18 SI, training of the models may be performed offline and even outside the 3GPP RAN. On the other hand, for two-sided models, coordination in model training, or at least model pairing, at the DUT and TE could be still relevant and some rules might need to be defined in RAN4 on the training/pairing procedure and training data used for training. For example, the RAN1 agreements include several options for two-sided model training (CSI compression use-case):

3 updated templates for “1-on-1 joint training”, “multi-vendor joint training” and “separate training”. 1 updated template for “model generalisation/scalability” [RAN1#112, R1-2301940] Agreement​ For how to separate the templates for different training types/cases for AI/ML-based CSI compression without generalization/scalability verification, the following is considered:​ - The determined template in the RAN1#111 working assumption is entitled with “1-on-1 joint training”​ - A second separate template is introduced to capture the evaluation results for “multi-vendor joint training”​ - Note: this table captures the results for the joint training cases of 1 NW part model to M>1 UE part models, N>1 NW part models to 1 UE part model, or N>1 NW part models to M>1 UE part models. An example is multi-vendor Type 2 training.​ - A third separate template is introduced to capture the evaluation results for “separate training”​ - FFS: additional KPIs for each template, e.g., overhead, latency, ect.

Training of one-sided UE- or NW-based models can be expected to be done offline and even outside of the 3GPP model. However, training of DUT and TE sides of the two-sided model might need to be coordinated, including the exchange of the information of the counterpart model(s) and common dataset for training.

Hence, the previous agreement from RAN4#106bis-e meeting should be clarified in the following way:

For Release 18, RAN4 does not need to study requirements/tests for training for one-sided models. However, for two-sided models, assumptions on model training or model pairing for DUT and TE needs to be clarified in RAN4.

Data collection from the real networks, however, is a different aspect, that is generic both for one-sided and two-side models. One existing example is the requirement on Minimization of Drive Tests (MDT) defined in RAN4 TS 38.133, Clause 4.3.

Following the RAN1 and RAN2, there are ongoing discussions of training, inference and monitoring data collection framework in general, and on the training data needs for particular use cases, e.g.,

RAN2#121bis-e [2]: Observation: RAN2 may need to consider enhancements for AIML to existing functionality for data collection, e.g. for timing control (e.g. for MDT/RRM). Agreements​ - P1: RAN2 to understand/determine/capture requirements of data collection for the LCM functionalities and document the results. FFS on the exact presentation format. Expect RAN1 to provide some related information. ​ - P2: RAN2 to capture the analysis (see P1 above) separately for the use-cases, i.e., CSI feedback enhancement, beam management and positioning enhancement. FFS how we do the formatting/presentation of the results. ​ - P3: Study the applicability (and limitations) of each identified data collection framework for each of the identified LCM purposes, i.e., inference, monitoring and (offline) training. FFS how we do the formatting/presentation of the results.​ - P4: With more progress on architectural discussion, consider the suitability of each identified data collection framework for the termination points and mapping with the location of LCM purposes/functions (inference, monitoring, (offline) training) ​ - Model sidedness (UE side, NW side, two sided) FFS RAN1#110 Agreement For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact: - Signaling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), Reference signals - Content/type of the collected data - Other aspect(s) is not precluded RAN1#112-bis-e [2]: In CSI compression using two-sided model use case, further study the necessity and potential specification impact of the following aspects related to the ground truth CSI format for NW side data collection for model training: ... Agreement​ Regarding data collection for NW-side AI/ML model, study necessity, benefits and beam-management-specific potential specification impact from RAN1 point of view on the following additional aspects ​ - Mechanism related to the reporting​ - Additional information for content of the reporting​ - FFS: Information associated with or configured for the reported data samples, e.g., timestamps, SNR, data quality, etc. Working Assumption​ Regarding data collection at least for model training for AI/ML based positioning, at least the following information of data with potential specification impact are identified.​ - Ground truth label​ - At least for model training​ - Report from the label data generation entity​ - Measurement (corresponding to model input)​ - At least for model training​ - Report from the measurement data generation entity​ - Quality indicator​ - For and/or associated with ground truth label and/or measurement at least for model training​ - Report from the label and/or the measurement data generation entity and/or as request from a different (e.g., data collection, etc.) entity​ - RS configuration(s)​ - At least for deriving measurement​ - Request from data generation entity (UE/PRU/TRP) to LMF and/or as LMF assistance signaling to UE/PRU/TRP​ - Note1: there may not be any enhancements on top of existing RS configuration(s) or any new RS configuration(s) for positioning measurement​ - Time stamp​ - At least for and/or associated with training data for model training​ - Separate time stamp for measurement and ground truth label, when measurement and ground truth label are generated by different entities​ - Report from data generation entity together with training data and/or as LMF assistance signaling​ - Note2: there may not be any enhancements on top of time stamp in existing positioning measurement report or any new time stamp report for positioning measurement

Based on the discussions above, we can conclude that data collection is an important topic in the other WGs. Data collection can be considered as a part of model LCM, and can be used either for monitoring of model/functionality performance or for training data collection. Hence, we can expect that new reports and/or specific reporting configurations may be introduced or some of the existing reporting mechanisms will need higher accuracy. In this case, new requirements in RAN4 will be needed. The discussion is rather use-case specific. Additionally, the conclusions will depend on the agreements on the data collection framework currently discussed in RAN2 [2].

A general framework on data collection is discussed in RAN2 and use-case specific data collection needs are discussed in RAN1. There are already RAN4 requirements on MDT in TS 38.133.

Requirements on data collection (e.g., accuracy, specific configurations, time-stamp, etc.) should be discussed in RAN4 based on the outcomes of RAN1 and RAN2 discussions.

Baseline performance

At the RAN4#106-bis-e meeting, the following way forward on the potential reuse of existing legacy performance requirements was captured:

2.1.2 RAN4 Scope and baseline performance Agreement: - Defining AI/ML requirements - For the cases with the existing legacy performance - Take the legacy performance as baseline for existing use cases/procedures/functionalities/measurements that are to be enhanced by AI/ML based methods - FFS how to define “legacy performance” (whether on meeting/exceeding existing RAN4 requirements, or a wider criterion taking into account generalization) - New or enhanced performance requirements/tests could be considered for existing use cases/procedures/functionalities/measurements that are to be enhanced by AI/ML based methods - For the cases without the existing legacy performance - New or enhanced performance requirements/tests could be considered for the use cases/procedures/functionalities/measurements that are to be enhanced by AI/ML based methods - Generalization verification aspects - Study the necessity and feasibility of defining requirements or test to verify the generalization of AI/ML

Firstly, we would like to point out that even when legacy requirements exist for a certain feature/use-case, it does not mean that it can be re-used directly just because there is no guarantee that this legacy implementation is supported by a new device with AI/ML-based implementation. We do not expect that the definition of the requirement/test can necessitate the support of legacy feature by the device (UE and/or gNB). Therefore, legacy performance can be defined based on the features that are mandatory for the UE. For example, if AI/ML -based CSI feedback compression is compared to some legacy CSI feedback approach (Type I or Type II) this kind of legacy feedback should be mandatory for the device. Otherwise, such a comparison would not be possible.

For the cases with the existing legacy performance, only mandatory supported features can be considered for the baseline performance in the requirements.

Secondly, comparison to the legacy performance also necessitates that requirements/tests are arranged in such a way that legacy feature is tested against the AI/ML-enabled feature and in similar conditions. In other words, it will be impossible to compare the performance of the feature just with the outputs (i.e., direct results of inference) of the AI/ML models. Additionally, the results may not be comparable if the legacy feature is tested in one condition (e.g., propagation model is provided by the channel simulator in OTA testing setup) whereas the inputs for AI/ML model are taken from pre-generated dataset.

Indeed, AI/ML- enabled functionality is much more than just AI/ML model. Figure 2 provides a general overview of how ML model is used as a part for ML-enabled feature. The Radio Resource Management (RRM) functionality is shown as an example, but a similar description is applicable to any ML-enabled feature.

Figure 1: Scope of UE testing for one ML-assisted RRM functionality.

The structured use of ML-enabled Features, Functionalities and ML Models has been proposed earlier in RAN1 discussions Error! Reference source not found.. Testing of specific model inference is not sufficient, nor possible in most of the implementation solutions, in RAN4 because the AI/ML Model is only an enabler of the corresponding feature. Utilization of the model in the device also include many other operations such as data pre/post-proceeding, and additional algorithms on top of the AI/ML model. Legacy RAN4 requirements are defined on the feature level.

RAN4 requirements should be defined on the level of Functionality/Feature. A Logical/Physical ML Model can be identified but only as an enabler of the feature/functionality and should not be tested in isolation from the feature/functionality, i.e., model- specific requirements and tests shall be precluded.

Requirements on model generalization performance

Generalization is a new aspect introduced by ML-enabled Features. Traditionally, requirements are formulated so that parametrized/ rule-based features and functionalities are tested in a few typical test conditions e.g., in a few stationary predefined propagation conditions.

For ML models, on the one hand, the range of applicable conditions can be wider than in the original training dataset. On the other hand, when the environment is getting too different from the original design it may result in unreliable inference results. It is hard to define the generalization limits of the ML model.

Changes in the radio conditions might cause either (temporary) degradation of the currently active Functionality or deactivation/switch of the ML-enabled Functionality, or ML Model, and use of a legacy/ fallback algorithm.

- Due to the capability of the ML Models to adapt to/learn from the input dataset, it can be expected that existing tests can be relatively easy applicable to one or several UE side Functionalities and ML models. However, the main ML testability challenge is when the Functionalities and underlying ML Model(s) is(are) used in the conditions different from the ones used for their training.

It is still under discussion in the other WGs, but in our view, Functionalities and Logical Models may be mapped to the (typical) conditions. In general, nothing is stopping to determine the performance of legacy features, if available as a baseline, in the same conditions and compare those. For example, the AI/ML functionality can be tested in different propagation conditions to evaluate its generalization capabilities. The legacy requirement could be defined only for one of these channel conditions. In some sense, the legacy requirement may not be defined, but it is straightforward to acquire performance of legacy feature in different channel condition to establish a baseline for the AI/ML feature.

Generalization capabilities of AI/ML-enabled features can be tested not only in stationary radio conditions (e.g., each applicable conditions separately) but also in scenarios when the radio conditions are changing (e.g., across different applicable conditions). The performance of legacy feature, if available, can be still used as a baseline in various conditions.

High level testing framework

At the RAN4#106-bis-e meeting, the following way on high-level testing framework was captured in the way forward:

2.1.4 High level testing framework Agreement: - RAN4 should design the tests such that performance is guaranteed and to avoid that a UE can easily pass the test but perform poorly in the field. - This framework is not directly enforceable but should be considered for all the tests to be introduced - This also applies to LCM tests, if they are defined.

In our view, the agreement above describes the baseline principles of RAN4 requirements/testing, but it is still lacking the AI/ML specifics. One of such new aspects is that changes to the AI/ML models (e.g., off-line-based updates to already existing models or shipment of new models) and correspondingly to AI/ML-based feature performance is possible in the field, i.e., after the device has been deployed. In this case, testing of AI/ML based models would not even make a lot of sense if the models can be updated later, when the device is already in the field.

The new aspect is that performance of the AI/ML-enabled features can be different in the test environment and in the field because the changes/updated of the underlying physical models can take place.

Therefore, it is not sufficient to design the pre-deployment tests and requirements well enough. It is also necessary to introduce and test new procedures that will ensure that performance of the feature does not change or degrade in the field. One way to achieve that is to implement monitoring procedures as a part of LCM. However, in this case, the degradation of the performance will be only observed only in the real network, i.e., when any performance issues should be avoided.

Monitoring procedures implemented as a part of LCM can only be used to mitigate performance degradation in a reactive manner, i.e., after badly performing functionality/model is rolled out to the devices and degradation is identified in the field.

The only way to avoid possible post-deployment degradation of performance in the field is to extend RAN4 testing to cover the changes in AI/ML enabled features (i.e., due to the changes/updated/introduction of new ML models/functionalities) in the pro-active manner. It is necessary to ensure that the updates have been validated or tested before they are activated at the devices.

RAN4 testing framework should be extended to validate/test post-deployment changes in AI/ML-enabled features before they are activated at the devices, i.e., before possible degradation is observed based on LCM monitoring.

Conclusion

In this paper, we share our view on general aspects of testing of AI/ML enabled features in RAN4. In particular, we discuss a need for requirements on data collection, RAN4 baseline pefromance, and high-level testing framework.

The following observations and proposals were made:

- Training of one-sided UE- or NW-based models can be expected to be done offline and even outside of the 3GPP model. However, training of DUT and TE sides of the two-sided model might need to be coordinated, including the exchange of the information of the counterpart model(s) and common dataset for training.

- For Release 18, RAN4 does not need to study requirements/tests for training for one-sided models. However, for two-sided models, assumptions on model training or model pairing for DUT and TE needs to be clarified in RAN4.

A general framework on data collection is discussed in RAN2 and use-case specific data collection needs are discussed in RAN1. There are already RAN4 requirements on MDT in TS 38.133.

Requirements on data collection (e.g., accuracy, specific configurations, time-stamp, etc.) should be discussed in RAN4 based on the outcomes of RAN1 and RAN2 discussions.

For the cases with the existing legacy performance, only mandatory supported features can be considered for the baseline performance in the requirements.

RAN4 requirements should be defined on the level of Functionality/Feature. A Logical/Physical ML Model can be identified but only as an enabler of the feature/functionality and should not be tested in isolation from the feature/functionality, i.e., model- specific requirements and tests shall be precluded.

Changes in the radio conditions might cause either (temporary) degradation of the currently active Functionality or deactivation/switch of the ML-enabled Functionality, or ML Model, and use of a legacy/ fallback algorithm.

Generalization capabilities of AI/ML-enabled features can be tested not only in stationary radio conditions (e.g., each applicable conditions separately) but also in scenarios when the radio conditions are changing (e.g., across different applicable conditions). The performance of legacy feature, if available, can be still used as a baseline in various conditions.

The new aspect is that performance of the AI/ML-enabled features can be different in the test environment and in the field because the changes/updated of the underlying physical models can take place.

Monitoring procedures implemented as a part of LCM can only be used to mitigate performance degradation in a reactive manner, i.e., after badly performing functionality/model is rolled out to the devices and degradation is identified in the field.

RAN4 testing framework should be extended to validate/test post-deployment changes in AI/ML-enabled features before they are activated at the devices, i.e., before possible degradation is observed based on LCM monitoring.

- References

- R4-2306299, WF on AI/ML RAN4 studies, Qualcomm Incorporated, Electronic Meeting, RAN4#106bis-e, 17 April –26 April, 2023.

- R2-2306437, Progress and Next Steps: Rapporteur's Insights, Ericsson, Qualcomm Incorporated, Incheon, Korea, May 22 – 26, 2023.

- R4-2308602, On Issues Related to Use Cases for AI/ML in NR Air Interface, Nokia, Nokia Shanghai Bell, RAN4#107, Incheon, Korea, May 22 – 26, 2023.

- R4-2308031, On Interoperability and Testability Aspects of AI/ML for NR Air Interface, Nokia, Nokia Shanghai Bell, RAN4#107, Incheon, Korea, May 22 – 26, 2023.