3GPP TSG-RAN WG4 Meeting #107	R4-2308048

Incheon, ROK, 22 – 26 May, 2023

Source:	ZTE Corporation

Title:	Discussion on Network A requirements

Agenda item:	8.26.2.3

Document for:	Approval

Introduction

At RAN 95 meeting the revised WI “Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR” [1] was approved.  In the last meeting, we had the discussion on Network A requirements, and the outcomes were captured in [2]. Based on the outcomes, the following issues need to be further discussed.

- Parameters for L1/L3 measurement requirements

- In this paper we will continue to discuss the related issues and provide our views on the above issue.

Discussion

Issue 3-1-1: Principle on layer 1 and layer 3 measurement requirements after gap collision handling - Proposals - P1: Frameworks of LBT failure in NR-U design can be used as starting point when discussing NW A L3/L1 requirement impact due to MUSIM gaps (Apple) - P2: Reuse the same principle of Rel-17 concurrent gaps WI to define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Ntotal /Navailable for network A requirements when MUSIM gaps are configured. (CMCC xiaomi vivo ZTE oppo Huawei Ericsson Qualcomm MTK Nokia) - P3: RAN4 to postpone the detail NW-A’s requirement discussion until RAN4 achieves the consensus on MUSIM gaps’ priority. (Ericsson Nokia) Agreements: No Candidate options: Recommendations: Discuss at the next meeting Issue 3-1-2: On parameters for L1/L3 measurement requirements - Proposals - P1: (CMCC): - For L3 measurement, Navailable need to be updated to cover MUSIM gaps - For L1 measurement, Noutside_MG and Navailable need to be updated to cover MUSIM gaps - P2: The following parameters need to be updated to account for collisions with MUSIM gaps (MTK): - Kp for intra-frequency and inter-frequency measurements without gaps (Xiaomi) - Kgap for intra-frequency and inter-frequency measurements with gaps (Xiaomi) - Kgap_EUTRA for inter-RAT measurements - Kp_CSI-RS for CSI-RS L3 measurements - Kp,PRS,I for NR positioning measurements - CSSFintra for intra-frequency measurements - CSSFinter for intra-frequency measurements - CSSFinterRAT for intra-RAT measurements - P scaling factor for L1-RSRP and L1-SINR measurements (xiaomi) - P3: Suggest the following update on parameters for L1/L3 measurement requirements (vivo ZTE) - For SSB based or CSI-RS based RLM, BFD and CBD, scaling factor P can be reused without any update, the definition of Ntotal, Noutside_MG and Navailable need updated. - For intra-frequency and inter-frequency measurements without gaps, scaling factor Kp can be reused and the definition of Ntotal and Navailable will be updated. - For intra-frequency and inter-frequency measurements with gaps, scaling factor Kgap can be reused and the definition of Ntotal and Navailable will be updated. - For inter-RAT E-UTRAN TDD/FDD measurement, scaling factor Kgap_EUTRA can be reused and the definition of Ntotal and Navailable will be updated. - For L1-RSRP/L1-SINR measurement, scaling factor P can be reused and the definition of Ntotal, Noutside_MG and Navailable will be updated. - For NR measurement for positioning, scaling factor Kp,PRSIi can be reused and the definition of Ntotal and Navailable will be updated. - For CSI-RS based L3 measurements, scaling factor Kp_CSI-RS can be reused and the definition of Ntotal and Navailable will be updated. - P4: (Huawei) - For L3 measurement outside MG, Kp in the requirements is updated - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion or non-dropped MUSIM gap occasion within the window W. - For L3 and positioning measurement with MG, existing requirements can be re-used. - For L1 measurement outside MG, Navailable, Noutside_MG in the requirements are updated - Noutside_MG is the number of SSB resource occasions that are not overlapped with any MG nor MUSIM gap within the window W - Navailable is the number of SSB resource occasions that are not overlapped with any MG, MUSIM gap nor any SMTC occasion within the window W - P5: Clarification for L3 measurement without gap, SMTC should not be fully overlapping with MUSIM gap (oppo) - P6: (Qualcomm) For intra-frequency and inter-frequency measurements without gaps in network A, modify the scaling factor Kp as follows: - The duration of the window W is max(SMTC period, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer - Ntotal is the total number of SMTC occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions For inter-frequency and inter-frequency measurements with gaps in network A, modify the scaling factor Kgap as follows: - The duration of the window W is max(SMTC period, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer - Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions - Navailable is the number of SMTC occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions For inter-RAT measurements with gaps in network A, modify the scaling factor Kgap_EUTRA as follows: - The duration of the window W is MGRP_max, where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within FR1 - Ntotal is the total number of associated measurement gap occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions - Navailable is the number of non-dropped associated measurement gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions For CSI-RS L3 intra-frequency measurements without gaps modify the scaling factor Kp_CSI-RS as follows: - The duration of the window W is max(CSI-RS period, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer - Ntotal is the total number of CSI-RS occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions - Navailable is the number of CSI-RS occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions For CSI-RS L3 inter-frequency measurements with gaps modify the scaling factor Kp_CSI-RS as follows: - The duration of the window W is max(CSI-RS period, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer - Ntotal is the total number of CSI-RS occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions - Navailable is the number of CSI-RS occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions For NR positioning measurements with gaps modify the scaling factor Kp,PRS,i as follows: - The duration of the window W is max(TPRS,i, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the positioning frequency layer - Ntotal is the total number of associated measurement gap occasions covering PRS occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions - Navailable is the number of non-dropped associated measurement gap occasions covering PRS occasions within the window W, after accounting for measurement gap and MUSIM gap collisions Clarify the definition of CSSFintra for intra-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted. Clarify the definition of CSSFinter for inter-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted. Clarify the definition of CSSFinterRAT for intra-RAT measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted. For L1-RSRP and L1-SINR measurements modify the scaling factor P as follows: - The duration of the window W is max(TL1, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the serving cell - Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions - Noutside_MG is the total number of SSB resource occasions that do not overlap with measurement gap occasions, MUSIM gap occasions nor SMTC occasions within the window W - Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions Agreements: No Candidate options: Recommendations: postpone the discussion until issue 3-1-1 is solved. Issue 3-1-3: On the time window W for aperiodic gap - Proposals - P1: Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer. (vivo xiaomi oppo Huawei Nokia Qualcomm) - P2: max(SMTC period, MGRP_max)+[M], where MGRP_max is the largest periodicity among all the periodic gaps and [M] is a time margin for the one-shot aperiodic MUSIM gap. (MTK) Agreements: P1 - Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer Candidate options: Recommendations: Close this issue.

In the last meeting, we didn’t discuss the Network A requirements because of the limited time, but as suggested by moderator, we shall continue to discuss it. What the reason for studying the NW A requirements is that the configured MUSIM gaps may collide with layer3 or layer 1 resources in NW A. The collision will impact RRM requirements in NW A. Most companies consider the principle on layer 1 and layer 3 measurement requirements after gap collision handling and the majority of them agreed that we should reuse the principle used in Rel-17 concurrent gaps as baseline to define NW A L1/L3 measurement requirements when MUSIM gaps are configured.

TS 38.133 defines the concrete factors on L3 measurement as below:

When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps, Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows: - For a window W of duration max(SMTC period, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and/or per-FR measurement gap within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion: - Ntotal is the total number of SMTC occasions within the window, including those overlapped with measurement gap occasions within the window, and - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.8.3. Kp = 1 when Navailable = 0.

The scaling factor is defined by the Kp and window W, that is, is defined by the total number of measurement occasion (Ntotal ) in a window W and the available number of measurement occasion(Navailable) in the same window W.Based on these, we suggest that we shall reuse the R17 con-MG outcomes as baseline and only add the MUSIM gaps for MUSIM gaps. So option 2 is fine to us.

Proposal 1: Reuse the principle used in Rel-17 concurrent gaps as the baseline to define network A L1/L3 measurement requirements when MUSIM gaps are configured.

Proposal 2: On L3 measurements: the scaling factors shall be re-defined for MUSIM gaps impact with reusing the R17 con-MG principle, as below:

- Intra-frequency (without gap):

- Kp=Ntotal/Navailable

- Ntotal is the total number of SMTC occasions within the window W, including those overlapped with MGs and MUSIM gaps within the window.

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion and  non-dropped MUSIM gap occasions within the window W.

- Kp = 1 when Navailable = 0.

- The duration of the window W equals max{SMTC period, MGRP_max, MUSIM gap period}.

- Inter-frequency:

Kgap=Ntotal/Navailable

- Ntotal is the total number of SMTC occasions that are covered by instances of the associated MG within the window W, including those overlapped with other MGs and MUSIM gaps within the window.

- Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated MG within the window W.

- c.    The duration of the window W equals max{SMTC period, MGRP_max, MUSIM gap period}.

TS 38.133 defines the concrete factors on L1 measurement as below:

When UE supports concurrent measurement gap and concurrent gaps are configured, - P value for SSB resource to be measured is defined as - Ntotal / Noutside_MG in FR1 - Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0 - Ntotal / Navailable in FR2 with Navailable > 0 - For a window W of duration max(TL1, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion: - Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and - Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W - Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W - TL1 is periodicity of the target SSB.

- The same way in L1 measurement.

Proposal 3: On L1 measurements: the scaling factors shall be re-defined for MUSIM gaps impact with reusing the R17 con-MG principle, as below:

P value for SSB resource to be measured is defined as

Ntotal / Noutside_MG in FR1

Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

Ntotal / Navailable in FR2 with Navailable > 0

where,

- Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions,MUSIM gaps or SMTC occasions within the window, and

- Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor MUSIM gap occasion within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion, MUSIM gap nor any SMTC occasion within the window W

- TL1 is periodicity of the target SSB.

- The duration of the window W equals max{SMTC period, MGRP_max, MUSIM gap period}.

Conclusion

In this paper we provided our views on NW A requirements related to MUSIM gaps.

Proposal 1: Reuse the principle used in Rel-17 concurrent gaps as the baseline to define network A L1/L3 measurement requirements when MUSIM gaps are configured.

Proposal 2: On L3 measurements: the scaling factors shall be re-defined for MUSIM gaps impact with reusing the R17 con-MG principle, as below:

- Intra-frequency (without gap):

- Kp=Ntotal/Navailable

- Ntotal is the total number of SMTC occasions within the window W, including those overlapped with MGs and MUSIM gaps within the window.

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion and  non-dropped MUSIM gap occasions within the window W.

- Kp = 1 when Navailable = 0.

- The duration of the window W equals max{SMTC period, MGRP_max, MUSIM gap period}.

- Inter-frequency:

Kgap=Ntotal/Navailable

- Ntotal is the total number of SMTC occasions that are covered by instances of the associated MG within the window W, including those overlapped with other MGs and MUSIM gaps within the window.

- Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated MG within the window W.

- The duration of the window W equals max{SMTC period, MGRP_max, MUSIM gap period}.

Proposal 3: On L1 measurements: the scaling factors shall be re-defined for MUSIM gaps impact with reusing the R17 con-MG principle, as below:

P value for SSB resource to be measured is defined as

Ntotal / Noutside_MG in FR1

Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

Ntotal / Navailable in FR2 with Navailable > 0

where,

- Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions,MUSIM gaps or SMTC occasions within the window, and

- Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor MUSIM gap occasion within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion, MUSIM gap nor any SMTC occasion within the window W

- TL1  is periodicity of the target SSB.

- The duration of the window W equals max{SMTC period, MGRP_max, MUSIM gap period}.

- References

- RP-221018, New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, MediaTek Inc, Intel Corporation

- R4-2303248, WF on NR Dual Tx/Rx Multi-SIM,Vivo