3GPP TSG-RAN WG4 Meeting # 107	R4-2308121

# Incheon, Korea, 22th - 26th, May, 2023

Source: 	Samsung, KT corporation

Title: 	Views on signaling information for lower MSD

Agenda Item:	8.5.1.3.1

Document for:	Discussion

- Introduction

In this paper, we share our views on the remaining issues of lower MSD which are captured in [1].

- 2. Discussion

2.1 Conditions to indicate Lower MSD capability

The WF in last meeting is to further discuss below options:

- Candidate options

- Option 1: For the purpose of MSD improvement, if the minimum requirement for a given REFSENS exception case falls into the interval of MSD ≤ Thi dB, the actual MSD should be at least one-level lower (i.e., actual MSD ≤ Thi-1 dB) in order for the UE to report the low-MSD capability. If the actual MSD is larger than the maximum threshold ThM-1 (i.e. out of range), the UE cannot report low-MSD capability for this REFSENS exception case (Samsung, Xiaomi, Nokia, AT&T, Skyworks, HW)

- If UE reports the lower MSD capability, the reported MSD value should be improved at least by TBD dB against a specified MSD

- Option 2: For the purpose of MSD improvement, if the minimum requirement for a given REFSENS exception case falls into the interval of MSD ≤ Thi dB, the actual MSD should be at least one-level lower (i.e., actual MSD ≤ Thi-1 dB) in order for the UE to report the low-MSD capability. If the actual MSD is larger than the maximum threshold ThM-1 (i.e. out of range), the UE cannot report low-MSD capability for this REFSENS exception case (QC, OPPO, vivo)

- If UE reports the lower MSD capability, the reported MSD value should be improved at least by TBD dB against a specified MSD

- Option 3: Others (Meta, MediaTek, Apple)

Option 1 is preferred by us which is intended to prohibit UE from indicating lower MSD capability when the specified MSD itself is small or the MSD improvement is small. The difference of Option 1 and Option 2 is whether the sub-bullet “If UE reports the lower MSD capability, the reported MSD value should be improved at least by TBD dB against a specified MSD” is applied, in our view one purpose of this sub-bullet is for the case that the actual MSD is just slightly larger than the maximum lower MSD threshold, then this condition is needed to prohibit UE from indicating lower MSD capability when the MSD improvement is small.

We can generally categorize the actual MSD into below three cases:

- Actual MSD＞the maximum lower MSD capability threshold

- Actual MSD ＜The minimum lower MSD capability threshold

- Actual MSD falls into the interval of two adjacent lower MSD capability thresholds.

Thus, if the main bullet of Option 1 is agreed, the sub-bullet should also be agreed as a package, otherwise the condition for lower MSD indication for case 1) is incomplete. In addition, below principles could be turned into spec language inserted into MSD threshold table as notes.

| Specified MSD | Condition to report lower MSD capability |
| --- | --- |
| ＞The maximum lower MSD threshold | - The actual MSD should be at least less than the maximum lower MSD threshold - The actual MSD should be improved at least by X dB against a specified MSD |
| ＜The minimum lower MSD threshold Note: If the minimum lower MSD threshold is 0, then this case is not needed. | No need to report lower MSD capability |
| Fall into the interval of two adjacent lower MSD thresholds | - The actual MSD should be at least one-level lower than the specified MSD in terms of lower MSD capability class - The actual MSD should be improved at least by X dB against a specified MSD |
| Note: The exact value of X should be determined after the lower MSD thresholds are concluded |  |

Proposal 1: It is suggested to adopt above table as condition to report lower MSD capability.

2.2 MSD orders

- WF

- The MSD source to be reported is selected from the set of {UL harmonic, Harmonic mixing, cross-band ISO, IMDn}.

  - FFS handling of UL/DL harmonic order

  - FFS on max MSD order n

  - If new MSD type is identified and specified by RAN4 in the future, the new MSD type can also be considered for indicating lower MSD capability

It was agreed in RAN4#106 all MSD mechanism and orders could be considered for lower MSD capability indication, but it does not mean the orders is necessary to be indicated. We think the orders for harmonic/harmonic mixing/cross band isolation is not necessary to be reported, since the order for these three MSD mechanism is predefined and only in the spec (one exception is harmonic for n5-n78 which suffers two harmonic order, which could be deemed as one exception as a rare case, since only the two bands are far separated from each other has a chance to suffer two orders), and NW also does not care about the order in our view.

Proposal 2: No need to report order for harmonic/ harmonic mixing/cross band isolation

In terms of the max order n for IMD, currently the maximum order specified in spec for IMD is 9 which is for intra-band contiguous ULCA, the values are all under 2.7dB for PC2 and PC3 as well. It is also observed all MSD values for IMD7 is under 8.5dB for PC3 and PC2. However, it is unclear now the value would be how much larger for PC1.5.

In addition, as captured in [3], the MSD analysis for IMD should be performed at least up to IMD13 for contiguous ULCA and IMD11 for non-contiguous UL CA if filter rejection is less than 20dB.

With above, we think take 13 as the maximum IMD order would be safer, since if 9 is taken, only 3-bit might be reserved for IMD2 to IMD9 in RAN2 which is not future-proof.

Proposal 3: Take 13 as the maximum order for IMD.

2.3 Candidate MSD thresholds

Our preference is 3-bit solution as shown in below table, to better serve the purpose one table applicable to all MSD mechanism and power classes.

| Bit | Maximum allowed actual MSD (i.e. Thresholds) | Lower MSD Capability classes | Note |
| --- | --- | --- | --- |
| 000 | 0dB | Ⅰ | Actual MSD = 0 |
| 001 | 3 dB | Ⅱ | Actual MSD ≤ 3 |
| 010 | 6 dB | Ⅲ | Actual MSD ≤ 6 |
| 011 | 9 dB | IV | Actual MSD ≤ 9 |
| 100 | 12 dB | Ⅴ | Actual MSD ≤ 12 |
| 101 | 15 dB | Ⅵ | Actual MSD ≤ 15 |
| 110 | 18 dB | Ⅶ | Actual MSD ≤ 18 |
| 111 | 21dB | Ⅷ | Actual MSD ≤ 21 |

Proposal 4: Our preference of lower MSD thresholds is 3-bit solution with maximum threshold around 20dB.

2.4 Conformance test for lower MSD

As illustrated in our paper [2], there is a common issue for both specified MSD and lower MSD capability in terms of conformance test, i.e., when UE does not support the specified worst case configuration, or even does not support any of the specified configurations due to lack of support of the channel bandwidth specified in the test configuration, in this case which UL/DL configuration and MSD value to assume for conformance test?

For the specified MSD conformance test issue, it is a legacy one(probably since Rel-17), a LS was sent out to RAN5 [4] in last meeting with three scenarios identified, part of content of this LS is reproduced as below, RAN4 make some recommendation for each case for RAN5 further check. Furthermore, please note that these principles are also applicable for lower MSD capability.

Additionally, the following principles as one possible future-proof methodology applicable to Rel-17 and onwards have been discussed in RAN4, which just provide some information on what RAN4 is thinking and no intention to have impact on RAN5’s decision.

For certain MSD mechanism of one band combination

- In case UE supports the specified worst case configuration which corresponds to the largest MSD, this specified worst case configuration is supposed to be selected for conformance test;

- In case UE does not support the specified worst case configuration which corresponds to the largest MSD, but the second configuration (if specified) which is an optionally defined one,  the specified second configuration is supposed be selected for conformance test;

- In case UE does not support any of the specified configurations, the worst case configuration the UE supported itself for this band combination could be selected as test configuration and should conform to the largest MSD specified.

Actually the principle behind above recommendation is also no additional (new) conformance test point be set for lower MSD capability against specified MSD. More specifically.

- In case UE supports the specified worst case configuration which corresponds to the largest MSD, this configuration is selected as test configuration for verifying both existing specified MSD and lower MSD capability → No additional test point needed for lower MSD compared with existing specified MSD

- In case UE does not support the specified worst case configuration, but support the second test configuration (if introduced )which is an optionally defined one to address operator’s demand, the second configuration is selected as test configuration for verifying both existing specified MSD and lower MSD capability → No additional test point needed for lower MSD compared with existing specified MSD

- In case UE does not support any of the specified configuration, the worst case configuration the UE supported itself for this band combination should be chosen as test configuration for verifying both existing specified MSD and lower MSD capability → No additional test point needed for lower MSD compared with existing specified MSD

Proposal 5: No additional (new) conformance test point be set for lower MSD capability against specified MSD. More specifically:

- In case UE supports the specified worst case configuration which corresponds to the largest MSD, this configuration is selected as test configuration for verifying both existing specified MSD and lower MSD capability

- In case UE does not support the specified worst case configuration, but support the second test configuration (if introduced )which is an optionally defined one to address operator’s demand,  the second configuration is selected as test configuration for verifying both existing specified MSD and lower MSD capability

- In case UE does not support any of the specified configuration, the worst case configuration the UE supported itself for this band combination should be chosen as test configuration for verifying both existing specified MSD and lower MSD capability

Note: Whether 1)2)3) is valid, should wait for RAN5’s final confirmation.

2.5 Whether to report CBW of aggressor UL and victim DL

If the conformance test configuration for Lower MSD and specified MSD described in Proposal 5 can be agreed, then it is expected the condition (the principles in Proposal 5) to derive and verify lower MSD capability for a band combination would be clarified in RAN4 spec with some general description or some notes added in the REFERENS exception tables.

With that, it would be clear to NW what is the aggressor UL and victim DL CBW for one band combination in terms of lower MSD capability. For example in case a UE does not support any specified configuration, the NW is still aware that the lower MSD capability for this UE for certain MSD mechanism is attached to the worst case configuration the UE supported itself for this MSD mechanism.

Proposal 6: If Proposal 5 is agreeable, then correspondingly CBW of aggressor UL and victim DL are not necessary to be included in the essential information for lower MSD capability.

- 3. Conclusion

Proposal 1: It is suggested to adopt below table as condition to report lower MSD capability.

| Specified MSD | Condition to report lower MSD capability |
| --- | --- |
| ＞The maximum lower MSD threshold | - The actual MSD should be at least less than the maximum lower MSD threshold - The actual MSD should be improved at least by X dB against a specified MSD |
| ＜The minimum lower MSD threshold Note: If the minimum lower MSD threshold is 0, then this case is not needed. | No need to report lower MSD capability |
| Fall into the interval of two adjacent lower MSD thresholds | - The actual MSD should be at least one-level lower than the specified MSD in terms of lower MSD capability class - The actual MSD should be improved at least by X dB against a specified MSD |
| Note: The exact value of X should be determined after the lower MSD thresholds are concluded |  |

Proposal 2: No need to report order for harmonic/ harmonic mixing/cross band isolation.

Proposal 3: Take 13 as the maximum order for IMD.

Proposal 4: Our preference of lower MSD thresholds is 3-bit solution with maximum threshold around 20dB.

Proposal 5: No additional (new) conformance test point be set for lower MSD capability against specified MSD. More specifically:

- In case UE supports the specified worst case configuration which corresponds to the largest MSD, this configuration is selected as test configuration for verifying both existing specified MSD and lower MSD capability

- In case UE does not support the specified worst case configuration, but support the second test configuration (if introduced )which is an optionally defined one to address operator’s demand,  the second configuration is selected as test configuration for verifying both existing specified MSD and lower MSD capability

- In case UE does not support any of the specified configuration, the worst case configuration the UE supported itself for this band combination should be chosen as test configuration for verifying both existing specified MSD and lower MSD capability

Note: Whether 1)2)3) is valid, should wait for RAN5’s final confirmation.

Proposal 6: If Proposal 5 is agreeable, then correspondingly CBW of aggressor UL and victim DL are not necessary to be included in the essential information for lower MSD capability.

- 4. Reference

[1] R4-2306590, WF on lower MSD, Huawei, RAN4#106bis-e

[2] R4-2304717 Views on signaling design for lower MSD, Samsung, RAN4#106bis-e

[3] R4-2105336 Way forward on MPR and AMPR PC2 for intra-band UL contiguous CA, RAN4#98bis-e, Skyworks

[4] R4-2306591 LS on clarification of test configurations for CADC MSD requirements, Samsung, RAN4#106bis-e