3GPP TSG-RAN4 Meeting #107												R4-2308190

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Agenda item:	8.29.6

Source: 	CMCC

Title: 	Discussion on NCR demod requirements

Document for:	Discussion

# 1. Introduction

In last RAN4 meeting, a WF is approved [1]. In this meeting, we focus on the discussion of NCR remaining demod issues.

- Additional CBW 5MHz/15kHz

- new requirements on PDCCH/PDSCH for signaling of Access link beam change indication

# 2. Discussion

- Additional CHBW 5MHz/15kHz

The agreement in last meeting is listed as below:

• Define new requirements for FDD/15 kHz and TDD/30 kHz for 1 layer only by reusing/adapting requirements for IAB-MT. o 10M/15kHz; 40M/30kHz o FFS whether need to consider additional CHBW 5MHz/15kHz

10MHz/15kHz and 40M/30kHz are reusing the same value as from IAB-MT spec. For IAB-MT it only supports n41, n77, n78 and n79. At the stage when defining IAB-MT demod requirements, 10MHz and 40MHz are the specified minimum CBW for n41, n77, n78 and n79 respectively. But for NCR, it is applicable for all FR1 operation bands and 5MHz is the minimum CBW for certain operation bands. Therefore, additional 5MHz is suggested to extend demod requirements to any CBW configuration.

Proposal 1: it’s suggested to consider additional CBW 5MHz/15kHz considering 5MHz is minimum CBW for certain NCR operation bands.

- new requirements on PDCCH/PDSCH for signaling of Access link beam change indication

The agreement in last meeting is listed as below:

Issue 1-1-3: Whether define new requirements for FR2 Agreement: • Reuse IAB-MT requirements for NCR-MT demodulation and only consider Rank 1 with 100MHz/120kHz • FFS new requirements on PDCCH/PDSCH for signaling of Access link beam change indication

In RAN1 agreements, access link beam change indication is not dedicated for FR2. Instead, it is applicable for both FR1 and FR2. Besides, in RAN4 RF requirements, 1-O and 1-H NCR-fwd are defined which implicitly means FR1 also support beam forming for forwarding access part. Therefore, requirements for signaling of access link beam change indication should be applicable for both FR1 and FR2.

Proposal 2: requirements for signaling of access link beam change indication should be applicable for both FR1 and FR2.

- test metric of new test case for specifically for MAC-CE Access link beam indication transmitted over PDSCH if introduced

The agreement in last meeting is listed as below:

Issue 1-2-1: How to define PDSCH requirements Agreements: • Both 70% and 30% throughput cases agreed as baseline assumption for the test cases reused from IAB-MT requirements o Other values higher than 70% not precluded • FFS for the test metric of new test case for specifically for MAC-CE Access link beam indication transmitted over PDSCH if introduced

Very high demod accuracy is required to help NCR point at where serving UE is. interference occurs for other UE if MT point at this UE using wrong beam direction due to wrong demod of beam indication. besides, wrong beam direction will enlarge UE connection delay and may lead to interruption at UE side. therefore, it’s much important to make sure MT would accurately demod access indication information and avoid severe interference.

Observation 1: it’s much important to make sure MT would accurately demod access indication information to avoid connection interruption or severe interference to other UE due to wrong beam direction.

For NCR-MT, the MAC CE will only carry beam related side control information, compared with throughput, BLER is much directly and significant for this kind of information although it is transmitted through PDSCH. In addition to, 0.3 BLER which equals to legacy 70% throughput loss is not enough because it will introduce severe interference for other UE and also enlarge serving UE connection delay due to wrong beam direction.

One suggestion is to use BLER as test metric for MAC-CE access link beam indication transmitted over PDSCH. For example, 1% BLER is suggested.

Proposal 3: one suggestion is to use 1% BLER as test metric for MAC-CE access link beam indication transmitted over PDSCH.

- adapt PDCCH requirement for testing of DCI type 5_0 used for access link beam change indication

The agreement in last meeting is listed as below:

Issue 1-3-1: How to define PDCCH requirements Agreements: • Reuse UE requirements for FDD and TDD • FFS for adapt PDCCH requirement for testing of DCI type 5_0 used for access link beam change indication.

Latest RAN1 agreements of DCI format 2_8 to indicate non-periodic beam indication is listed as below. It’s noted RAN1 approve to use DCI format 2_8 rather than 5_0 according to newest approved agreements [2].

##### 7.3.1.3.9 Format 2_8 DCI format 2_8 is used for notifying the aperiodic beam indication and associated time resources The following information is transmitted by means of the DCI format 2_8 with CRC scrambled by NCR-RNTI: - Beam index 1, Beam index 2, …, Beam index N The bitwidth of each beam index field is determined by the higher layer parameter ncr-AperiodicBeamFieldWidth. - Time resource indication 1, Time resource indication 2, …, Time resource indication N The bitwidth of each time resource indication field is determined by max log2I,1, where I is the number of time domain resources configured by ncr-AperiodicFwdConfig. The bit field indexes of a time resource indication field are mapped to the time domain resources configured by ncr-AperiodicFwdConfig according to an ascending order of a resource identity configured by ncr-AperiodicFwdTimeResourceId, with the bit field index 0 mapped to the time resource with the smallest resource identity. The N beam indexes are sequentially associated with the N time resource indications with one to one mapping. N is configured by the higher layer parameter numberOfFields. The size of DCI format 2_8 is up to 128 bits.

It seems the main difference between format 2_8 and 1_0/1_1 is the payload size difference which need to wait for final conclusion of above parameter ncr-AperiodicBeamFieldWidth, ncr-AperiodicFwdConfig, numberOfFields. so RAN4 needs to wait for RAN1/2 conclusion before make conclusion about whether to adapt current requirements for 40MHz CBW PDCCH.

Observation 2: RAN1 use DCI format 2_8 rather than 5_0 to indicate non-periodic beam indication. The main difference from 1_0/1_1 is the payload side difference.

- whether need to define PMI reporting requirements

The agreement in last meeting is listed as below:

Issue 2-2-1: Whether define PMI requirements Agreements: • FFS whether need to define PMI reporting requirements

from our understanding, it’s better to define PMI reporting requirements since actual implementation also support this functionality. explanations are listed as below:

- it is possible that NCR-MT is deployed with variable propagation environment to gNB. Usually, the NCR-MT would be deployed LOS to gNB with very good propagation environment. But we should also note that at environment where NCR-MT is deployed, propagation loss may also be variable and not LOS to gNB due to limited deployment places. For example, NCR-MT may be deployed outside window rather than on roof of building due to window is more feasible to deploy NCR to let MT outdoor and forwarding-link access part indoor, there may be shelters between NCR-MT and gNB and then NCR-MT can’t point directly at gNB with LOS. therefore, it’s much safe to consider time variable propagation environment rather than only focus on stable LOS link, PMI reporting is needed.

- PMI is defined to improve SINR. For some side information, especially access beam indication information, higher SINR is required to avoid miss/wrong control information detection.

Proposal 4: it’s suggested to define PMI reporting requirements for NCR-MT.

# 3. Conclusions

In this contribution, NCR requirements are discussed with following observations and proposals:

Proposal 1: it’s suggested to consider additional CBW 5MHz/15kHz considering 5MHz is minimum CBW for certain NCR operation bands.

Proposal 2: requirements for signaling of access link beam change indication should be applicable for both FR1 and FR2.

Observation 1: it’s much important to make sure MT would accurately demod access indication information to avoid connection interruption or severe interference to other UE due to wrong beam direction.

Proposal 3: one suggestion is to use 1% BLER as test metric for MAC-CE access link beam indication transmitted over PDSCH.

Observation 2: RAN1 use DCI format 2_8 rather than 5_0 to indicate non-periodic beam indication. The main difference from 1_0/1_1 is the payload side difference.

Proposal 4: it’s suggested to define PMI reporting requirements for NCR-MT.

# 4. Reference

[1] R4-2305934, WF on NCR-MT demodulation requirements, ZTE Corporation

[2] R1-2304264, Introduction of Rel-18 network controlled repeaters, Huawei