3GPP TSG-RAN4 Meeting #107												R4-2308192

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Agenda item:	8.14.1.4

Source: 	CMCC

Title: 	Discussion on ATG simulation assumption

Document for:	Discussion

# 1. Introduction

In last meeting, a WF on ATG simulation scenario has been approved [1]. In this contribution, we focus on the remaining simulation parameter discussion. The remaining issue is how to simulate the worst deployment case for non-synchronized gNB-gNB interference.

# 2. Discussion

Following lists the simulated ACLR and ACS for ATG BS and ATG UE among scenarios 1-4 and 9-12 for synchronization case. according to simulation results, legacy ACLR and ACS requirements for TN BS and TN UE is enough for ATG BS and ATG UE to avoid interference.

Proposal 1: legacy TN BS ACLR and ACS value could be reused for ATG BS.

Proposal 2: legacy UE ACLR and ACS value could be reused for ATG UE.

## Simulation scenario 1

1. Final simulation results table

| Company | Simulation scenarios | Throughput loss, detailed explanation in sub-topic 2-3 | Relative ATG BS ACLR offset(dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | -20 | -10 | 0 | 10 | 20 | 30 | 40 | 50 |  |
| CMCC | 1 (4GHz ATGDLTNDL) | 5% in the whole network | 0.00127 | 0.00028 | 0.00018 | 0.00017 | 0.00017 | 0.00017 | 0.00017 | 0.00017 |  |
|  |  | 50% in the whole network | 3.25e-08 | 7.22e-09 | 4.71e-09 | 4.45e-09 | 4.43e-09 | 4.43e-09 | 4.43e-09 | 4.43e-09 |  |
|  |  | 5% of users within the cell with largest throughput loss for the case of TN DL victim | 29.0094 | 9.97200 | 6.97549 | 6.56611 | 6.53312 | 6.52981 | 6.52948 | 6.52945 |  |
|  |  | 50% of users within the cell with largest throughput loss for the case of TN DL victim | 0.08720 | 0.01852 | 0.01190 | 0.01126 | 0.01114 | 0.01114 | 0.01114 | 0.01114 |  |
| Note 1: offset 0 means ATG ACLR/ACS=legacy ACLR ACS. There is no uniform ACLR/ACS offset step suggestion. |  |  |  |  |  |  |  |  |  |  |

2. Final simulation results plot

ATG BS ACLR value that meets the 5% throughput loss requirement(average or worst)

Note：P1 users within the cell with largest throughput loss for average；P2 the whole network for average

## Simulation scenario 2

1. Final simulation results table

| Company | Simulation scenarios | Throughput loss, detailed explanation in sub-topic 2-3 | Relative ATG UE ACLR offset |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | -30 | -25 | -20 | -15 | -10 | -5 | 0 |
| CMCC | 2 (4GHz ATGULTNUL) | 5% in the whole network | 3.60748 | 1.17673 | 0.37612 | 0.11932 | 0.03783 | 0.01203 | 0.00387 |
|  |  | 50% in the whole network | 0.00013278 | 4.199e-05 | 1.328e-05 | 4.202e-06 | 1.331e-06 | 4.232e-07 | 1.361e-07 |
|  |  | 5% of users within the cell with largest throughput loss for the case of TN DL victim | 61.3863 | 29.5883 | 11.3904 | 3.90913 | 1.27238 | 0.40810 | 0.13162 |
|  |  | 50% of users within the cell with largest throughput loss for the case of TN DL victim | 2.84800 | 0.91923 | 0.29265 | 0.09279 | 0.02941 | 0.00935 | 0.00301 |
| Note 1: offset 0 means ATG ACLR/ACS=legacy ACLR ACS. There is no uniform ACLR/ACS offset step suggestion. |  |  |  |  |  |  |  |  |  |

2. Final simulation results plot

ATG BS ACLR value that meets the 5% throughput loss requirement(average or worst)

## Simulation scenario 3

1. Final simulation results table

| Company | Simulation scenarios | Throughput loss, detailed explanation in sub-topic 2-3 | Relative ATG UE ACS offset |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | -30 | -25 | -20 | -15 | -10 | -5 | 0 | 5 | 10 |
| CMCC | 3 (4GHz TNDLATGDL) | 5% in the whole network | 85.3387 | 72.8419 | 57.4920 | 41.7638 | 26.4825 | 13.3474 | 5.44959 | 1.62034 | 0.71596 |
|  |  | 50% in the whole network | 35.8428 | 20.9282 | 9.00668 | 2.77994 | 0.58819 | 0.07370 | 0.01538 | 0.00414 | 0 |
| Note 1: offset 0 means ATG ACLR/ACS=legacy ACLR ACS. There is no uniform ACLR/ACS offset step suggestion. |  |  |  |  |  |  |  |  |  |  |  |

2. Final simulation results plot

ATG BS ACLR value that meets the 5% throughput loss requirement(average or worst)

## Simulation scenario 4

1. Final simulation results table

| Company | Simulation scenarios | Throughput loss, detailed explanation in sub-topic 2-3 | Relative ATG BS ACS offset |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | -40 | -35 | -30 | -25 | -20 | -15 | -10 | -5 | 0 |
| CMCC | 4 (4GHz TNULATGUL) | 5% in the whole network | 68.0318 | 41.4734 | 19.2070 | 7.59788 | 3.13830 | 1.63030 | 1.14537 | 0.99075 | 0.94169 |
|  |  | 50% in the whole network | 0.53644 | 0.17206 | 0.05598 | 0.01918 | 0.00753 | 0.00385 | 0.00268 | 0.00232 | 0.00220 |
| Note 1: offset 0 means ATG ACLR/ACS=legacy ACLR ACS. There is no uniform ACLR/ACS offset step suggestion. |  |  |  |  |  |  |  |  |  |  |  |

2. Final simulation results plot

ATG BS ACLR value that meets the 5% throughput loss requirement(average or worst)

## Simulation scenario 9

1. Final simulation results table

| Company | Simulation scenarios | Throughput loss, detailed explanation in sub-topic 2-3 | Relative ATG BS ACLR offset(dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | -20 | -10 | 0 | 10 | 20 | 30 | 40 | 50 |
| CMCC | 9 (2GHz ATGDLTNDL) | 5% in the whole network | 0.00155 | 0.00035 | 0.00023 | 0.00021 | 0.00021 | 0.00021 | 0.00021 | 0.00021 |
|  |  | 50% in the whole network | 2.69e-07 | 6.00e-08 | 3.91e-08 | 3.70e-08 | 3.68e-08 | 3.68e-08 | 3.68e-08 | 3.68e-08 |
|  |  | 5% of users within the cell with largest throughput loss for the case of TN DL victim | 22.4757 | 6.89439 | 4.69931 | 4.47412 | 4.45146 | 4.44919 | 4.44897 | 4.44894 |
|  |  | 50% of users within the cell with largest throughput loss for the case of TN DL victim | 0.03207 | 0.00709 | 0.00462 | 0.00437 | 0.00435 | 0.00435 | 0.00435 | 0.00435 |
| Note 1: offset 0 means ATG ACLR/ACS=legacy ACLR ACS. There is no uniform ACLR/ACS offset step suggestion. |  |  |  |  |  |  |  |  |  |  |

2. Final simulation results plot

ATG BS ACLR value that meets the 5% throughput loss requirement(average or worst)

## Simulation scenario 10

1. Final simulation results table

| Company | Simulation scenarios | Throughput loss, detailed explanation in sub-topic 2-3 | Relative ATG UE ACLR offset |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | -10 | -5 | 0 | 5 | 10 | 15 | 20 |
| CMCC | 10 (2GHz ATGULTNUL) | 5% in the whole network | 17.9338 | 6.65604 | 2.26787 | 0.77173 | 0.28492 | 0.12943 | 0.08015 |
|  |  | 50% in the whole network | 1.297e-05 | 4.126e-06 | 1.327e-06 | 4.419e-07 | 1.620e-07 | 7.346e-08 | 4.546e-08 |
|  |  | 5% of users within the cell with largest throughput loss for the case of TN DL victim | 100 | 26.0125 | 9.65114 | 3.41985 | 1.28144 | 0.58525 | 0.36304 |
|  |  | 50% of users within the cell with largest throughput loss for the case of TN DL victim | 6.07826 | 2.01618 | 0.65744 | 0.21988 | 0.08071 | 0.03663 | 0.02267 |
| Note 1: offset 0 means ATG ACLR/ACS=legacy ACLR ACS. There is no uniform ACLR/ACS offset step suggestion. |  |  |  |  |  |  |  |  |  |

2. Final simulation results plot

ATG BS ACLR value that meets the 5% throughput loss requirement(average or worst)

## Simulation scenario 11

1. Final simulation results table

| Company | Simulation scenarios | Throughput loss, detailed explanation in sub-topic 2-3 | Relative ATG UE ACS offset |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | -10 | -5 | 0 | 5 | 10 | 15 | 20 | 25 | 30 |
| CMCC | 11 (2GHz TNDLATGDL) | 5% in the whole network | 52.0106 | 34.3276 | 19.7637 | 9.91390 | 4.99892 | 3.08716 | 2.43239 | 2.21971 | 2.15187 |
|  |  | 50% in the whole network | 16.9153 | 7.39553 | 1.77695 | 0.56308 | 0.22416 | 0.11497 | 0.08580 | 0.07780 | 0.07526 |
| Note 1: offset 0 means ATG ACLR/ACS=legacy ACLR ACS. There is no uniform ACLR/ACS offset step suggestion. |  |  |  |  |  |  |  |  |  |  |  |

2. Final simulation results plot

ATG BS ACLR value that meets the 5% throughput loss requirement(average or worst)

## Simulation scenario 12

1. Final simulation results table

| Company | Simulation scenarios | Throughput loss, detailed explanation in sub-topic 2-3 | Relative ATG BS ACS offset |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | -40 | -35 | -30 | -25 | -20 | -15 | -10 | -5 | 0 |
| CMCC | 4 (4GHz TNULATGUL) | 5% in the whole network | 34.0252 | 16.2571 | 6.43914 | 2.38294 | 0.96103 | 0.49530 | 0.34633 | 0.29905 | 0.28408 |
|  |  | 50% in the whole network | 0.09508 | 0.03036 | 0.00986 | 0.00338 | 0.00133 | 0.00068 | 0.00047 | 0.00041 | 0.00039 |
| Note 1: offset 0 means ATG ACLR/ACS=legacy ACLR ACS. There is no uniform ACLR/ACS offset step suggestion. |  |  |  |  |  |  |  |  |  |  |  |

2. Final simulation results plot

ATG BS ACLR value that meets the 5% throughput loss requirement(average or worst)

.

# 3. Conclusions

In this contribution, ATG simulation results are discussed with following observations and proposals:

Proposal 1: legacy TN BS ACLR and ACS value could be reused for ATG BS.

Proposal 2: legacy UE ACLR and ACS value could be reused for ATG UE.

# 4. Reference

[1] R4-2306607, WF on ATG co-existence evaluation, CMCC