3GPP TSG-RAN4 Meeting #107												R4-2308196

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Agenda item:	8.20.2.2.3

Source: 	CMCC

Title: 	Study on feasibility at FR1 UE side

Document for:	Discussion

# 1. Introduction

WF on SBFD feasibility study and RF impact has been approved in last meeting [1], besides, FR1 and FR2 TP s are also endorsed as in [2][3]. In this contribution, we focus on the sub-band filtering issue for FR1 SBFD-aware UE only.

# 2. Discussion

Agreements about sub-band in-channel selectivity value is listed as below, until now there is no agreement about the feasibility and necessity of sub-band filtering.

For FR1: 33dB for simulation usage purpose only based on the assumption of same bandwidth between interference and wanted signals

Following show UE-UE CL values from co-existence calibration data or derived from calculation with 1m UE-UE distant assumption.

Table 1. UE-UE CL value based on simulation and link calibration

| UE-UE CL | From calibration date |  |  | calculated with 1m separation assumption |
| --- | --- | --- | --- | --- |
|  | 5% of CDF | Average Minimum CL among all companies’ date | Minimum CL among all companies’ data |  |
| FR1 | 116.4dB | 63dB | 45dB | 44dB |

If we assume 23dBm max output power, following table show the received power at Rx LNA input port if assume no sub-band RF filtering. this table is used to show whether in close proximity case, UE Tx will block UE Rx or not.

Table 2. received power at Rx LNA input port if assume no sub-band RF filtering

| Received power from Tx sub-band/dB | From calibration date |  |  | calculated with 1m separation assumption |
| --- | --- | --- | --- | --- |
|  | 5% of CDF | Average Minimum CL among all companies’ date | Minimum CL among all companies’ data |  |
| FR1 | -93.4 | -40 | -22 | -21 |

It seems when Tx UE and Rx UE is 1m distant and aggressor transmit with max 23dB output power, input power at UE Rx side is even larger than specified max input power, i.e. -21dBm is larger than -25dBm.

Observation 1: for FR1, in extreme case, Rx interference at LNA input port from Tx sub-band is relatively large if without sub-band RF filtering assumption, e.g. maybe larger than max input power, when aggressor and victim UE are in very close proximity(1m) and aggressor transmit with max output power

Following figure 1 and table 3 show the IBE value VS delta RB for different LCRB configuration. when analyzing interference within Rx sub-band from Tx sub-band leakage, we only use IBE value since this factor dominates the sub-band co-channel selectivity according to the TP [2].

Fig 1: for FR1, the relationship between IBE and delta RB for QPSK (noted, we only list delta RB up to 10PRB for simplification because the curve is flat for even larger delta RB)

Table 3. IBE value with different delta RB and LCRB are listed as below for following analysis.

| IBE value/dB | LCRB | Delta RB |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1 | 3 | 5 |
| FR1 | 1 | -18 | -28 | -38 |
|  | 51 | -18 | -28 | -32 |
|  | 273 | -18 | -25 | -25 |

Following table show the received interference within Rx sub-band from Tx sub-band leakage.

Table 4. interference in Rx sub-band from Tx sub-band leakage

| interference in Rx sub-band from Tx sub-band leakage/dB | IBE value/dB | From calibration date |  |  | calculated with 1m separation assumption |
| --- | --- | --- | --- | --- | --- |
|  |  | 5% of CDF | Average Minimum CL among all companies’ date | Minimum CL among all companies’ data |  |
| FR1 | 18 with delta RB=1 | -111.4 | -58 | -40 | -39 |
|  | 28 with delta RB=3 | -121.4 | -68 | -50 | -49 |

For FR1 worst case, interference in Rx sub-band from Tx sub-band leakage is in the range from -121.4dBm to -39dBm which is severe interference. although actual interference could be much less based on RAN1 enhanced UE-UE CLI measurement mechanism to cancellate interference, additional sub-band filter, e.g. RF filter at Tx side still help to reduce interference.

Observation 2: for FR1, in extreme case, Rx interference in Rx sub-band from Tx sub-band leakage is severe and maybe in the range -121.4~ -39dBm. Sub-band filter could help to reduce interference.

As discussed in last meeting, due to the complexity and power consumption constraint, a UE may not be able to conduct a sub-band filtering with a size of any PRB number. UE sub-band filtering capability is also OK.

Proposal 1: if it’s hard to conduct sub-band filtering for every SBFD-aware UE, sub-band filtering capability is candidate solution.

# 3. Conclusions

In this contribution, UE feasibility is discussed with following observations and proposals:

Observation 1: for FR1, in extreme case, Rx interference at LNA input port from Tx sub-band is relatively large if without sub-band RF filtering assumption, e.g. maybe larger than max input power, when aggressor and victim UE are in very close proximity(1m) and aggressor transmit with max output power.

Table 2. received power at Rx LNA input port if assume no sub-band RF filtering

| Received power from Tx sub-band/dB | From calibration date |  |  | calculated with 1m separation assumption |
| --- | --- | --- | --- | --- |
|  | 5% of CDF | Average Minimum CL among all companies’ date | Minimum CL among all companies’ data |  |
| FR1 | -93.4 | -40 | -22 | -21 |

Observation 2: for FR1, in extreme case, Rx interference in Rx sub-band from Tx sub-band leakage is severe and maybe in the range -121.4~ -39dBm. Sub-band filter could help to reduce interference.

Proposal 1: if it’s hard to conduct sub-band filtering for every FR1 SBFD-aware UE, sub-band filtering capability is candidate solution.

# 4. Reference

[1] R4-2305969, WF for SBFD feasibility study and requirements impact from UE aspect, Qualcomm Inc.

[2] R4-2305970, TP to TR 38.858 on Feasibility of FR1 UE aspects, MediaTek Inc.

[3] R4-2305971, TP to TR 38.858 feasibility of and impact on FR2 UE RF requirements, Qualcomm Inc.