3GPP TSG-RAN4 Meeting #107												R4-2308201

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Agenda item:	8.5.1.3.1

Source: 	CMCC

Title: 	Discussion on lower MSD capability

Document for:	Discussion

# 1. Introduction

In last meeting, a WF on study for lower MSD signaling design has been approved [1]. In this contribution, we focus on remaining lower MSD capability discussion.

# 2. Discussion

## 2.1 how to reduce capability overhead

In last meeting, one operator proposes that if MSD capability overhead is heavy, this feature may not be used in commercial network. so it’s better to find out how to reduce capability overhead.

Proposal 1: to make MSD capability used in commercial network, it’s better to study how to reduce capability overhead.

One solution is to allow gNB query UE capability and UE only report certain capability filtered by gNB’s query information. Such query information may include following information:

band combinations, Power class, Tx power, aggressor and victim CBW, victim operation band.

Proposal 2: to reduce MSD capability overhead, one solution is to allow gNB query UE capability and UE only report certain capability filtered by gNB’s query information. Query information could include following information, e.g. band combinations, power class, Tx power, aggressor and victim CBW, victim operation band.

## 2.2 power related report for MSD

MSD performance is related to Tx power, for example, 3dB max output power difference may lead to max 10dB MSD difference. Larger Tx power at UE side, better throughput for UL but may lead to worse MSD for DL. in some extreme case, when UE transmit with max power to maximize UL performance, certain DL victim RB can’t be allocated due to much severe MSD. Therefore, gNB needs the relationship between UL Tx power and DL MSD performance to trade off UL and DL performance.

Observation 1: gNB needs the relationship between UL Tx power and DL MSD performance to trade off UL and DL performance.

Following list several solutions for relationship between MSD capability and Tx power:

- Solution 1: categorize MSD with several typical Tx power level. But this method requires UE to have the information for different Tx power levels, heavy workload at UE side.

- Solution 2: filter MSD for different power with filtering parameter when query UE capability. Same shortcoming as solution1, heavy workload at UE side to find out the relationship between MSD and Tx power

- Solution 3: additional information to show under which Tx power, all the MSD would be negligible, e.g. less than 3dB or 5dB. gNB could use this information for final UE scheduling algorithm or deciding final UE Tx power

  - This information help gNB to know which UE could be allocated to MSD-victim RB since the MSD is negligible for this UE when it is at certain location with less Tx power.

  - in some cases when DL throughput is bottleneck due to MSD-victim RBs are unavailable, gNB could reduce UE Tx power to make the MSD acceptable for DL. This MSD information could help gNB fully utilize all RBs even for the RB with severe MSD to avoid the case that certain RB can’t be allocated due to severe MSD at max Tx power.

According to above analysis, solution 3 is more preferred.

Proposal 3: it’s suggested to report additional information to show under which Tx power, all the MSD would be negligible, e.g. less than 3dB or 5dB. gNB could use this information for final UE scheduling algorithm or deciding final UE Tx power.

In last meeting, single-bit low-MSD indicator for all MSD types is supported by most companies as listed below.

single-bit low-MSD indicator for a UE that all MSD types for this BC have been improved to above a threshold (HW, vivo, Samsung, Meta, [OPPO]) – Option a: A joint solution of one bit low MSD indication per BC with the per victim band per MSD type per band combination signaling, one bit low MSD indication can be used if all MSD types for this BC have been improved to above a threshold. (CHTTL) – Option b: Single bit indication of small MSD capability (MSD <= [3]dB) can be reported to apply the improved MSD level by the high order IMD/harmonic problems (Meta)

Solution in proposal 3 is the extension of legacy single-bit indicator with Tx power information and only focus on the case when MSD is negligible. To be honest, compared with accurate MSD enhancement, it’s much directly and simply for gNB to know under which case MSD is negligible.

Legacy 1 bit indicator has following shortcomings compared with solution in proposal 3.

- only consider max Tx power, but usually UE may not transmit with max power, gNB need to know the Tx power information to decide whether sacrifice UL Tx power to fully utilize MSD-victim RB.

- only report the relative MSD enhancement which may require gNB to memorize all minimum MSD RF requirement before using MSD capability informaiton.

- This information could only help gNB know that UE has certain great enhancement but to be honest, now it’s not sure how gNB performs with this detailed MSD enhancement performance and whether there is performance gain with MSD enhancement capability. But solution in proposal 3 is much directly and tell gNB under which Tx power, MSD is negligible, then gNB could decide UE Tx power to ignore DL MSD if necessary, the performance gain is directly.

Observation 2: solution in proposal 3 is the extension of legacy single-bit indicator with Tx power information and only focus on the case when MSD is negligible. it’s much directly and simply for gNB to know under which case MSD is negligible.

## 2.3 aggressor and victim bandwidth for MSD capability

MSD performance is related to CBW combination of aggressor and victim, different CBW combination will lead to different MSD. There are several principles for the aggressor and victim bandwidth when define MSD minimum requirements. e.g. minimum aggressor and minimum victim CBW for harmonic types MSD. but, usually operators’ spectrum is different from above aggressor/victim bandwidth configuration, e.g. aggressor CBW is larger than minimum CBW for harmonic types. if we only report MSD with the same CBW configuration as minimum RF requirements, this MSD information can’t be fully used by gNB because such MSD value is not applicable for configured CBW and gNB have no information of how to translate MSD value among different CBW. In other words, gNB could only use this MSD capability for qualitative analysis rather than quantitative analysis.

Observation 3: if only report MSD with the same aggressor/victim CBW configuration as minimum MSD RF requirements, this MSD information can’t be fully used by gNB because the value is not applicable for configured CBW and gNB have no information of how to translate MSD value among different CBW.

To make gNB utilize this MSD enhancement information, it’s better to also report configured aggressor and victim bandwidth combination to gNB together with corresponding MSD value. Here the aggressor and victim bandwidth are the carrierBandwidth information configured for Pcell and Scell.

It’s better not to require UE to know MSD for all aggressor and victim CBW combinations, which is heavy workload for UE vendor. CBW configuration is fixed in commercial network, so only the CBW combinations used by commercial networks could be reported by UE. i.e. UE could only report capability information for several typical CBW combinations.

Proposal 4: it’s suggested to report MSD values for the aggressor and victim CBW that are configured by network rather than the CBW combinations only for testing points. UE could only report capability information for several typical CBW combinations that are used by commercial network rather than all CBW combinations.

Detailed CBW combinations could be requested by gNB and UE reply the MSD under corresponding aggressor and victim CBW combinations.

Proposal 5: to reduce capability overhead, gNB could query CBW configurations for aggressor and victim carrier and UE reply corresponding MSD.

## 2.4 how to trigger UE capability

UE will report enhanced absolute MSD to gNB to avoid extra workload of translating relative MSD value into final absolute MSD. Default behavior is assuming UE support minimum MSD requirements as in 38.101 when there is no enhanced MSD capability. gNB have to memorize current MSD requirements for all the band combinations. When gNB perform scheduling algorithm or configure/active Scell configuration/activate, gNB take such MSD value into consideration. besides, other factors, CSI, DL data buffer, gNB/UE capability will also be taken into consideration. Usually, several dB (e.g. 1-2dB) MSD enhancement will not change final scheduling algorithm because the granularity or accuracy of all inputs factors for scheduling algorithm are comparable or larger than several dB (e.g. 1-2dB). so usually, there should be certain threshold of MSD enhancement for triggering UE capability.

Observation 4: several dB (e.g. 1-2dB) MSD enhancement will not change final scheduling algorithm because the granularity or accuracy of all inputs factors for scheduling algorithm are comparable or larger than several dB (e.g. 1-2dB).

# 3. Conclusions

In this contribution, MSD capability related issues are discussed with following observations and proposals:

Proposal 1: to make MSD capability used in commercial network, it’s better to study how to reduce capability overhead.

Proposal 2: to reduce MSD capability overhead, one solution is to allow gNB query UE capability and UE only report certain capability filtered by gNB’s query information. Query information could include following information, e.g. band combinations, power class, Tx power, aggressor and victim CBW, victim operation band.

Observation 1: gNB needs the relationship between UL Tx power and DL MSD performance to trade off UL and DL performance.

Proposal 3: it’s suggested to report additional information to show under which Tx power, all the MSD would be negligible, e.g. less than 3dB or 5dB. gNB could use this information for final UE scheduling algorithm or deciding final UE Tx power.

Observation 2: solution in proposal 3 is the extension of legacy single-bit indicator with Tx power information and only focus on the case when MSD is negligible. it’s much directly and simply for gNB to know under which case MSD is negligible.

Observation 3: if only report MSD with the same aggressor/victim CBW configuration as minimum MSD RF requirements, this MSD information can’t be fully used by gNB because the value is not applicable for configured CBW and gNB have no information of how to translate MSD value among different CBW.

Proposal 4: it’s suggested to report MSD values for the aggressor and victim CBW that are configured by network rather than the CBW combinations only for testing points. UE could only report capability information for several typical CBW combinations that are used by commercial network rather than all CBW combinations.

Detailed CBW combinations could be requested by gNB and UE reply the MSD under corresponding aggressor and victim CBW combinations.

Proposal 5: to reduce capability overhead, gNB could query CBW configurations for aggressor and victim carrier and UE reply corresponding MSD.

Observation 4: several dB (e.g. 1-2dB) MSD enhancement will not change final scheduling algorithm because the granularity or accuracy of all inputs factors for scheduling algorithm are comparable or larger than several dB (e.g. 1-2dB).

# 4. Reference

[1] R4-2306590, WF on lower MSD, Huawei, HiSilicon

[2] R4-2220513, TP for TR 38.881: Feasibility study from companies and conclusion for lower MSD improvement, Huawei, HiSilicon