3GPP TSG-RAN4 Meeting #107												R4-2308203

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Agenda item:	8.29.2.1

Source: 	CMCC

Title: 	Discussion on NCR forwarding requirements

Document for:	Discussion

# 1. Introduction

In last RAN4 meeting, a WF is approved [1]. In this meeting, we focus on the discussion of NCR forwarding link remaining issues.

# 2. Discussion

## 2.1 fwd link UL output power for 1-H and 1-O

Last meeting agreement is listed as below:

For 1-H: - for DL part: To follow the option 2; - For UL part: FFS For 1-O: - for DL part: To follow the option 2 with 9dB scaling factor; - For UL part: FFS

One issue proposed in last meeting is that for large array, total power could exceed any UE power category. Usually the active NTXU,active is small for NCR, e.g. 1 or 2 or 4, considering NCR would be smaller compared with normal gNB. Final scaling factor would be at most 6dB and final output power equals to 24+6=30dB which is very near to PC1.5 29dBm. but if active NTXU,active is larger than 4, e.g. 8, then scaling factor is 9dB and final output power could be 24+9=33dBm even larger than PC1 31dBm power limit. For this large antenna array, it is much like WA class rather than LA and RF requirements should follow WA rather than reusing LA. Therefore, it’s better to limit the scaling factor for LA case, our suggestion of scaling factor upper limit is 6dB, i.e. 4 multiple Ncells which lead to max 30dB output power, a little larger than PC1.5. But for WA, the same scaling factor value as DL is OK, i.e. 8*Ncells.

Proposal 1: scaling factor for 1-H and 1-O fwding link UL output power is suggested as below.

- For LA, max scaling factor is 6dB, implicating max 30dBm output power, i.e. NTXU,counted = min(NTXU,active , 4*Ncells)

- For WA, max scaling factor is 9dB, i.e. NTXU,counted = min(NTXU,active , 8*Ncells)

## 2.2 Transmit ON/OFF power and transition period

The agreement for NCR ON/OFF requirement is listed as below:

Agreement: - For NCR-MT and NCR-Fwd switch ON-OFF together, proposed to have further study for it; - For NCR-MT and NCR-Fwd switch ON-OFF individually, propose to Reuse the Rel-17 repeater type 1-C requirements;

According to RAN1 agreements, the ON-OFF indication is implicit for fwd-link. If there is access link beam then it implicitly means ON state whereas when there is no access link beam, it implicitly means OFF state. Another case for fwd-link OFF is that when MT part is OFF then fwd-link is OFF accordingly.

Observation 1: according to RAN1 agreements, when NCR-MT is OFF, NCR-fwd will be OFF simultaneously.

So it is possible that fwd-link and MT-link turn OFF simultaneously, also possible that fwd-link is OFF whereas MT link turn ON and also possible that NCR-MT is ON while NCR-fwd turn ON.

We need to analyze following three cases:

- Case 1: NCR-MT and NCR-Fwd turn OFF simultaneously

- Case 2: NCR-MT turn ON whereas NCR-fwd is OFF

- Case 3: NCR-fwd turn ON whereas NCR-MT is ON

Proposal 2: when considering ON-OFF switching, we need to consider following three cases:

- Case 1: NCR-MT and NCR-Fwd turn OFF simultaneously

- Case 2: NCR-MT turn ON whereas NCR-fwd is OFF

- Case 3: NCR-fwd turn ON whereas NCR-MT is ON

For RF repeater, the downlink OFF power is the same as BS spec and uplink OFF power is the same as UE spec. for NCR-fwd, it’s suggested to reuse the same gNB value and UE value for downlink and uplink respectively when NCR-fwd switching individually. For NCR-MT, it’s suggested to reuse the same value as UE when it switching individually.

for case 1, the OFF power for uplink is the sum of MT part and fwd-uplink, i.e. -50dBm+3=-47dBm. the OFF power for downlink is the sum of fwd-downlink and MT part DL, i.e. i.e. -85+(-50)=-50dBm.

Proposal 3: when NCR-MT and NCR-Fwd turn OFF simultaneously, the OFF power of uplink is the sum of MT part and fwd-uplink, i.e. -50dBm+3=-47dBm. the OFF power for downlink is the sum of fwd-downlink and MT part DL, i.e. i.e. -85+(-50)=-50dBm.

As for transition period,

- for case 1, individual MT part is the same as UE spec, individual fwd-link is the same as RF repeater spec; the issue is whether there is delay between MT OFF and fwd-link OFF although they should be turned OFF at the same time according to RAN1 agreements. When fwd-link and MT share the same RF component, it seems no delay. When fwd-link and MT doesn’t share any RF component, there may be certain delay to transfer the OFF information to fwd-link. For example, when NCR-MT and fwd-link is 300m distant, the delay is about 1us which is almost negligible compared with 10us. therefore, we could assume MT part and fwd part could turn OFF at the same time.

- for case 2, the transition period is the same as UE spec.

- for case 3, the transition period is the same as RF repeater.

Proposal 4: as for transition period, we have following suggestion:

- for case 1, MT part is the same as UE spec, fwd-link is the same as RF repeater spec individually. There is no delay between MT part OFF state and fwd-link OFF state.

- for case 2, the transition period is the same as UE spec.

- for case 3, the transition period is the same as RF repeater.

## 2.3 NCR forwarding EVM

Following are the agreements.

For 1-H: To reuse the Rel-17 repeater type 1-C requirement for each TAB connector of NCR-Fwd type 1-H; For 1-O: FFS

whether for 1-C RF repeater DL or UL, the repeater EVM requirement is applicable for an input power range from what is required to reach the maximum output power to the minimum power level as below for DL and UL respectively.

Table 6.6.1.1-1: Minimum input power for repeater EVM

| Repeater DL class | Minimum input power spectral density (dBm/MHz) |  |
| --- | --- | --- |
|  | QPSK, 16 QAM, 64QAM | 256QAM1 |
| WA | -82 | -75 |
| MR | -77 | -70 |
| LA | -74 | -67 |
| Note 1: support of 256QAM is based on the declaration |  |  |

Table 6.6.2.1-1: Minimum input power for repeater EVM

| Repeater UL class | Minimum input power spectral density (dBm/MHz) |  |
| --- | --- | --- |
|  | QPSK, 16 QAM, 64QAM | 256QAM1 |
| WA | -82 | -75 |
| LA | -74 | -67 |
| Note 1: support of 256QAM is based on the declaration |  |  |

Above minimum input power is derived based on SNR which equals to 20log(EVM) as below. This minimum input power is used to implicitly evaluate NF since some TE vendors say the NF testing is not feasible for FR2.

Minimum input power = 20log10(EVM)+thermal noise + NF

For 1-O, above equation still applies but we need to take Rx antenna gain into consideration, so minimum input power at RIB point equals to 1-C requirement minus Rx antenna gain where Rx antenna gain needs further check.

Proposal 5: for 1-O fwd-link, minimum input power for EVM equals to 1-C requirements minus Rx antenna gain.

As for the EVM requirements, it’s suggested to reuse the same as 1-C as below.

Proposal 6: the EVM requirements for fwd-link UL and DL are as below:

| Parameter | Unit | Average Repeater EVM Level |
| --- | --- | --- |
| QPSK, 16 QAM, 64QAM | % | 8 |
| 256 QAM | % | 3.5 1 |
| Note 1: support of 256QAM is based on the declaration. |  |  |

# 3. Conclusions

In this contribution, NCR requirements are discussed with following observations and proposals:

Proposal 1: scaling factor for 1-H and 1-O fwding link UL output power is suggested as below.

- For LA, max scaling factor is 6dB, implicating max 30dBm output power, i.e. NTXU,counted = min(NTXU,active , 4*Ncells)

- For WA, max scaling factor is 9dB, i.e. NTXU,counted = min(NTXU,active , 8*Ncells)

Observation 1: according to RAN1 agreements, when NCR-MT is OFF, NCR-fwd will be OFF simultaneously.

Proposal 2: when considering ON-OFF switching, we need to consider following three cases:

- Case 1: NCR-MT and NCR-Fwd turn OFF simultaneously

- Case 2: NCR-MT turn ON whereas NCR-fwd is OFF

- Case 3: NCR-fwd turn ON whereas NCR-MT is ON

Proposal 3: when NCR-MT and NCR-Fwd turn OFF simultaneously, the OFF power of uplink is the sum of MT part and fwd-uplink, i.e. -50dBm+3=-47dBm. the OFF power for downlink is the sum of fwd-downlink and MT part DL, i.e. i.e. -85+(-50)=-50dBm.

Proposal 4: as for transition period, we have following suggestion:

- for case 1, MT part is the same as UE spec, fwd-link is the same as RF repeater spec individually. There is no delay between MT part OFF state and fwd-link OFF state.

- for case 2, the transition period is the same as UE spec.

- for case 3, the transition period is the same as RF repeater.

Proposal 5: for 1-O fwd-link, minimum input power for EVM equals to 1-C requirements minus Rx antenna gain.

Proposal 6: the EVM requirements for fwd-link UL and DL are as below:

| Parameter | Unit | Average Repeater EVM Level |
| --- | --- | --- |
| QPSK, 16 QAM, 64QAM | % | 8 |
| 256 QAM | % | 3.5 1 |
| Note 1: support of 256QAM is based on the declaration. |  |  |

# 4. Reference

[1] R4-2305931, WF on system parameters and RF requirements, ZTE

[2] RP-223505, Revised WID on NR network-controlled repeaters, ZTE, Sanechips.

[3] R1-2304216, Session Notes of AI 9.17.4, AT&T