3GPP TSG-RAN WG4 Meeting # 107												R4-2308232

Incheon, KR, May 22 – May 26, 2023

Source: 	vivo

Title: 	Discussion on the impact of power imbalance for sDCI UE

Agenda Item:	8.8.2.1

Document for:	Approval

- Introduction

In the last meeting, the functionality verification based requirement concept is confirmed and several simulation results are submitted. In this contribution, we further discuss the impact of power imbalance due to the fixed DL power and whether the current evaluation is still reasonable for sDCI UE.

- Discussion

In [1], the requirement concept is confirmed and using legacy spherical power level for both AoAs as fixed DL power is the starting point. Obviously, this scheme is the easiest way for the test setup, but we still want to discuss whether this scheme is reasonable for both sDCI and mDCI UE.

In the current evaluation, we use -1dB as the criterion to judge whether the test points are pass or fail and only when both SINR from 2AoAs are larger than -1dB, we can conclude this AoA pair is pass. The different power scheme will cause the SINR of each test point changes which will lead to different N% result. However, as long as we can guarantee that the power scheme and final requirement are matched, there is no essential difference between the different power schemes, just the value of the requirement will change.

Observation 1: From the requirement design perspective, there is no essential difference between different fixed DL power schemes. As long as the power scheme and final requirement are matched, the only difference is the value of the final requirement.

One more thing we want to discuss here is the power imbalance. When both AoAs use the same fixed DL power level, due to the antenna gain and inter-beam interference being different for the signal from 2 AoA, the SINR at baseband for 2 layers are different, which means using the same fixed DL power will bring power imbalance artificially.

Observation 2: Power imbalance exists under the fixed DL power due to the antenna gain/interference difference.

For multi-DCI UE, each layer can be decoded separately and we can get the throughput of each layer, so the power imbalance doesn’t matter and the -1 dB criterion above to judge each test point from a pair of AoA is still reasonable. However, things become different for sDCI UE because the 2 layers of sDCI UE share one transmission block and we can only get a total throughput of 2-layer MIMO, and there is no doubt that the power imbalance will affect this total throughput, and simply using -1dB as the criterion for each test point does not make sense anymore. Figure 1 shows that how power imbalance will affect the throughput of sDCI UE.

Figure 1 impact of power imbalance for sDCI UE

In the simulation, the SINR1 and SINR2 represent the SINR value from 2AoA. Obviously, due to the power imbalance, even though one of SINRs is less than -1 dB, the total throughput still can achieve 95% RMC and pass the test.

Observation 3: The -1 dB criterion cannot accurately reflect the actual pass/fail situation of sDCI UE due to the power imbalance.

Does raising or lowering the power of one of the AoA can help with this problem? The answer is NO. Simply changing the power level cannot remove the power imbalance, and the only way to solve this issue is for each test point, the DL power should be changed, but this scheme is too complicated from both the verification and evaluation perspectives.

Observation 4: Simply raising or lowering the power level cannot remove the power imbalance, and the only way is for each test point, the DL power should be changed, but this scheme is too complicated from the verification and evaluation perspective.

As a compromise, we can only focus on that whether the sDCI UE can pass the same requirement as mDCI under power imbalance. In Figure 1, the simulation shows that when both SINRs is larger than -1 dB, the total throughput still can be larger than 95% RMC, which means the -1 dB in simulation still can ensure the AoA pair can pass the test. The only omission here is that case one of the SINR<-1 but passing the test is not reflected in the requirement. As we mention above, even though we realize this omission but it is hard to be verified or evaluated, so we think it is enough to confirm the same requirement can be applied to both sDCI and mDCI UE under the current evaluation method.

Observation 5: Even though the power imbalance exists, the current evaluation metric still can ensure sDCI and mDCI UE can pass the test under the same requirement.

Based on the analysis and observation above, we propose:

Proposal 1: Confirm that the legacy spherical power level is used as the fixed DL power, and no need to discuss other power schemes.

Proposal 2: Confirm that sDCI and mDCI can share exactly the same requirement under the current fixed DL power scheme, and no need to define specific requirements for sDCI UE.

- Conclusion

In this contribution, we provide our analysis on the impact of power imbalance and whether same requirement can be applied to both sDCI and mDCI UE. Our proposals are listed below.

Observation 1: From the requirement design perspective, there is no essential difference between different fixed DL power schemes. As long as the power scheme and final requirement are matched, the only difference is the value of the final requirement.

Observation 2: Power imbalance exists under the fixed DL power due to the antenna gain/interference difference.

Observation 3: The -1 dB criterion cannot accurately reflect the actual pass/fail situation of sDCI UE due to the power imbalance.

Observation 4: Simply raising or lowering the power level cannot remove the power imbalance, and the only way is for each test point, the DL power should be changed, but this scheme is too complicated from the verification and evaluation perspective.

Observation 5: Even though the power imbalance exists, the current evaluation metric still can ensure sDCI and mDCI UE can pass the test under the same requirement.

Proposal 1: Confirm that the legacy spherical power level is used as the fixed DL power, and no need to discuss other power schemes.

Proposal 2: Confirm that sDCI and mDCI can share exactly the same requirement under the current fixed DL power scheme, and no need to define specific requirements for sDCI UE.

- References

- R4-2306604	WF on FR2-1 UERF requirements for FR2 multi-Rx, vivo, Apple, RAN4#106e-bis