3GPP TSG-RAN WG4 Meeting # 107												R4-2308235

Incheon, KR, May 22 – May 26, 2023

Source: 	vivo

Title: 	Discussion on FR2 OTA test method

Agenda Item:	8.3.2

Document for:	Approval

- Introduction

In the last meeting, the general flow for multi-Rx verification is agreed but the details need to wait for the conclusion in RF session. In this contribution, we discuss the impact of different test grid.

- Discussion

In [1], the constant step size grid is agreed as the only feasible test grid and in RF session [2], the step size for requirement evaluation is suggested to be less than 5°

3. Run EM simulation to obtain per-beam antenna gain patterns

- Constant step size is suggested <= 5°

- Performance difference between V/H element can be considered

- Normalize antenna gain to align with the gain drop between peak EIS and spherical coverage in current spec (option 1 in 2.6)

  - Other calibration method that described in 2.6 also can be used.

In [2], the functionality based requirement concept is confirmed and under such verification method, the UE performance is sensitive to the step size since the beam selection may be impacted. We provide a preliminary simulation result to show the difference:

Figure 1 UE performance under different step size

The results show that UE performance degraded under a finer test grid and the reason here is that more AoA pair will be tested under a finer grid which requires UE to have a higher capability, as shown below.

Figure 2 illustration on the difference between step size

Considering finer test grids also suffer from long test time, we prefer only to use sparse test grids to verify the performance of multi-Rx UE.

Observation: Finer test grid requires higher UE capability and also enlarges the test time.

Proposal: The step size of the test grid for multi-Rx verification should be 15°.

- Conclusion

In this contribution, we analyze the impact of test grid step size, and our proposal is listed below:

Observation: Finer test grid requires higher UE capability and also enlarges the test time.

Proposal: The step size of the test grid for multi-Rx verification should be 15°.

- References

- R4-2302923     WF for FR2 OTA test enhancement SI, Qualcomm, RAN4#106

- R4-2306604	WF on FR2-1 UERF requirements for FR2 multi-Rx, vivo, Apple, RAN4#106e-bis.