3GPP TSG-RAN WG4 Meeting # 107	R4-2308238

# Incheon, KR, May 22nd – May 26th, 2023

Source: 	vivo

Title: 	Further discussion on 4Tx UE RF requirements

Agenda Item:	8.5.1.1

Document for:	Discussion

Introduction

In RAN4#106bis-e, the key progress for 4Tx UE RF requirements is documented in the WF [1] and topic summary [2].

For the relation between configured transmitted power and TPMI configuration that was raised in [3], since this is a release 15/16 issue, a separate discussion paper and LS were submitted in maintenance agenda as in [4][5].

In this paper, more discussions were provided.

Discussion

Non-full power related configurations

In last meeting WF, the following WF issue is discussed:

Whether non-full power (power scaling / fall back) cases are needed to verified?

It turns out to be that there are some different understandings of what kind of power scaling should be considered. Our understanding is that there are the following cases, and some analysis were provided respectively:

- Power scaling introduced in 38.213 as in Rel-15:

This case actually does not related to 4Tx feature, and has never been considered for verification since Rel-15. Though some more discussions are on going as in [4][5] to discuss the RAN1 and RAN4 impact, specific verification seems not needed.

Observation 1: Power scaling introduced in 38.213 as in Rel-15 has never been considered for verification.

- Power scaling introduced in 38.213 since Rel-16 for Full tx power feature for some Modes and TPMIs

This is actually is a continuous and similar case to previous Rel-15 case. From Rel-16, some TPMI configurations for certain Full power modes would still have this power scaling.

Also similar to previous Rel-15 case This case actually does not related to 4Tx feature, and not considered since Rel-16. The discussion ongoing as in [4][5] to discuss the RAN1 and RAN4 impact seems also would not impact the current status. More discussion can be considered if further progress can be made on this issue.

Observation 2: For the power scaling (factor s) introduced in 38.213 since Rel-16 for Full Tx power feature for some Modes and TPMIs, no verification is done from Rel-16, since only the full power is design target.

- The fallback introduced by ΔPPowerClass in TS 38.101-1/2

The ΔPPowerClass are generally introduced by duty cycle related scheme and SRS antenna configuration.

Currently, the duty cycle was not considered in typical test configurations verifications, and considering the existing status of UE support for duty cycle method, and the existing test configurations on slot / duty cycle configuration seems not need to be revised.

Similar case is for SRS antenna switching, in that no verification is needed.

Observation 3: The fallback introduced by ΔPPowerClass has never been considered for verification.

- Others (Implementation specific)

It was argued that for some UE implementations of 4Tx, there are restrictions for actual achievable power in many cases, and the power class declared cannot be always achieved.

Probability this is the main contributor that can be regarded specific 4Tx. However, in our understanding, this case is actually pretty much similar to the bullet b. In another word, the main discussion of Full power mode in RAN1 is actually try to design specific schemes for different UE implementations. For different UE implementations, as long as Full Tx power feature is supported, the s =1 can almost always be achieved in certain cases, and s < 1 can hardly be avoided for some other configurations.

In this sense, this implementation specific issue is actually the same to previous bullet b, similar to Power scaling introduced in 38.213 since Rel-16 for Full Tx power feature for some Modes and TPMIs. As baseline, only full power configurations are suggested to be verified, since this is the actual design target, not implementation compromise.

Observation 4: The implementation specific restriction on achievable power class in certain condition is similar to the Full Tx power cases from release-16, and the considerations are similar.

Based on the previous observations, the following proposal is provided:

Proposal 1: No need to verify the non-full power cases in 4Tx UE RF requirements.

However, it is admitted that the current discussion of configured transmitted power and TPMI configuration as in [4][5] is still on going and some impact may still here. So this proposal can be somewhat revisited after more feedback from RAN1 can be obtained.

Verification of other Ranks

Apart from rank = 4, there are proposals to also other specify other ranks to be verified. The agreements from last meeting is as following:

Issue 1-1-2: Whether Full power rank 2 configurations are needed for verification?

Agreement:

- For Stage-1, Full power rank 2 configurations are not considered for verification.

- For Stage-2, more discussions are needed for the following options (Currently no Tx architecture precluded). E.g.

  - Not needed

  - Mode 1 only

  - Mode 1 + Mode 2

Views are still diverse, it seems that there is no simple yes or no for this. As commented earlier, we prefer no more rank 2 specification for simplicity consideration. If rank = 2 is deemed as a typical configuration that would like more verification, the mode should also be restricted.

Proposal 2: Not to introduce rank = 2 full power tests. If more configuration(s) be considered for ULFPTx mode(s) for test configuration and requirements, Mode 1 with TPMI = 6 for 2-layer seems to be a candidate.

UL-MIMO Coherence

After some study, we found that UL-MIMO coherence requirements may need to be further updated to reflect the 4Tx requirements. One proposal is as below:

6.4D.4	Requirements for coherent UL MIMO

For coherent UL MIMO, Table 6.4D.4-1 lists the maximum allowable difference between the measured relative power and phase errors between any different antenna connectors in any slot within the specified time window from the last transmitted SRS on the same antenna connectors, for the purpose of uplink transmission (codebook or non-codebook usage) and those measured at that last SRS. The requirements in Table 6.4D.4-1 apply when the UL transmission power at each antenna connector is larger than 0 dBm for SRS transmission and for the duration of time window.

Table 6.4D.4-1: Maximum allowable difference of relative phase and power errors in a given slot compared to those measured at last SRS transmitted

| Difference of relative phase error | Difference of relative power error | Time window |
| --- | --- | --- |
| 40 degrees | 4 dB | 20 msec |

This is a simple extension of the current requirements from two antenna case to multiple antenna connector case.

Proposal 3: Extend the UL-MIMO coherence requirements for 4Tx requirements, and reuse the current requirements for any antenna connectors combination.

Conclusion

In this paper, the following Observations and proposals for 4Tx requirements were provided:

Observation 1: Power scaling introduced in 38.213 as in Rel-15 has never been considered for verification.

Observation 2: For the power scaling (factor s) introduced in 38.213 since Rel-16 for Full tx power feature for some Modes and TPMIs, no verification is done from Rel-16, since only the full power is design target.

Observation 3: The fallback introduced by ΔPPowerClass has never been considered for verification.

Observation 4: The implementation specific restriction on achievable power class in certain condition is similar to the Full Tx power cases from release-16, and the considerations are similar.

Proposal 1: No need to verify the non-full power cases in 4Tx UE RF requirements.

Proposal 2: Not to introduce rank = 2 full power tests. If more configuration(s) be considered for ULFPTx mode(s) for test configuration and requirements, Mode 1 with TPMI = 6 for 2-layer seems to be a candidate.

Proposal 3: Extend the UL-MIMO coherence requirements for 4Tx requirements, and reuse the current requirements for any antenna connectors combination.

- References

[1] R4-2306595, WF on 4Tx UE RF requirements, vivo, RAN4#106bis-e

[2] R4-2306286, Topic summary for [106][124] FR1_enh2_part2, Moderator (vivo) , RAN4#106bis-e

[3] R4-2304161, Relation between configured transmitted power and TPMI configuration, Nokia, RAN4#106bis-e

[4] R4-2308248, Discussion on power scaling factor s and impact to RAN4, vivo , RAN4#107

[5] R4-2308249, [Draft] LS on power scaling and PHR in 38.213, vivo , RAN4#107