3GPP TSG-RAN WG4 Meeting # 107	R4-2308240

# Incheon, KR, May 22nd – May 26th, 2023

Source: 	vivo

Title: 	Further discussion of lower MSD Signalling

Agenda Item:	8.5.1.3.1

Document for:	Discussion

Introduction

In RAN4#106bis-e, major progress was made on the topic of lower MSD signalling. The main progress was documented in the WF [1]. The detailed discussion can reference to the topic summary in [2]. A comprehensive LS is also sent to RAN2 for preliminary work as in [3] which contains many current agreements on signalling. In addition, another LS on clarification of test configurations were sent to RAN5 in [4] for test point selection related topic. Furthermore, one TR for 38.881 is also approved in the post meeting Email approval process.

However, there are still some open issues on some of the key issues. In this paper, some views and proposals are provided.

Discussion

Conditions to indicate the lower MSD capability

In the WF [1], majority companies support option 1 or 2 which mandate UE has to achieve next more strict level of MSD region compared to the region of what minimum requirements, then can report low MSD. However, there is a difference in whether this condition is needed or not:

- If UE reports the lower MSD capability, the reported MSD value should be improved at least by TBD dB against a specified MSD

In [7], a very good illustration has been provided to illustrate the situation. The intention is understandable in that a certain amount of guaranteed improvement is preferred.

However, it should be noted that if this kind of requirements was introduced, it means that the evaluation and verification would not only based on the unified thresholds, but also based on the “relative” values compared to the minimum requirements of every specific case. This would not only greatly increase the complexity, but also contradicting with the previous agreement of use “absolute thresholds” for lower MSD.

Observation 1: The idea of mandating the reported MSD value should be improved at least by TBD dB , is against the previous agreement of use “absolute thresholds”, and may greatly increase the complexity of verification.

Based on this situation, we think the possible improvement related to this verification is not worth it and brings much more harm than good, thus should not be utilized.

Proposal 1: Do not consider mandating the reported MSD value should be improved at least by TBD dB.

Whether CBW of aggressor UL and victim DL should be reported for lower MSD capability

In the WF, an overwhelming majority of companies (10) choose to not report this CBW information, and a few other (2) companies propose to discuss together with conformance test. Only two companies clearly support the reporting of this information.

After the analysis of the conformance test, the most likely scheme would also circumvent the reporting of CBW information, and we also think this information is likely not needed to be reported.

Proposal 2: Do not report CBW of aggressor UL and victim DL for lower MSD capability.

MSD types and orders

Currently, there are still no conclusion of MSD type and order information and the WF is as following:

- The MSD source to be reported is selected from the set of {UL harmonic, Harmonic mixing, cross-band ISO, IMDn}.

  - FFS handling of UL/DL harmonic order

  - FFS on max MSD order n

  - If new MSD type is identified and specified by RAN4 in the future, the new MSD type can also be considered for indicating lower MSD capability

Actually, the most companies already accept that based on the idea of selecting worst case, the harmonic order is not needed. This would greatly help to control the complexity, and also should be sufficient for the verification.

For the IMD order, the original proposal of up to 9 is already have much margin than actually needed. It is believe even an order of 4/5 is actually enough considering the case larger than this is quite limited.

Proposal 3: The harmonic order is not needed for signalling, and IMD order up to 4/5 is enough.

Candidate MSD thresholds

Currently the WF is as following:

- Candidate options:

  - Option 1: 0~15dB (Nokia, AT&T)

  - Option 2: 0~20dB (HW, Samsung, Skyworks, Meta, vivo, OPPO, Xiaomi)

  - Option 3: 0~[25/27]dB (LGE, QC)

  - Option 4: improved MSD in granularity of 1dB (MTK, ZTE)

It can be seen that the range 0~20 is the majority choice (better than half) even among 4 different options. If options can be simplified, this option is likely to gain more support. We can also accept 0~15dB here, but any value larger than that seems a bit too far from what “lower MSD” is designed for.

Proposal 4: Choose 0~20dB as compromise range for lower MSD. Any range larger than that would be deviating too much from what “lower MSD” should pursued.

Based on the range, the following suggested thresholds as in [6] was resubmitted. This could be regarded as a trade off between various factors.

Proposal 5: The following thresholds are proposed:

  - 0≤ MSD＜3 dB

  - 3≤ MSD＜6 dB

  - 6≤ MSD＜12dB

  - 12≤ MSD＜[20]dB

Conformance test for lower MSD

This has been specifically discussed with the LS to RAN5 of [4], it is proposed to continue discussion on this issue after some feedback could be obtained from RAN5.

Proposal 6: Continue discuss conformance test configurations related topic for lower MSD after receiving RAN5 reply.

Other approaches for lower MSD capability reporting

In this issue, many different approaches were proposed. Among them, the single -bit low-MSD indicator for a UE seems to gain support from a considerable number of companies and have different variations. This seems a potential convergence point that may bring some simplicity for network and merit to be discuss more.

Proposal 7: Single-bit low-MSD indicator for a UE is proposed to do more study and considered.

Conclusion

In this paper, some views were provided for the remaining issues:

Observation 1: The idea of mandating the reported MSD value should be improved at least by TBD dB, is against the previous agreement of use “absolute thresholds”, and may greatly increase the complexity of verification.

Proposal 1: Do not consider mandating the reported MSD value should be improved at least by TBD dB.

Proposal 2: Do not report CBW of aggressor UL and victim DL for lower MSD capability.

Proposal 3: The harmonic order is not needed for signalling, and IMD order up to 4/5 is enough.

Proposal 4: Choose 0~20dB as compromise range for lower MSD. Any range larger than that would be deviating too much from what “lower MSD” should pursued.

Proposal 5: The following thresholds are proposed:

  - 0≤ MSD＜3 dB

  - 3≤ MSD＜6 dB

  - 6≤ MSD＜12dB

  - 12≤ MSD＜[20]dB

Proposal 6: Continue discuss conformance test configurations related topic for lower MSD after receiving RAN5 reply.

Proposal 7: Single-bit low-MSD indicator for a UE is proposed to do more study and considered.

- References

[1] R4-2306590, WF on lower MSD, Huawei, RAN4#106bis-e

[2] R4-2306285, Topic summary for [106-bis-e][123] FR1_enh2_part1, Moderator (Huawei), RAN4#106bis-e

[3] R4-2306594, draft LS on lower MSD capability, Huawei, RAN4#106bis-e

[4] R4-2306591, draft LS on clarification of test configurations for CA/DC MSD requirements, Samsung, RAN4#106bis-e

[5] R4-2305563, TR 38.881 v0.4.0, Huawei, RAN4#106bis-e

[6] R4-2305083, Discussion of lower MSD Signalling, vivo, RAN4#106bis-e

[7] R4-2304187, Lower MSD requirements handling, Nokia, RAN4#106bis-e