# 3GPP TSG-RAN WG4 Meeting #107	R4-2308273

# Incheon, KR, May 22 – May 26, 2023

Source: 	vivo

Title: 	Further discussion on remaining issues for SL CA

Agenda Item:	8.31.2.3

Document for:	Approval

Introduction

In the last meeting, RAN4 fully started the discussion on NR SL CA, including general aspects, channel bandwidth class and aggregated channel bandwidth, Tx/Rx RF requirements [1]. For some remaining issues for SL CA, we provide our views in this contribution.

Discussion

SL CA general part

For the general aspects for SL CA, it was agreed to capture the requirement for SL intra-band CA under suffix E in RAN4 specification. Currently, section E.1 is for V2X operating bands, E.2 for V2X operating bands for con-current operation. We suggest to allocate a new section E.3 for SL CA. For SL-U, it is still under discussion how to capture the requirements. For inter-band con-current operation with SL-U bands, it can be merged with E.2 for con-current operation.We suggest to arrange the requirements for suffix ‘E’ as follows:

- E.1 V2X operating bands

- E.2 V2X operating bands for con-current operation

- E.3 Sidelink CA

The contents for E.1 and E.2 can be kept as it is and the additional requirements for SL CA can be added in the consecutive E.3.

Proposal 1: Arrange the requirements for NR SL CA as following sections：

- E.1 V2X operating bands

- E.2 V2X operating bands for con-current operation

- E.3 Sidelink CA

SL Tx RF requirements

In [1], the methodology on Tx requirements for SL intra-band contiguous CA was discussed.

Methodology on Tx requirements for SL intra-band contiguous CA

  - Option 1: Reusing the existing requirements of NR intra-band contiguous CA

  - Option 2: LTE intra-band contiguous CA requirements are considered as baseline

  - Option 3: For further study

In the last meeting, we agreed to reuse NR CA bandwidth classes A and B and the SL CA configurations for SL_n47 was agreed as follows:

| NR V2X intra-band con-current operating configuration | NR Band | Interface | Channel bandwidth (MHz) (NOTE 1) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| SL_n47B | n47 | PC5 | 10, 20, 30, 40 | 70 | 0 |
|  | n47 |  | 10, 20, 30 |  |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1 in TS38.101-1. |  |  |  |  |  |

It can be observed that at least the terminology for bandwidths class and aggregated bandwidths followed NR CA, not LTE CA. For the Tx requirements, requirements were defined for LTE V2X_47B and V2X_47C, only intended for 20M or 30M maximum aggregated bandwidth, which is not the same with NR SL CA. The Tx requirements is likely to be re-evaluated for NR SL CA. Instead of saying reusing the requirements for LTE or NR intra-band contiguous CA, we need to study the Tx requirements case by case.

Proposal 2: Instead of saying reusing the requirements for LTE or NR intra-band contiguous CA, we need to study the Tx requirements case by case.

In NR V2X, both PC3 and PC2 are defined for single carrier operation in band n47. For the power class supported in SL_n47, PC3 is confirmed in Rel-18. It needs further discussion whether to introduce PC2 for NR SL CA in band n47. From our perspective, it is beneficial to support PC2 in SL_n47B since it can reuse the PC2 PA for single carrier operation in band n47. We suggest to study PC2 for SL_n47B for SL CA in Rel-18.

Proposal 3: It is suggested to study PC2 for SL_n47B for SL CA in Rel-18.

SL Rx RF requirements

In the last meeting, we agreed to specify reference sensitivity by considering the aggregated channel bandwidth for SL intra-band contiguous CA. However, this agreement is confusing to us. Since for SL CA, the reference sensitivity should be defined for each component carrier with all carriers active. For SL_n47B, the reference sensitivity defined for single carrier operation in n47 as shown in Table 7.3E.2-1 shall apply for each component carrier with two CCs are activated at the same time.

Table 7.3E.2-1: Reference sensitivity of NR V2X Bands (PC5) (Section 7.3E.2, TS38.101-1)

| NR V2X Band | SCS kHz | Channel bandwidth / PREFSENS_V2X(dBm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5MHz4 | 10 MHz | 20 MHz | 30 MHz | 40 MHz | Duplex Mode |
| n47 | 15 |  | -92.5 | -89.2 | -87.4 | -86.1 | HD |
|  | 30 |  | -92.1 | -89.4 | -87.7 | -86.2 |  |
|  | 60 |  | -92.9 | -89.1 | -87.9 | -86.4 |  |

Proposal 4: For SL_n47B, the reference sensitivity defined for single carrier operation in n47 as shown in Table 7.3E.2-1 shall apply for each component carrier with two CCs are activated at the same time.

Conclusion

This contribution discusses remaining issues for SL CA. The following observations and proposals are made:

Proposal 1: Arrange the requirements for NR SL CA as following sections：

- E.1 V2X operating bands

- E.2 V2X operating bands for con-current operation

- E.3 Sidelink CA

Proposal 2: Instead of saying reusing the requirements for LTE or NR intra-band contiguous CA, we need to study the Tx requirements case by case.

Proposal 3: It is suggested to study PC2 for SL_n47B for SL CA in Rel-18.

Proposal 4: For SL_n47B, the reference sensitivity defined for single carrier operation in n47 as shown in Table 7.3E.2-1 shall apply for each component carrier with two CCs are activated at the same time.

- References

[1] R4-2306637, WF on SL CA, Huawei, HiSilicon, RAN4#106bis-e.