3GPP TSG-RAN WG4 Meeting #107	R4-2308274

Incheon, Korea, May 22 – May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.872 | CR | - | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR 38.872 to introduce the general part and UE RF architecture assumption for CA_n26-n28 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_sub1GHz_combo_enh |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SI Study on enhancement for sub-1GHz NR band combinations, the study of CA_n26-n28 for a smart phone has been proposed. The general part and UE RF architecture assumption need to be introduced for CA_n26-n28. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce the general part and UE RF architecture assumption for CA_n26-n28, based on the approved WF R4-2306471. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The general and UE RF architecture assumption for CA_n26-n28 will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1, 5.6.2.1, 5.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change for TR 38.872>>

5.6	CA_n26-n28

5.6.1	General

In this configuration the uplink is also CA_n26A-n28A, but there are two fall back options:

- DL_n26A-n28A_UL_n28A_BCS0 (already completed)

- UL_n26A-n28A_UL_n26A_BCS0

The implementation of the band n28 of CA_n26_n28 has been discussed, and the pros and cons of the following two cases can be further traded-off, whether and/or how to do the down-selection in the future is for further study.

Case 1: lower 30MHz of n28 (UL: 703~733MHz, DL: 758~788MHz) is supported.

Case 2: entire n28 frequency range (UL: 703~748MHz, DL: 758~803MHz) is supported.

Figure 5.6.1-1 CA_n26-n28 frequency ranges

Besides, as a reference for further study, proponent operator provided some related information about the allocated spectrum in the 700MHz and extended 850 MHz bands as sketched in the following figure and table.

Figure 5.6.1-2 n26 and n28 spectrum allocation

Table 5.6.1-1 n26 and n28 spectrum allocation

|  | Uplink | Downlink |
| --- | --- | --- |
| n28 (national) | 713 MHz – 733 MHz | 768 MHz – 788 MHz |
| n26 (Metropolitan) | 814 MHz – 824 MHz 835 MHz – 845 MHz | 859 MHz – 869 MHz 880 MHz – 890 MHz |
| n26 (Regional) | 814 MHz – 824 MHz 830 MHz – 845 MHz | 859 MHz – 869 MHz 875 MHz – 890 MHz |

5.6.2	UE RF architecture assumption

The potential UE RF architectures for CA_n26A-n28A case 1 and case 2 can be considered as shown below. Baseline architecture for UE RF requirements development will be further discussed. As the performance of band n26 is similar to band n28, after the development of CA_n5-n28, some architectures could be used as a reference to CA_n26-n28.

5.6.2.1	UE RF architecture with lower 30MHz of n28 supporting

Architecture of case 1 (lower 30MHz of n28 (UL: 703~733MHz, DL: 758~788MHz) is supported) is listed as below. Entire n28 presents a greater challenge to filter implementation, so further research on feasibility and applicable filtering techniques should be considered.

The topology for architecture 1 with 2 antenna, as shown in Figure 5.6.2.1-1:

1) antenna 1 with quadplexer supporting n26DL+n26UL+n28AUL+n28ADL

2) antenna 2 with duplexer supporting n26DL+n28ADL

The topology for architecture 2 with 2 antenna, as shown in Figure 5.6.2.1-2:

1) antenna 1 with triplexer supporting n26DL+n28AUL+n28ADL

2) antenna 2 with triplexer supporting n26DL+n26UL+n28ADL

The topology for architecture 3 with 3 antenna, as shown in Figure 5.6.2.1-3:

1) antenna 1 with duplexer supporting n28AUL+n28ADL

2) antenna 2 with duplexer supporting n26UL+n26DL

3) antenna 3 with duplexer supporting n26DL+n28ADL

Figure 5.6.2.1-1 Architecture 1: Quadplexer solution for CA_n26-n28 with two antennas

Figure 5.6.2.1-2 Architecture 2: Triplexer solution for CA_n26-n28 with two antennas

Figure 5.6.2.1-3 Architecture 3: Three-antenna solution for CA_n26-n28

5.6.2.2	UE RF architecture with entire n28 frequency range supporting

Architecture of case 2 (entire n28 frequency range (UL: 703~748MHz, DL: 758~803MHz) is supported) is listed as below.

The topology for architecture 4 with 2 antenna, as shown in Figure 5.6.2.2-1:

1) antenna 1 with dual-quadplexer supporting n26UL+ n26DL+ n28AUL+ n28ADL_ n26UL+ n26DL+ n28BUL+ n28BDL

2) antenna 2 with duplexer supporting n26DL+n28DL

The topology for architecture 5 with 2 antenna, as shown in Figure 5.6.2.2-2:

1) antenna 1 with dual-triplexer supporting n26DL+ n28AUL+n28ADL_ n26DL+ n28BUL+n28BDL

2) antenna 2 with triplexer supporting n26DL+n26UL+n28UL

The topology for architecture 6 with 3 antenna, as shown in Figure 5.6.2.2-3:

1) antenna 1 with dual-duplexer supporting n28AUL+n28ADL_ n28BUL+n28BDL

2) antenna 2 with duplexer supporting n26UL+n26DL

3) antenna 3 with duplexer supporting n26DL+n28ADL

Figure 5.6.2.2-1 Architecture 4: Dual-quadplexer solution for CA_n26-n28 with two antennas

Figure 5.6.2.2-2 Architecture 5: Dual-triplexer solution for CA_n26-n28 with two antennas

Figure 5.6.2.2-3 Architecture 6: Three-antenna solution for CA_n26-n28

For architecture 4 with 2 antenna, there are some potential implementation challenges:

- The main antenna should simultaneously cover the whole band from n28UL to n26DL with the length of 191MHz, the bandwidth ratio is as high as 23.9%, it would far exceed the bandwidth ratio for a typical planar antenna design in a smartphone, therefore the radiative performance for CA_n26A-n28A is expected to be compromised.

- The performance requirements of quadplexer such as filter isolation and insertion loss are strict, posing challenge to the implementation and cost of the UE front-end, and the feasibility of two quadplexers may need further study.

For architecture 5 with 2 antenna, there are some advantages and potential implementation challenges:

- A two-antenna architecture with three triplexers can be easier to design with proper delta T and delta R, so architecture 2 seems to be more feasible.

- The difficulties of triplexer implementation still remain with two gaps of 10MHz, n26 BW of 35MHz and n28DL of 45MHz.

For architecture 6 with 3 antenna, there are some advantages and potential implementation challenges:

- For the 3-antenna architecture, there are two antennas on the main signal path, which eases the difficulty of multiplexer design and implement. The frequency range for each main antenna is narrower compared with 2-antenna architecture, and the antenna BW ratios of the two antennas would be close to the current devices.

- 3 LB antennas present a greater challenge to the dimension of the mobile phone than 2-amtenna architecture.

<<End of Change>>