3GPP TSG-RAN WG4 Meeting # 107	R4-2308317

Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.9.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on L3 enhancement for FR2 SCell activation delay reduction

Document for:	Discussion

# Introduction

The Rel-18 WI of even further RRM enhancement was approved in [1] and further revised in [2]. One of the objectives is to study the SCell activation delay reduction in FR2. In RAN4#105, some high-level principle was agreed in [3]. In RAN4#l06 meeting, the scope of the solutions was further narrowed down [4]. In RAN4#106bis, considerable progress has been made with agreements captured in [5]. In this paper, we further provide our views of RRM impact of the objective and provide further analysis on the solutions.

# Discussion

## 2.1 L3 measurement reporting for legacy unknown case

For L3 reporting during SCell activation, considerable progress has been made in last meeting, and an LS [6] is sent to RAN2 for signaling design. The status is summarized as follows:

Sub-topic 1-1 L3 reporting during SCell activation Issue 1-1-1: when to report L3 measurement results for unknown FR2 SCell activation enhancement - Agreements (GTW, Monday Apr 17, 2023) - UE needs to report the L3 measurement result after SCell activation command - FFS if additional solutions should be considered. Decision on additional solutions need to be made no later that in RAN4 #107. Issue 1-1-2: FFS on how to trigger L3 report for unknown FR2 SCell activation enhancement - Agreements (GTW, Monday Apr 17, 2023) - The L3 measurement report for unknown FR2 Scell activation after Scell activation command is triggered by Scell activation command. - FFS whether to have additional events for report triggering - Send LS (R4-2306321) to let RAN2 to decide. Issue 1-1-3: FFS on how to configure L3 report for unknown FR2 SCell activation enhancement - Agreement: - The one-time report after SCell activation command is assumed in this enhancement - Send LS (R4-2306321) to let RAN2 to decide. Issue 1-1-4: FFS on how to report L3 measurement result for unknown FR2 SCell activation enhancement - Agreement: - Send LS (R4-2306321) to let RAN2 to decide. Issue 1-1-5: FFS on content in valid L3 measurement report for unknown FR2 SCell activation enhancement Agreement: - L3 measurement result with SSB index

Based on the progress in last meeting, the triggering/configuration/reporting are up to RAN2 to decide, and there is no need to have duplicated discussion in RAN4.

Proposal 1: RAN4 to wait for RAN2 conclusion on triggering/configuration/reporting, and there is no need to have further discussion in RAN4.

Another issue is about whether to define criteria on whether the results are valid or not, and whether UE shall report if UE has no valid measurement results. The status is summarized as follows:

Issue 1-1-6: If measurement results are available, the UE will report them to the NW. How to determine the measurement result is available? FFS: - Option 1 (Apple, OPPO, Huawei, Xiaomi, vivo): - No need to define criteria to determine the L3 measurement result is available or not for FR2 unknown SCell activation enhancement. - Option 2 (Nokia, Ericsson): - The L3 measurement report is considered as valid only if it fulfils the measurement requirement for a deactivated Scell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2). - Option 3 (Intel): - If UE already has available L3 measurement result when Scell activation command is triggered, the measurement result can be reported only if it satisfied the following conditions: - Measurement should satisfy the requirement for a deactivated Scell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2) - the time duration between measurement and Scell activation command is less than [X]s. - Option 4 (MTK): - RAN4 to define a new window [W]s to determine the freshness of the available L3-RSRP measurements at the UE prior to the reception time of Scell activation command. - Proposal 6: If L3-RSRP measurements are within [W], then the UE can send L3-RSRP report. - Option 6 (vivo): - The criteria to determine whether a measurement result is valid (i.e. available), including the signal quality thresholds and the maximum time-delta of the measurement before the reporting, are configured by network - Option 7 (CMCC, ZTE): - for L3 measurement report after Scell activation command, the reported SS-RSRP, SS-RSRQ, and SS-SINR measurements need to meet the accuracy requirements spedified in Clause 10. - Option 8 (ZTE): - To address the concern of the quick report is valid or not, the verification lies in two aspects: - 1)Whether the measurement report is fresh enough. - 2)Whether the quality of measurement result in the report is good or available enough. - The 1st aspect can be ignored since same issue exists in legacy L3 report. The 2nd aspect should be considered to guarantee the NW can obtain useful RSRP and SSB index info from the report. - Option 9 (Apple, CMCC, LGE, CTC, QC, OPPO): - Option 1 and the valid reporting must meet the existing measurement delay/accuracy requirement. Issue 1-1-7: FFS on necessity of L3 measurement reporting if UE has no valid measurement results? FFS: - Option 1 (Apple, ZTE, CTC, OPPO, Ericsson, Xiaomi, HW, vivo, QC): No need to report L3 measurement reporting after receiving SCell activation command if UE has no valid measurement results. - Option 2 (Nokia, Intel, vivo, Ericsson): The UE may continue the measurement and report a valid L3 measurement result when it is ready, if there is no valid L3 measurement result at the time of Scell activation. - Option 3 (LGE, CTC, MTK): If a UE has no valid measurement results, UE needs to report even the lowest RSRP reported value to notice whether a UE has valid measurement results to the network. - Option 4 (vivo, MTK): If the reporting of the results is carried by uplink L1L2 signaling, and UE has no valid measurement results at the L1L2 reporting occasion, UE may report -Inf value, e.g. ‘RSRP<-156dBm’. - Option 5 (MTK): when report triggering conditions are met, but UE has no fresh L3-RSRP measurements within [W] period, then instead of sending the report UE indicates unknown status for the target Scell.

From our understanding, there are following aspects to be studied regarding the definition of valid measurement result.

- Whether UE need to report L3 report if UE has no valid/available measurement report

- Whether to define criteria for valid/available measurement report

- Whether to have conditions on the time span between the measurement and SCell activation command

Firstly, there is no need to have criteria for valid/available measurement report, which is similar as legacy known case. In legacy requirements, RAN4 only define the time instance when the results are reported and there is no restriction on when the results are obtained. It is possible that the measurement is performed long before the reporting instance (e.g. for periodic reporting). But we also agree that it may cause some problem for the case mentioned above. However, it is up to UE implementation on how long UE shall maintain the previous measurement result. Some companies proposed to clarified that the measurement shall meet the accuracy/delay requirements. From our understanding, it is not clear what the delay refers to since it is normal L3 measurement for deactivated SCell before the reporting is triggered. Regarding the accuracy requirements, the measurement could be performed at T1 and reported at T2 after Scell activation, then it is not clear what is the ideal result when talking about measurement accuracy.

Observation 1: The accuracy/delay requirements for measurement reporting after Scell activation command is not clear.

Proposal 2: No need to define criteria to determine the L3 measurement result is available/valid or not for FR2 unknown SCell activation enhancement.

Then regarding whether to report if UE has no valid/available measurement report. Since it is up to UE implementation to decide whether the results are valid or not, there is no need to report anything (e.g. minimum value) to NW as it won’t give any useful information. Instead, it may cause confusion to NW that NW may consider that the quality of the SCell is poor and delete it accordingly.

Proposal 3: No need to report L3 measurement reporting after receiving SCell activation command if UE has no valid measurement results.

However, we believe it worth discussion what is the impacts on SCell activation requirements if the time instance when the measurement is performed is far away from Scell activation command. For instance, the deactivated Scell is measured at t1 and the scell activation command is received at t2. If the measCycle is large or there are multiple carriers to be measurement, the actual measurement periodicity for this deactivated SCell could be long. Then, the timing drift is nonnegligible, and it is questionable whether UE can directly perform fine timing based on stored information.

Observation 2: When the time instance when the measurement is performed is far away from Scell activation command (e.g. large measCycle with multiple carriers to be measured), whether UE can skip L3 and L1 parts is questionable.

However, as analyzed above, it is hard to determine when the result is obtained by the UE since it is up to UE implementation to scheduling the measurement activities. In legacy requirements, there is similar discussion for known SCell activation, where RAN4 use following descriptions. For L3 reporting after SCell activation, we think similar approach can be used as applicability conditions.

- TFirstSSB+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms. - TFirstSSB_MAX + Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms.

Observation 3: It is hard to determine when the result is obtained by the UE since it is up to UE implementation.

Proposal 4: RAN4 to discuss applicability conditions for requirements based on L3 reporting after SCell activation that the measurement period of the SCell being activated is equal to or smaller than X ms.

Regarding the delay reduction, the status is summarized as follows:

Issue 1-1-8: FFS: If valid L3 measurement results are reported, L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported. FFS: - Option 1 (Apple, Nokia, Intel, QC, MTK, LGE, Xiaomi, vivo, CMCC, ZTE, CTC, OPPO): - if UE reports valid L3 measurement result, L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported; - Proposal 1a (Apple, CTC, QC): after receiving SCell activation command, - if UE reports valid L3 measurement result before n+X slot (n is the slot when UE received SCell activation command), L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported; - otherwise, it shall be treated as legacy unknown case. - Proposal 1b(QC, CTC): - RAN4 shall define the reporting delay requirements from MAC-CE to trigger report and actual measurement report. If UE do not report measurement results during the reporting delay requirements, the new measurement configuration shall be expired. - Proposal 1c(vivo): - Send LS to RAN2, asking RAN2 to design a timer, so as to supervise the longest period between gNB triggering the reporting and the actual result being sent by the UE in PUSCH. The RRM requirements are not enhanced in R18 for the case when a UE does no report a valid measurement result before this timer expires. - Proposal 1d(OPPO): - FFS the side conditions. - Proposal 1e(Nokia): - A-TRS for fast SCell activation can be triggered based on the valid L3 measurement report after SCell activation command. - Proposal 1f(Xiaomi): - If no valid result to be reported, the UE needs to perform L3 measurement and L1 measurement with/without enhanced requirement according to UE capability. - Option 2 (HW, vivo, Apple (bullet #2)): (moderator: this option is also up to the conclusion in issue 1-1-1) - When the new triggered measurement reporting is reported before SCell activation command, it can be considered as known cases. - When the new triggered measurement reporting is reported after SCell activation, it can be considered as unknown case where the uncertainty for TCI can be defined as the time period between TCI configuration/activation relative to the first valid L1-RSRP reporting and the new triggered L3 measurement report which occurs earlier. - Option 3 (Ericsson): - RAN4 to agree that L3 part can be skipped and whether L1 part can be skipped or not pending on whether NW sends TCI command or not. - Option 4 (moderator): - use option 1 as starting point, - FFS on the detailed requirement design, e.g. option 2.

Most companies support the L1 and L3 can be skipped. However, as commented and explained in last meeting, we have concerns on such statement. From UE side, L1 procedure and SCell activation are independent, we cannot declare that L1 procedure can be skipped if UE can report L3 reports after SCel activation. Besides, from NW side, it is up to NW implementation to choose how to configure the TCI (e.g. whether based on L3 report after SCell activation command or based on legacy L1-RSRP). More important, for L1-RSRP and the L3 report, which one comes earlier varies case by cases.

Proposal 5: It is up to NW implementation to choose whether to configure TCI based on L3 report after SCell activation command or based on L1-RSRP as legacy cases.

In legacy requirements, the uncertainty for TCI configuration for unknown case are define as follows:

Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to - SCell activation command for known case; - First valid L1-RSRP reporting for unknown case. Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to - SCell activation command for known case; - First valid L1-RSRP reporting for unknown case. Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to - SCell activation command for known case; - First valid L1-RSRP reporting for unknown case.

As mentioned above, for first L1-RSRP and L3 reporting after SCell activation command, it is indeterminate which one can occur earlier. What’s more, for instance, even L3 reporting happens before L1-RSRP, NW can still choose to configure TCI based on L1-RSRP. Thus, the requirements can be updated accordingly that the uncertainty for TCI can be defined as the time period between TCI configuration/activation relative the first valid L1-RSRP reporting and the NW triggered L3 measurement report which occurs earlier. More specifically, when L3 measurement is reported and NW configure TCI based on L3 report, UE shall be able to proceed the SCell activation (e.g. fine timing) regardless of whether L3 and L1 measurement has been completed or not.

Proposal 6:

When the NW triggered measurement reporting is reported after SCell activation, it can be considered as unknown case where the uncertainty for TCI can be defined as the time period between TCI configuration/activation relative the first valid L1-RSRP reporting and the NW triggered L3 measurement report which occurs earlier.

## 2.2 Beam related enhancement

In last meeting, the beam sweeping factor reduction and sample reduction were discussed jointly with following compromised way forward:

Issue 1-2-1: Beam sweeping factor enhancement in L3 and L1 part of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception) Agreement: - For unknown FR2 SCell activation enhancement, introduce the UE capability to support Rx beam sweeping factor less than 8 for cell detection part and SSB based L1-RSRP measurement. - if UE has full set (N=8) of beam sweeping during AGC settling part during FR2 unknown SCell activation procedure. - Introduce beam sweeping factor capability of X1 for cell detection part (X1*Trs) and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement - Beam sweeping factor capability X1/X2 are two independent capabilities - Note: above enhancement only applies for FR2 unknown SCell activation enhancement - Note: How to capture in spec can be discussed in CR stage - The candidate values for X1/X2 are FFS Issue 1-2-2: can X1(Beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation) be zero? (not related with WI of FR2 multi-Rx chain DL reception) The discussion is related with the previous agreement: Agreement in last meeting R4-2303228 Issue 1-3-3: enhancement of “8*Trs” part of current FR2 unknown Scell activation delay Agreement: - To keep “X1*Trs” part of current FR2 unknown Scell activation delay in the delay requirement for FR2 Scell activation enhancment. - X1 can be less than 8 in the beam sweeping factor reduction discussion for cell detection stage in L3 part based on UE capability.. FFS: - Option 1 (Apple, MTK, Xiaomi, HW (option 4b in issue 1-2-1 comments), Intel, QC, OPPO): no - Option 2 (Ericsson, vivo, Nokia, HW (option 4a in issue 1-2-1 comments)): Yes

The only remaining issue is to discuss the value of X1 and X2 and how to indicate the value (separate indicators or per pair indicator). For X1, we think (4,6) is enough since it is for L3 when rough bema is used. For X2, since X2 is to indicate the beam sweeping factor for L1 measurement, it is reasonable to keep the same granularity as maxNumberRxBeam.

Proposal 7: The candidate value for X1 and X2 are (4,6) and (2,3,4,5,6,7) respectively.

## 2.3 Others

There was discussion on using SSB periodicity to replace SMTC periodicity for FR2 unknown SCell activation with following proposals:

Issue 1-5-1: whether to use SSB periodicity instead of SMTC periodicity for FR2 unknown Scell activation FFS: - Option 1 (Apple, QC, Ericsson, Nokia, ZTE, Intel, OPPO): - For enhanced unknown FR2 Scell activation requirement, RAN4 to use SSB periodicity instead of SMTC periodicity. - Option 2 (HW): - Do not change the definition/usage of SMTC periodicity for FR2 unknown Scell activation requirements. - Option 3 (Intel): - For L3 measurement during L3 sync steps, the measurement interval is dependent on SMTC periodicity or SSB periodicity of Scell. - Option 4 (Moderator): - For enhanced unknown FR2 Scell activation requirement, RAN4 to use SSB periodicity instead of SMTC periodicity. - Such enhancement is only applied for FR2 unknown SCell activation. Agreement (GTW, Tuesday, Apr 25, 2023) - FFS whether to use SSB periodicity instead of SMTC periodicity for FR2 unknown Scell activation - Option 1: - Do not change the definition/usage of SMTC periodicity for FR2 unknown Scell activation requirements. - Option 2: - For enhanced unknown FR2 Scell activation requirement, RAN4 to use SSB periodicity instead of SMTC periodicity. - Such enhancement is only applied for FR2 unknown SCell activation.

Existing definition of TSMTC_MAX and Trs are shown as follows. For FR2 unknown SCell activation, the targeting scenario is the case when there is no active cell in the same band. Thus, TSMTC_MAX here equal to the SMTC of the to-be-activated SCell.

- In FR2, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA; in case of FR2 inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated. Trs is the SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs is applied with Trs = 5ms assuming the SSB transmission periodicity is 5ms. There are no requirements if the SSB transmission periodicity is not 5ms

Then, the SMTC could be derived from three ways: 1) SMTC explicitly configured in RRC configuration; 2) SMTC in the MO; 3) 5ms periodicity.

smtc The SSB periodicity/offset/duration configuration of target cell for NR SCell addition. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in sCellConfigCommon. The smtc is based on the timing of the SpCell of associated cell group. In case of inter-RAT handover to NR, the timing reference is the NR PCell. In case of intra-NR PCell change (standalone NR) or NR PSCell change (EN-DC), the timing reference is the target SpCell. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

We have raised some issues on replacing SMTC periodicity by SSB periodicity in last meeting. For above three cases, SMTC is configured in SCellConfig, it is already stated that the periodicity shall be same as ssb-periodicityServingCell, thus there is no need to have such replacement. For the case when SMTC is not configured either in SCellConfig or MO, UE shall assume that SSB periodicity is 5ms and there are no requirements if the periodicity is not 5ms. For this case, we prefer not to have such replacement which may cause compatibility issue. Thus, the replacement shall only be considered when SMTC is only configured in MO.

Besides, when considering multiple SCell activation, UE shall still follow the SMTC periodicity for AGC settling.

Proposal 8: For enhanced unknown FR2 Scell activation requirement, RAN4 to use SSB periodicity instead of SMTC periodicity when the SMTC is only configured in MO.

# Conclusions

Proposal 1: RAN4 to wait for RAN2 conclusion on triggering/configuration/reporting, and there is no need to have further discussion in RAN4.

Observation 1: The accuracy/delay requirements for measurement reporting after Scell activation command is not clear.

Proposal 2: No need to define criteria to determine the L3 measurement result is available/valid or not for FR2 unknown SCell activation enhancement.

Proposal 3: No need to report L3 measurement reporting after receiving SCell activation command if UE has no valid measurement results.

Observation 2: When the time instance when the measurement is performed is far away from Scell activation command (e.g. large measCycle with multiple carriers to be measured), whether UE can skip L3 and L1 parts is questionable.

Observation 3: It is hard to determine when the result is obtained by the UE since it is up to UE implementation.

Proposal 4: RAN4 to discuss applicability conditions for requirements based on L3 reporting after SCell activation that the measurement period of the SCell being activated is equal to or smaller than X ms.

Proposal 5: It is up to NW implementation to choose whether to configure TCI based on L3 report after SCell activation command or based on L1-RSRP as legacy cases.

Proposal 6:

When the NW triggered measurement reporting is reported after SCell activation, it can be considered as unknown case where the uncertainty for TCI can be defined as the time period between TCI configuration/activation relative the first valid L1-RSRP reporting and the NW triggered L3 measurement report which occurs earlier.

Proposal 7: The candidate value for X1 and X2 are (4,6) and (2,3,4,5,6,7) respectively.

Proposal 8: For enhanced unknown FR2 Scell activation requirement, RAN4 to use SSB periodicity instead of SMTC periodicity when the SMTC is only configured in MO.

# References

[1] RP-220977 New WID: Even Further RRM enhancement for NR and MR-DC

[2] RP-221696 Revised WID: Even Further RRM enhancement for NR and MR-DC

[3] R4-2220440, WF on R18 eFeRRM - FR2 SCell activation enhancement

[4] R4-2303228, Way forward on R18 RRM enhancement (part 1)

[5] R4-2306315, WF on R18 RRM enhancement- FR2 SCell activation delay reduction

[6] R4-2306321, LS on FR2 unknown SCell activation enhancement