3GPP TSG-RAN WG4 Meeting # 107	R4-2308335

Incheon, KR, May 22 – May 26, 2023

Title: 	Discussion on multi-panel simultaneous reception in FR2 eHST

Source: 	Huawei, HiSilicon

Agenda item:	8.13.4.1

Document for:	Discussion

Introduction

The R18 WI on enhanced NR support for high speed train scenario in FR2 was discussed. One objective of this WI is simultaneous multiple-panel operation in FR2 HST. The related content in WID is duplicated as below [1]. This contribution further provides analysis on RRM impact by multi-panel simultaneous reception in FR2 eHST.

- Specify the requirement for simultaneous multi-panel operation for train roof-mounted FR2 high power devices [RAN4]: - Maximum 2 active panels supporting the multi-panel simultaneous reception. - NOTE: Focus on FR2 HST specific requirements, and avoid the overlap with the scope of FR2 multi-Rx DL reception

Discussion

- Multi-RX operation

In last meeting, the following agreement is achieved in [2]. It means that CPE has to perform simultaneous reception and transmission from two directions. However multi-TX operation is neither within this WI scope nor within R18 Multi-RX WI. Moreover it has been agreed in RAN4#104bis meeting, simultaneous UL transmission from multi-panel is precluded from this WI [3].

Agreement: - RAN4 to follow Rel-17 NR FR2 HST for CPE configuration assumptions where the CPE UE is equipped with 2 panels each for both TX and RX pointing in opposite directions Way forward: - RRM requirement impact from HST FR2 for simultaneous multi-panel operation should only be focused on - Measurements - L1 measurement - Simultaneous data reception and L1 measurement - Scheduling restriction between simultaneous data reception and L1 measurement in multi-panel operation - FFS other potential requirement impacts including, simultaneous data reception and L3 measurements, L1 and L3 simultaneous measurements and accordingly measurement/scheduling restriction, - FFS based on identifying potential gains or issues

- Transmission Scheme for FR2 HST multi-panel simultaneous reception Agreements: - Simultaneous UL transmissions from multiple UE panels is not in the scope of the WI. - RAN4 to consider at least NC JT scheme in HST FR2 Enhanced deployments.

Proposal 1: Simultaneous UL transmissions with 2 panels is not supported in the WI.

In RANP #98e meeting, simultaneous L3 measurement is removed from R18 multi-RX WID [4].

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier - Enhanced RRM requirements: - The following requirements should be studied and specified if necessary: - L1-RSRP measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs)

In last meeting, companies would like to check whether L3+L3 measurement on different panels shall be supported in Rel-18 FR2 HST enhancement WI. In HST SFN scenario, all cells are dedicated HST cell and the serving cell is a “supper” cell with large ISD. UE would perform L3 measurement when it is at neighbour at cell edge. At cell edge, in HST scenario the target cell is the cell UE is approaching to. Then there is no need to consider L3+L3 measurement with simultaneous multi-panel, as target cell is only in one direction.

In non-SFN HST scenario, if neighbour cell is not dedicated HST scenario and the location is not fixed, then the scenario is the same as that in R18 multi-Rx. So the conclusion from Rel-18 FR2 multi-Rx can be reused for HST FR2 UE with multi-panel operation.

Proposal 2: Simultaneous L3 and L3 measurements is not supported in R18 FR2 HST.

In [5], Simultaneous L3 and L1 measurements is not supported in R18 multi-Rx as well.

#### Issue 1-1-6: Simultaneous L3 measurements and L1 measurements - Simultaneous L3 and L1 measurements is not supported in this release for multi-Rx. - Note: The agreement has no impact on scheduling restriction discussion for L3 measurements. Feasibility and benefit of scheduling restriction relaxation for L3 measurements is FFS.

In HST scenario, for the non-SFN HST scenario, to identify all candidate target cell, UE shall perform L3 measurement with RX beam sweeping in wide-ranging. Then it is obvious that the simultaneous L3 and L1 measurements is not able to be supported.

Proposal 3: Simultaneous L3 and L1 measurements is not supported in R18 FR2 HST.

Based on RAN4 agreements in [6, 7], group-based reporting is considered for enabling simultaneous DL receptions from two different beams. To perform group-based reporting, UE needs to perform L1-RSRP measurement in order to report L1-RSRP measured results of one or more resource pairs.

Regarding L1 measurement in the serving cell, we admit that in HST scenario, CPE is fixed equipped on the top of train then the rotation issue for hand-held UE is not necessary to be considered. However in HST scenario, as UE keeps moving, at some times UE is in the middle of two RRH and at some times UE is right under an RRH. For the case that UE is right under one RRH (approaching or leaving), the beam separation from the serving RRH and the proximity RRH is not sufficient. It means that HST scenario has no big difference as non-HST scenario which R18 multi-Rx focus on, the issues exist for R18 multi-Rx also exist for R18 HST with multi-RX.

Observation: Regarding L1 measurement in the serving cell, the issues exist for R18 multi-Rx also exist for R18 HST with multi-RX.

Moreover, if any beam sweeping of L1 measurements is performed, simultaneous reception for L1 and L1 measurement is not expected. The reason is that group-based reporting is considered for enabling simultaneous DL receptions from two different beams. If beam sweeping are performed on both panels, how to guarantee the simultaneous reception beams from different panels are always in reported pair? If beam sweeping is performed on one panels, the situation is worse. First the beam sweeping range would be limited, it means that only partial range would be scaned. Second the same issue as two sweeping case: how to guarantee the simultaneous reception beams from different panels are always in reported pair?

For both L1 measurements without RX beam sweeping, the simultaneous operation can be considered. Then corresponding measurement restriction would be further discussed accordingly.

The same issues are under discussion in R18 multi-Rx WI, conclusion in R18 FR2 multi-Rx can be reused for FR2 eHST.

Proposal 4: For L1/L1 measurement and L1 measurement and data with multi-panel simultaneous reception in FR2 eHST, conclusion in R18 FR2 multi-Rx can be reused.

Another open issue is related with prerequisite of multi-RX operation in HST scenario.

Agreement: - UE capability - The Rel-17 group-based reporting may be be considered in the FR2 HST: - FFS the necessity of Rel-17 group-based reporting in the FR2 HST - FFS the applicability of Rel-17 group-based reporting in the FR2 HST

R18 FR2 multi-Rx has agreed that R17 group-based reporting is a prerequisite for simultaneous reception [5]. We don’t know without group-based reporting scheme, how simultaneous reception from different beam works in HST scenario.

#### Issue 1-2-7: Necessity of group-based beam reporting for simultaneous reception - Rel-17 group-based reporting is used as a prerequisite to define requirement for simultaneous reception - Note: Simultaneous reception term above includes Data/Data, RS/RS and Data/RS simultaneous reception cases.

Proposal 5: Rel-17 group-based reporting is used as a prerequisite to define requirement for R18 FR2 multi-Rx simultaneous reception.

Conclusions

This contribution provides the consideration on R18 enhanced NR support for high speed train scenario in FR2. The following proposals are provided:

Proposal 1: Simultaneous UL transmissions with 2 panels is not supported in the WI.

Proposal 2: Simultaneous L3 and L3 measurements is not supported in R18 FR2 HST.

Proposal 3: Simultaneous L3 and L1 measurements is not supported in R18 FR2 HST.

Observation: Regarding L1 measurement in the serving cell, the issues exist for R18 multi-Rx also exist for R18 HST with multi-RX.

Proposal 4: For L1/L1 measurement and L1 measurement and data with multi-panel simultaneous reception in FR2 eHST, conclusion in R18 FR2 multi-Rx can be reused.

Proposal 5: Rel-17 group-based reporting is used as a prerequisite to define requirement for R18 FR2 multi-Rx simultaneous reception.

Reference

- RP-220985, New WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2), Samsung

- R4-2220397, WF on NR FR2 HST RRM Core requirements impact, Samsung

- R4-2217255, WF on other RRM core requirement impacts for FR2 HST enhancement, Nokia, Nokia Shanghai Bell

- RP-223527, Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm

- R4-2303302, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo

- R4-2220742, WF on NR FR2 multi-Rx chain DL reception – Part 1, vivo

- R4-2220743, WF on NR FR2 multi-Rx chain DL reception – Part 2, Qualcomm Incorporated