3GPP TSG-RAN WG4 Meeting # 107															R4-2308415

Incheon, KR, May 22 – May 26, 2023

Agenda item:			8.31.2.1.3 Rx requirements

Source:	LG Electronics

Title:	On Rx requirements for NR sidelink evolution

Document for:	Discussion and Approval

# Introduction

WF on NR sidelink evolution [1] captured the agreements made in the RAN4#106bis-e meeting and lists the open issues. This contribution addresses these single carrier related issues on Rx requirements and proposes a way forward.

- Issue 4-1-1: Reference measurement channels for NR SL-U

- Issue 4-2-2: RB configuration for REFSENS

# UE RX requirements for NR SL-U single carrier operation

## RB configuration for REFSENS and Reference measurement channels for NR SL-U

The RB definition for the RMC needs to follow the existing subchannel size definition as specified in 38.331.

The proposed number for NRB for NR SL-U maximum transmissions bandwidth and RMC is shown in table 1 below. The values for 20 and 40MHz are reused from NR V2X and for 60, 80 and 100MHz they are defined as follows:

| 60MHz | 80MHz | 100MHz |
| --- | --- | --- |
| 16x 10 | 18x 12 | 27x 10 |
| 5x 15 | 7x 15 | 9x 15 |

Table 1. NRB for NR SL-U maximum transmissions bandwidth configuration and RMC

| NR SL-U |  |  |  |  |  |  | 5.3.2 | Maximum transmission bandwidth configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NRB | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |  | NRB | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| 15k | 105 | 216 |  |  |  |  | 15k | 106 | 216 |  |  |  |
| 30k | 50 | 105 | 160 | 216 | 270 |  | 30k | 51 | 106 | 162 | 217 | 273 |
| 60k | 24 | 50 | 75 | 105 | 135 |  | 60k | 24 | 51 | 79 | 107 | 135 |

#### Proposal #1:

Use the NRB from Table 1 for NR SL-U maximum transmission bandwidth configuration and RMC.

# Conclusions

This contribution addresses single carrier related issues on Rx requirements and makes following proposals.

#### Proposal #1:

Use the NRB from Table 1 for NR SL-U maximum transmission bandwidth configuration and RMC.

# References

[1]	R4-2306632, WF on SLU single CC Rx requirements, OPPO