3GPP TSG-RAN WG4 Meeting #107		R4-2308416

# Incheon, KR, May 22 – May 26, 2023

Agenda item:			8.31.2.4 Co-channel coexistence for LTE sidelink and NR sidelink

Source:	LG Electronics

Title:	On Co-channel coexistence for LTE SL and NR SL

Document for:	Discussion and Approval

# Introduction

WF on NR SL con-current operation and co-channel coexistence scenario of LTE V2X and NR V2X [1] discusses open items linked with co-channel coexistence of LTE V2X and NR V2X. RAN1#112bis-e meeting reached an agreement as captured in section 2.1 below.

This document further discusses these items and makes proposals on how to move forward.

# Discussion

## Transmitted power requirements for NR SL co-channel coexistence with LTE SL

RAN1 agreed as follows.

Agreement (RAN1#112bis-e) For NR SL transmissions of 30kHz SCS with dynamic resource pool sharing, the power level of the NR PSCCH/PSSCH transmission in the first of NR SL slots overlapping with an LTE SL subframe is always larger than or equal to the power level(s) of the NR PSCCH/PSSCH transmission in the subsequent NR SL slot overlapping with the LTE SL subframe. - Note: How to ensure the above condition is up to UE implementation - FFS on whether same or different frequency allocation may be used in the second overlapping slot

The following cases can be considered.

- Case 1: LTE SL (15kHz SCS) + NR SL (15kHz SCS)

- Case 2: LTE SL (15kHz SCS) + NR SL (30kHz SCS)

Figure 3.1 and Figure 3.2 show the impact on LTE SL and NR SL due to each AGC from co-channel operation with Case1 and Case 2, respectively.

Figure 3.1: AGC impact of co-channel operation with LTE SL + NR SL (Case1)

Figure 3.2: AGC impact of co-channel operation with LTE SL + NR SL (Case2)

From the figures, the followings are observed.

Observation 1: For both NR SL SCS of 15kHz and NR SL SCS of 30kHz, there is no impact to NR SL due to co-channel operation with LTE SL. (Figure 3.1, Figure 3.2(A), 3.2(B), 3.2(C), 3.2(D)

Observation 2: For NR SL SCS of 15kHz, there is no impact to LTE SL due to co-channel operation with NR SL. (Figure 3.1)

Observation 3: For NR SL SCS of 30kHz, there is no impact to LTE SL due to co-channel operation with NR SL when the NR SL transmitted power in the first slot of NR SL slots overlapping with an LTE SL subframe is larger than or equal to the NR SL transmitted power in the subsequent NR SL slot overlapping with the LTE SL subframe. (Figure 3.2(C),3.2(D)

Observation 4: For NR SL SCS of 30kHz, there is impact to LTE SL due to co-channel operation with NR SL when the NR SL transmitted power in the first slot of NR SL slots overlapping with an LTE SL subframe is less than the NR SL transmitted power in the subsequent NR SL slot overlapping with the LTE SL subframe. (Figure 3.2(A),3.2(B)

Regarding the RAN1 agreement, the configurations of Figure3.2(A) and Figure3.2(B) should be excluded, which means that RAN4 should analyze if additional functionality or requirements are needed to ensure that NR SL UEs follow the behavior as shown in Figure 3.2 C and/or 3.2D. Is there a need for NR SL / LTE SL co-channel co-existence requirement for 30kHz SCS NR SL UE that ensured that power of the 2nd slot is never higher than the power of the 1st slot (relative requirement)?

Proposal 1: RAN4 should analyze if additional functionality or requirements are needed to ensure that NR SL UEs follow the behavior as shown in Figure 3.2 C and/or 3.2D. Is there a need for NR SL / LTE SL co-channel co-existence requirement for 30kHz SCS NR SL UE that ensures that power of the 2nd slot is never higher than the power of the 1st slot (relative requirement)?

In addition, RAN4 should analyze if this beahvior that ensures the co-channel coexistence can impact some other minimum performance requirements. One example here can be the PUMAX,f,c, which shall be within the bounds:

PCMAX_L,f,c  –  MAX{TL,c, T(PCMAX_L,f,c)}  ≤  PUMAX,f,c  ≤  PCMAX_H,f,c  +  T(PCMAX_H,f,c)

as specified in 6.2.4 in table 6.2.4-1.

In case that clarification in PUMAX,f,c requirement is deemed necessary it could be clarified that PUMAX,f,c tolerances are increased in case of co-channel coexistence or that PUMAX.f.c needs to be measured using the 1st slot only.

Proposal 2: In case that clarification in PUMAX,f,c requirement is deemed necessary it could be clarified that PUMAX,f,c tolerances are increased in case of co-channel coexistence or that PUMAX.f.c needs to be measured using the 1st slot only.

## Power imbalance performance with two links

The power imbalance performance requirements for single RAT defined in 36.101 clause 14.4 for LTE V2X and in 38.101-4 clause 11.1.6 for NR V2X require the receiver to tolerate ~17dB power imbalance for LTE V2X (Reference channel CD.10 with 3 RBs 20M 15kHz SCS) and ~25dB for NR V2X (Reference channel R.PSSCH.2-1.4 with 10 RBs 20M 30kHz SCS).

The power imbalance requirement with two links for LTE V2X and NR V2X is defined using same RAT for both interfering and victim UE, but the demodulation requirement is set only for the victim. As interfering signal in these test cases is basically acting as strong interfering in-band signal, that impacts the AGC, it is fair to assume that this already defined test case also models the scenarios for co-channel coexistence and no new tests cases are needed. There are differences between the waveforms of LTE V2X and NR V2X signals, but changing the modulation and numerology of the in-channel interfering signal while maintaining the power and bandwidth unchanged is expected to have negligible impact.

Proposal 3: The power imbalance requirement with two links for NR V2X is defined using 30kHz SCS for both interfering and victim UE, but the demodulation requirement is set only for the victim. As interfering signal in this test case is basically acting as strong interfering signal, that impacts the AGC, it is fair to assume that this already defined test case also models the scenarios for co-channel coexistence for NR V2X.

# Conclusions

Proposal 1: RAN4 should analyze if additional functionality or requirements are needed to ensure that NR SL UEs follow the behavior as shown in Figure 3.2 C and/or 3.2D. Is there a need for NR SL / LTE SL co-channel co-existence requirement for 30kHz SCS NR SL UE that ensures that power of the 2nd slot is never higher than the power of the 1st slot (relative requirement)?

In addition, RAN4 should analyze if this beahvior that ensures the co-channel coexistence can impact some other minimum performance requirements. One example here can be the PUMAX,f,c, which shall be within the bounds:

PCMAX_L,f,c  –  MAX{TL,c, T(PCMAX_L,f,c)}  ≤  PUMAX,f,c  ≤  PCMAX_H,f,c  +  T(PCMAX_H,f,c)

as specified in 6.2.4 in table 6.2.4-1.

In case that clarification in PUMAX,f,c requirement is deemed necessary it could be clarified that PUMAX,f,c tolerances are increased in case of co-channel coexistence or that PUMAX.f.c needs to be measured using the 1st slot only.

Proposal 2: In case that clarification in PUMAX,f,c requirement is deemed necessary it could be clarified that PUMAX,f,c tolerances are increased in case of co-channel coexistence or that PUMAX.f.c needs to be measured usinf the 1st slot only.

Proposal 3: The power imbalance requirement with two links for NR V2X is defined using 30kHz SCS for both interfering and victim UE, but the demodulation requirement is set only for the victim. As interfering signal in this test case is basically acting as strong interfering signal, that impacts the AGC, it is fair to assume that this already defined test case also models the scenarios for co-channel coexistence for NR V2X.

# References

[1]	R4-2306633, WF on NR SL con-current operation and co-channel coexistence scenario of LTE V2X and NR V2X, LG electronics