3GPP TSG-RAN WG4 Meeting # 107		R4-2308483

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.9.3

Source: 	OPPO

Title: 	On remaining issues of RRM requirements for FR1+FR1 NR-DC

Document for:	Approval

1	Introduction

A WF on Rel-18 R18 eFeRRM was approved in RAN4#106bis meeting [1].

- Issue 1-1-2: Requirements for RACH-less SCG activation (Tsearch) GTW Agreements: Requirements for RACH-less SCG activation (Tsearch) - Define requirements for cases when - 4) Target PSCell is known and TCI is known - Do not define requirements for other cases - 1) Target PSCell is unknown but TCI is known - 2) Target PSCell is unknown and TCI is unknown - 3) Target PSCell is known but TCI is unknown For the case 4) when Target PSCell is known and TCI is known, requirements for RACH-less SCG activation for FR1+FR1 NR-DC can be: - If RLM and BFD are configured and TCI state is known, Tsearch = 0 ms if the target cell is a known FR1 PSCell. - Issue 1-1-3: Requirements for RACH based PSCell activation Follow previous agreements: - For RACH based PSCell activation, if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms. If the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch =3* Trs ms. - Issue 1-1-4：Whether to jointly define SCG activation delay in R18 WI and R17 maintenance： RAN4 update the requirements for SCG activation delay for FR1+FR1 NR-DC in R18 if further progress achieved in R17 SCG activation delay requirement.

In this contribution, we discuss the leftover issues of RRM requirements for FR1-FR1 NR-DC.

2	Discussion

In this meeting, leftover issues (e.g., SCG activation/deactivation) of RRM requirements of FR1+FR1 NR-DC are to be discussed.

- Issue 1-1-1: General aspects related to BFD and RLM Candidate options: - Option 1: - A UE which has not detected either BFD or RLF on the deactivated PSCell, need no additional measurements at PSCell activation. - A UE which has detected either BFD or RLF on the deactivated PSCell is allowed additional measurements at PSCell activation. - Option 2: If UE has detected either BFD or RLF on the deactivated PSCell for RACH-less SCG Activation, UE fall back to RACH based activation. - Issue 1-1-5: Whether to indicate the beam failure during SCG activation procedure Candidate options: - Option 1 (Apple): UE report beam failure of the PSCell during PSCell activation (after receiving the SCG activation message) - Option 2 (vivo): Deprioritize this issue due to a corner case when BFD happens during activation procedure - Option 3 (Apple, OPPO, Ericsson): Ask RAN2 for clarifications Tentative agreement for 2nd round: Need further check and come back in next meeting.

As discussed in last meeting, most companies think the condition ‘no RLF/BF occurred’ is not enough to guarantee PSCell could always be known and cell detection could not be skipped since BF/RLF always happen in lower SNR condition, e.g., -10dB and the side condition for the PSS/SSS detection is Ês/Iot ≥ -6dB. The difference between beam failure or RLF on the deactivated PSCell and known/unknown PScell has been pointed out. We suggest to avoid to introduce new principle to replace current requirements definition based on Known/unknown PScell in RAN4.

Observation 1: The condition ‘no RLF/BF occurred’ is not enough to guarantee PSCell could always be known and cell detection could not be skipped.

Proposal 1: Keep current requirements definition based on known/unknown PScell in RAN4.

In TS38.331, it was specified that UE reports beam failure of the PSCell to the network while the SCG is deactivated, and it’s unclear if UE shall also report such beam failure of the PSCell during PSCell activation (after receiving the SCG activation message).

In our understanding, UE can report beam failure of the PSCell to the network even during SCG activation procedure. If UE has detected either BFD or RLF on the deactivated PSCell for RACH-less SCG Activation, UE fall back to RACH based activation. No need to specify new requirements such as additional measurements delay.

Proposal 2: UE can report beam failure of the PSCell to the network during SCG activation procedure.

Proposal 3: UE should fall back to RACH based activation and there is no need to specify new requirements such as additional measurements delay for the case UE detected either BFD or RLF on the deactivated PSCell.

However, we also think it is a common issue for both R17 and R18 SCG activation discussion. It seems beneficial to get clear clarification on whether to indicate the beam failure during SCG activation procedure and based on that RAN4 can discuss the requirements for beam failure or unknown TCI during PSCell activation. Thus, we support to send an LS in this meeting to ask RAN4 to confirm whether it is correct understanding that UE report beam failure of the PSCell during PSCell activation.

Proposal 4: Send LS to RAN2 to confirm whether the followings are correct understanding of UE behaviour:

- UE can report beam failure of the PSCell during PSCell activation.

- UE should fall back to RACH based activation if UE detected either BFD or RLF on the deactivated PSCell.

3	Conclusion

In this paper, we propose the following observations and proposals for RRM requirements of FR1+FR1 NR-DC.

Observation 1: The condition ‘no RLF/BF occurred’ is not enough to guarantee PSCell could always be known and cell detection could not be skipped.

Proposal 1: Keep current requirements definition based on known/unknown PSCell in RAN4.

Proposal 2: UE can report beam failure of the PSCell to the network during SCG activation procedure.

Proposal 3: UE should fall back to RACH based activation and there is no need to specify new requirements such as additional measurements delay for the case UE detected either BFD or RLF on the deactivated PSCell.

Proposal 4: Send LS to RAN2 to confirm whether the followings are correct understanding of UE behaviour:

- UE can report beam failure of the PSCell during PSCell activation.

- UE should fall back to RACH based activation if UE detected either BFD or RLF on the deactivated PSCell.

4	Reference

[1] R4-2306322, WF on RRM Core requirements for FR1-FR1 NR-DC, OPPO

[2] R4-2306236, Topic summary for [106bis-e][205] NR_RRM_enh3_part2, OPPO