3GPP TSG-RAN WG4 Meeting # 107		R4-2308516

Incheon, KR, May 22 – May 26, 2023

Agenda item:		8.30.3.3

Source:		Samsung

Title:	Discussion on enhanced unified TCI framework in Rel-18 MIMO evolution for downlink and uplink

Document for:		Discussion

- Introduction

The latest WID [1] has been approved in the RAN plenary RAN#98 meeting. The eUTCI is in object 2 which is shown as below:

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

In this contribution, we continue to discuss the open issues of how to specify RRM requirements for extension of Rel-17 Unified TCI framework.

- Discussion

Firstly, for the scope of Rel-18 MIMO evolution, it is still FFS on whether to support simultaneous reception in mTRP.

Issue 3-1-4: For extension of Rel-17 unified TCI framework, whether to support simultaneous reception in mTRP? - Proposals - Option 1: Not consider simultaneous reception in mTRP in Rel-18 (Intel, MediaTek, Apple, vivo, Samsung) - Option 2: Consider simultaneous reception in mTRP in Rel-18. FFS on how to do the extension (Nokia, Xiaomi, Ericsson) - Option 3: FFS if any aspects of unified TCI extension to mTRP impacts RRM requirements with multi-RX reception in FR2. (Apple, Huawei)

The Rel-18 WI “NR_FR2_multiRX_DL” are discussed. It covered Rel-15/Rel-16 TCI framework. There is similar discussion when UE can support DL simultaneous reception. To reduce the overlap work, we propose to deprioritize the discussion when UE can support DL simultaneous reception.

Proposal 1: Deprioritize the discussion when UE can support DL simultaneous reception. wait for further conclusion with multi-RX reception in FR2.

In Rel-17 feMIMO, Active downlink TCI state switching delay for unified TCI/ Active uplink TCI state switching delay for unified TCI are specified. In addition, TRP specific BFR and L1-RSRP measurements for a cell with different PCI from serving cell are supported for mTRP scenarios.

For whether/how to define RRM requirements for extension of unified TCI, it is caputured in [2] as below:

Issue 3-1-5: How to separate the TCI state switching requirements? - Proposals - Option 1: no RRM impacts - Option 2: wait for further RAN1 progress - Option 3: - Separate for sDCI and mDCI. - For sDCI based mTRP: - FFS on whether/how to impact TCI state switching requirements. - For mDCI based mTRP: - FFS on whether/how to impact TCI state switching requirements.

We think it better to discuss for sDCI and mDCI separately.

For mDCI based mTRP scenarios, in Rel-17, the network does not configure the field in a serving cell that is configured with more than one value for the coresetPoolIndex.

In Rel-18, RAN1 latest agreements in RAN1#112bis-e meeting are captured as:

Agreement If the UE is configured with SSB-MTC-AdditionalPCI and receives TCI state activation command (MAC-CE) that activates a set of joint/DL /UL TCI state(s) specific to each coresetPoolIndex value for M-DCI based MTRP in unified TCI framework extension, the activated joint/DL/UL TCI state(s) specific to one coresetPoolIndex value is associated with the serving cell PCI and the activated joint/DL /UL TCI state(s) specific to another coresetPoolIndex value can be associated with a PCI other than the serving cell PCI . Note: How to implement above in specification is up to spec editor

For each TRP with one coresetPoolIndex, the activated joint/DL /UL TCI states can be independent. The Rel-17 requirements can be reused for each coresetPoolIndex.

Another thing should be discussed in RAN4 is the condition defined requirement applicability.

Such as: The timing difference of arrival at UE between the SSBs of serving cell and cell with different PCI is less than CP length of the corresponding SCS

In Rel-18, this limitation is not needed if UE can support RTD is larger than CP.

Proposal 2: For mDCI based mTRP, for each TRP activated joint/DL/UL TCI states, the Rel-17 requirements can be reused for each coresetPoolIndex. For UE can support RTD is larger than CP, update the condition and the applicability of the requirements. FFS on whether to reuse the same Tfirst-SSB_List in Rel-17. For active UL or joint TCI state, UE can track timing/frequency from DL-RS from different cell if UE has such capability.

For sDCI based mTRP, RAN1 latest agreements in RAN1#112bis-e meeting are captured as:

Conclusion On unified TCI framework extension for S-DCI based MTRP operation, there is no consensus to support dynamic switching between single-TRP operation and multi-TRP operation for channels/signals based on the number of TCI states mapped to the received TCI codepoint in DCI format 1_1/1_2 - FFS: How to switch between Rel-17 sTRP operation and Rel-18 mTRP operation Agreement On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - TCI state activation command (MAC-CE) should indicate that each joint/DL/UL TCI state mapped to a TCI codepoint is the first or second joint/DL/UL TCI state (detail on how to indicate above is up to RAN2 design) - The first/second indicated joint/DL/UL TCI state(s) is updated according to the corresponding first/second joint/DL/UL TCI state(s) mapped to the TCI codepoint received by the UE - If the UE receives a TCI codepoint mapped with a sub-set of {first joint TCI state, second joint TCI state} or {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}, the UE shall update the first/second indicated joint/DL/UL TCI state(s) according to the first/second joint/DL/UL TCI state(s) in the subset and keep other indicated first/second joint/DL/UL TCI state(s) that is not updated by the received TCI codepoint Agreement On unified TCI framework extension, the Rel-17 timeline for updating the indicated joint/DL/UL TCI state(s) is retained, i.e., the indicated joint/DL/UL TCI state(s) applied to the DL reception or UL transmission in each slot is updated based on the Rel-17 beam application time Conclusion On unified TCI framework extension for S-DCI based MTRP, there is no consensus in RAN1 on whether to reuse the Rel-17 RRC parameter followUnifiedTCIstate as a part of the RRC configuration that informs the UE shall apply the first one, the second one, both, or none of the indicated joint/DL TCI states to a CORESET - Above does not impact how RAN2 writes their specifications Agreement On unified TCI framework extension for S-DCI based MTRP, the UE shall apply the first indicated joint/UL TCI state to PUSCH transmission(s) scheduled/activated by DCI format 0_0 (including DG and Type2 CG) Agreement On unified TCI framework extension for S-DCI based MTRP, an RRC configuration is provided to a Type1 CG configuration to inform that the UE shall apply the first, the second, or both indicated joint/UL TCI states to the corresponding CG-PUSCH transmission - If the first or the second indicated joint/UL TCI state is applied, the UE shall apply the first or the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If both indicated joint/UL TCI states are applied: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB - FFS: SDM and SFN based PUSCH Tx schemes

For sDCI based mTRP, one or two TCI states can be activated by MAC-CE. In addition, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception.

For MAC-CE based TCI state switching delay and DCI based TCI state switching delay for beam application time, the similar Rel-17 requirement can be reused.

For PDCCH, In Rel-17 unified TCI framework, it can be guaranteed that PDCCH and respective PDSCH follow the common beam for DL reception. In 38.133, no special requirement for PDSCH and PDCCH reception. For Rel-18, wait for RAN1 further progress.

In Rel-17 active uplink TCI state switching delay for unified TCI, PUCCH and PUSCH are used the same requirements. For Rel-18, whether to use the same requirements as Rel-17, we think more RAN1 progress is needed.

Proposal 3: For sDCI based mTRP, for PDSCH, for MAC-CE based TCI state switching delay and DCI based TCI state switching delay for beam application time, the similar Rel-17 requirement can be reused, the details of known condition and L1-RSRP/timing should be revised. For PDCCH, wait for further RAN1 progress. For active uplink TCI state switching delay for unified TCI, wait for further RAN1 progress.

Issue 3-1-9: Unified TCI extension if UE can support sTxMP? - Proposals - Option 1: (Apple) - R17 MAC CE TCI switch requirements for UL TCI can be applicable to R18 extension for mTRP and STxMP. - Option 2: (Intel, vivo, MTK) - For single-panel based scheme, Rel-17 UL TCI state list update delay can apply for MAC CE based TCI states activation in both sDCI and mDCI scenario. - Suggest to discuss multi-TX panel related requirement in future release. - Option 3: (Samsung, Huawei, Nokia, Ericsson) - FFS

Although in RAN1’s agreements:

Agreement On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17 - Consider, if STxMP is supported, Rel-18 MTRP scheme(s) with STxMP

Beam pair and enhanced group-based beam L1-RSRP for STxMP are discussing in RAN1 now with no consensus. It is not so clear for the RRM impacts of extension of unified TCI framework if UE can support STxMP.

Proposal 4: For unified TCI extension if UE can support STxMP, wait for further RAN1 progress.

- Conclusion

In this contribution, we provide our consideration of RRM requirements for extension of Rel-17 Unified TCI framework and our proposals are:

Proposal 1: Deprioritize the discussion when UE can support DL simultaneous reception. wait for further conclusion with multi-RX reception in FR2.

Proposal 2: For mDCI based mTRP, for each TRP activated joint/DL/UL TCI states, the Rel-17 requirements can be reused for each coresetPoolIndex. For UE can support RTD is larger than CP, update the condition and the applicability of the requirements. FFS on whether to reuse the same Tfirst-SSB_List in Rel-17. For active UL or joint TCI state, UE can track timing/frequency from DL-RS from different cell if UE has such capability.

Proposal 3: For sDCI based mTRP, for PDSCH, for MAC-CE based TCI state switching delay and DCI based TCI state switching delay for beam application time, the similar Rel-17 requirement can be reused, the details of known condition and L1-RSRP/timing should be revised. For PDCCH, wait for further RAN1 progress. For active uplink TCI state switching delay for unified TCI, wait for further RAN1 progress.

Proposal 4: For unified TCI extension if UE can support STxMP, wait for further RAN1 progress.

- Reference

[1] RP-223276, WID Update: MIMO Evolution for Downlink and Uplink, Samsung

[2] R4-2306362, WF on R18 NR MIMO RRM requirements, Samsung