3GPP TSG-RAN WG1 Meeting #107	R4-2308525

Seoul, Korea, May 22nd – 26th 2023

Agenda Item:	8.29.6

Source:	Ericsson

Title:	NCR-MT demodulation requirements

Document for:	Discussion

# 1	Introduction

During RAN4#106bis, a WF was agreed for NCR demodulation requirements. In general, the NCR demodulation requirements should follow IAB-MT or/and UE requirements according to the agreed WF. In this contribution, we review where simulations are needed or may be needed, along with some of the remaining issues.

# 2	Discussion

## 2.1	PDSCH

### 2.1.1	FR1

For FR1, PDSCH requirements are needed for the 15kHz SCS and the 30kHz SCS. For the 30kHz SCS, 4RX requirements can be taken from a subset of the IAB requirements. Since only rank 1 and one channel type is needed, the following requirements are needed for the non-FFS agreed requirement set:

- PDSCH 15kHz SCS, TDLA30-10, 10MHz, 2RX, 30% throughput -> Not available (UE spec uses a different channel)

- PDSCH 15kHz SCS, TDLA30-10, 10MHz, 2RX, 70% throughput -> Not available (UE spec uses 256QAM or a different channel)

- PDSCH 15kHz SCS, TDLA30-10, 10MHz, 4RX, 30% throughput -> Not available (UE spec uses a different channel)

- PDSCH 15kHz SCS, TDLA30-10, 10MHz, 4RX, 70% throughput -> Not available (UE spec uses 256QAM or a different channel)

- PDSCH 30kHz SCS, TDLA30-10, 40MHz, 2RX, 30% throughput -> Not available (IAB spec requirement is for 4RX, UE spec different channel)

- PDSCH 30kHz SCS, TDLA30-10, 40MHz, 2RX, 70% throughput -> Available in UE spec with QPSK modulation (Defined with 256QAM in IAB spec)

- PDSCH 30kHz SCS, TDLA30-10, 40MHz, 4RX, 30% throughput -> Available from IAB specification

- PDSCH 30kHz SCS, TDLA30-10, 40MHz, 4RX, 70% throughput -> Available from the UE specification for QPSK

From the above list, it can be seen that new simulations are needed for 5 cases. Furthermore, if QPSK is not acceptable for the requirement with 70% throughput then 2 further cases are needed.

Apart from the agreed requirements, two further FFS were proposed:

- 5MHz bandwidth

- BLER based PDSCH requirements, considering that the NCR-MT carries control information.

Taking into account that in reality, although the key parameters (FRCs, configurations etc.) can be re-used from the IAB or/and the UE specifications, new simulations are needed for most cases, it may be considered whether the existing agreements be re-considered. For example, all cases could be re-simulated with 1% BLER only, or all cases with 5MHz only. If all combinations would be included (existing bandwidths, 5MHz bandwidth, existing throughput points and 1% BLER) and if QPSK would not be acceptable for the 70% throughput then 23 new simulations would be required. If the red lines in the list above would be adopted, 5 new simulations would be required. If 1% BLER would be adopted with a single bandwidth option then 4 new simulations would be needed. In all cases, parameters and FRCs can be used from existing specifications; it is just a case of running simulations with a different channel model and/or operating point; there is not a need to develop new parameters.

RAN4 decide whether QPSK is sufficient for the 70% throughput requirements

Decisions on whether QPSK is sufficient, whether to support 5MHz and the PDSCH operation point of 1% will have a significant impact on the number of needed simulations.

Regarding the 1% BLER based requirements, it is correct that 1% BLER is likely to be more relevant when considering that the PDSCH is likely to carry control information. However, there exists also the risk that alignment of the simulations between companies may become more difficult for 1% BLER.

1% BLER is a correct point for control operation, but there may be a risk of lack of convergence between different companies.

### 2.1.1	FR2

For FR2, requirements are needed at least for 120kHz SCS. For the 60kHz SCS, there are no requirements available for rank1 in either the UE or the IAB specifications. This means that there are no FRCs nor parameters defined. With this in mind, we propose that requirements are not developed for the 60kHz SCS.

In the previous meeting, TDLA30-10 was agreed as the channel. However, we believe that this was applicable for FR1. For FR2, TDLA30-75 was adopted for FR2, and we propose that the same is needed for IAB-MT.

Use TDLA30-75 as the channel model for FR2.

If proposal 2 is adopted, then the requirements needed as agreed previously are as follows:

- PDSCH 120kHz SCS, TDLA30-75, 100MHz, 2RX, 30% throughput

- PDSCH 120kHz SCS, TDLA30-75, 100MHz, 2RX, 70% throughput -> Not available (IAB spec uses 64QAM, UE spec uses a different channel model)

It is also proposed to consider 1% BLER as an operating point for FR2 PDSCH. Again, there may be an issue with simulation alignment. Two simulations would be needed if 1% were to be included.

## 2.2	PDCCH

### 2.2.1	FR1

The coreset duration for FDD FR1 was left open at RAN4#106bis. By adopting option 1 from the WF, the number of new simulations of PDCCH can be minimized because existing UE requirements can be used:

- Option 1:

  - 1TX: coreset duration 2

  - 2TX: coreset duration 1

Adopt the following for FR1 PDCCH:

- 1TX: coreset duration 2

- 2TX: coreset duration 1

For TDD, coreset duration 1 should be used in all cases to minimize the number of simulations.

Taking into account the agreements at RAN4#106bis, the following requirements are needed for FR1 (coreset duration 1 in all cases):

- PUCCH, 15kHz SCS, 1TX, 10MHz, Aggregation level 2, coreset duration 2, 2RX, TDLA30-10 -> Available in 38.101-4

- PUCCH, 15kHz SCS, 1TX, 10MHz, Aggregation level 4, coreset duration 2, 2RX, TDLA30-10 -> Available in 38.101-4

- PUCCH, 15kHz SCS, 2TX, 10MHz, Aggregation level 8, coreset duration 1, 2RX, TDLA30-10 -> Available in 38.101-4

- PUCCH, 15kHz SCS, 1TX, 10MHz, Aggregation level 2, coreset duration 2, 4RX, TDLA30-10 -> Available in 38.101-4

- PUCCH, 15kHz SCS, 1TX, 10MHz, Aggregation level 4, coreset duration 2, 4RX, TDLA30-10 -> Available in 38.101-4

- PUCCH, 15kHz SCS, 2TX, 10MHz, Aggregation level 8, coreset duration 1, 4RX, TDLA30-10 -> Available in 38.101-4

- PUCCH, 30kHz SCS, 1TX, 40MHz, Aggregation level 2, coreset duration 1, 2RX, TDLA30-10 -> Available in 38.101-4

- PUCCH, 30kHz SCS, 1TX, 40MHz, Aggregation level 4, coreset duration 1, 2RX, TDLA30-10 -> Different channel in 38.101-4

- PUCCH, 30kHz SCS, 2TX, 40MHz, Aggregation level 8, coreset duration 1, 2RX, TDLA30-10 -> Different channel in 38.101-4

- PUCCH, 30kHz SCS, 1TX, 40MHz, Aggregation level 2, coreset duration 1, 4RX, TDLA30-10 -> Available in 38.174

- PUCCH, 30kHz SCS, 1TX, 40MHz, Aggregation level 4, coreset duration 1, 4RX, TDLA30-10 -> Available in 38.174

- PUCCH, 30kHz SCS, 2TX, 40MHz, Aggregation level 8, coreset duration 1, 4RX, TDLA30-10 -> Available in 38.174

Two new simulations are required to achieve the requirements as agreed. Existing parameters can be used, but a new channel is required.

Two further proposals are FFS after RAN4#106bis; 5MHz bandwidth and a lower BLER to reflect that the repeater is a two-stage link and so the error probability should be lower.

Each of the additional proposals would require new simulations of all 12 cases. Of course, the proposals could be combined to reduce the number of simulations.

Adding requirements for 5MHz and/or lower than 1% BLER target would add 12-24 simulations.

### 2.2.2	FR2

If requirements are defined only for 120kHz SCS, then the existing requirements from 38.174 are sufficient for FR2:

- PUCCH, 120kHz SCS, 1TX, 100MHz, Aggregation level 2, coreset duration 1, 2RX, TDLA30-75 -> Available in 38.174

- PUCCH, 120kHz SCS, 1TX, 100MHz, Aggregation level 4, coreset duration 1, 2RX, TDLA30-75 -> Available in 38.174

- PUCCH, 120kHz SCS, 2TX, 100MHz, Aggregation level 8, coreset duration 1, 2RX, TDLA30-75 -> Available in 38.174

Of course, if the requirement would be defined at a different error rate then the 3 simulation cases would need re-running.

## CQI

The CQI reporting requirements were agreed to be defined in AWGN only. Although for FR1 the requirements are defined for 40MHz, 4RX and 30kHz SCS, there is no need for any new simulation in order to define requirements for other bandwidth / RX / SCS. Similarly, for FR2 there is no need for new simulations.

No new simulations needed for CQI.

## PMI

38.174 defines PMI requirements for TDD, 4RX, 40MHz. For the other SCS/RX/bandwidths, the requirements in 38.101-4 can be used. If a 5MHz bandwidth were to be defined, then in principle a simulation may be needed to check that the PMI requirement remains valid. However, since it is defined in a channel with low delay spread, this can be expected to be the case.

In theory, for PMI, if 5MHz is adopted then the requirement should be checked. In practice, for the low delay spread channel it is unlikely the requirement will differ.

## RI

It was agreed at RAN4#106bis-e not to define requirements for RI

# Conclusion

In the previous sections we made the following observations:

Observation 1	Decisions on whether QPSK is sufficient, whether to support 5MHz and the PDSCH operation point of 1% will have a significant impact on the number of needed simulations.

Observation 2	1% BLER is a correct point for control operation, but there may be a risk of lack of convergence between different companies.

Observation 3	Adding requirements for 5MHz and/or lower than 1% BLER target would add 12-24 simulations.

Observation 4	No new simulations needed for CQI.

Observation 5	In theory, for PMI, if 5MHz is adopted then the requirement should be checked. In practice, for the low delay spread channel it is unlikely the requirement will differ.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 decide whether QPSK is sufficient for the 70% throughput requirements

Proposal 2	Use TDLA30-75 as the channel model for FR2.

Proposal 3	Adopt the following for FR1 PDCCH:

1TX: coreset duration 2

2TX: coreset duration 1