3GPP TSG-RAN WG4 Meeting # 107                               R4-2308573

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on principle of MSD due to two UL bands cross band isolation for CA_n5-n28

Agenda Item:	7.30.2.2

Document for:	Approval

Introduction

In last meeting, we have the way forward [1] on MSD for 2UL CA_n5-n28. RAN4 is going to discuss some principles for the MSD due to two UL cross band isolation in May meeting. In this paper, we’d like to share our views on the principles.

Discussion

As shown in figure 1, when both band n5 UL and n28 UL are configured for CA_n5-n28, band n28 DL receiver will be impacted by the leakage interference from band n5 UL and n28 UL.

Figure 1 The relationship for band n28 DL impacted by both band n5 UL and n28 UL

Generally, when 30MHz channel bandwidth is configured for band n28, the REFSENS degradation for band n28 DL has been analysed referring to TR 38.888 [3]. 25 RB (15kHz SCS) was defined for n28 UL configuration. In addition, it’s assumed that 50 dB Transmit port to receiver port duplexer rejection in n28 RX band as a worst case according to contribution [4], and -24 dBm TX noise level in RX band was observed. Finally, -78.5dBm REFSENS was specified for band n28 30MHz considering the interference from band n28 UL.

Observation 1: Currently, -78.5dBm REFSENS was specified for band n28 30MHz considering the interference from band n28 UL with 25 RB (15kHz SCS) UL configuration. And the minimum duplexer performance and PA linearity performance for band n28 have been guaranteed by band n28 30MHz REFSENS.

For CA_n5-n28 with 2 antennas implementation, referring to contribution R4-2202036, the n28 MSD evaluation for CA_n5-n28 is shown as below.

Figure 2 the n28 MSD evaluation for CA_n5-n28

40 ~ 45 dB isolation between band n5 Tx and band n28 Rx were assumed. Measured Tx noise in Rx band in channel level were assumed as -35.7 ~ -34.2 dBm. Finally, the MSD 17.5 due to cross band isolation from n5 Tx interference can be agreeable for companies in last meeting below.

Table 1 MSD test point for CA_n5-n28 due to two UL cross-band interference

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 788 | 30 | 3.1 | ACLR2 from n5 UL band ACLR1+ACLR2 from n28 UL band |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

Observation 2: Currently, 17.5dB was specified for band n28 5MHz MSD due to cross band isolation from n5 Tx interference. And the minimum filter performance and PA linearity performance for band n5 have been guaranteed by the one UL band cross band isolation MSD definition.

Based on these two kinds of requirements (band n28 30MHz REFSENS and MSD due to from n5 Tx interference for CA_n5-n28), generally the minimum RF components performances have been guaranteed. From that perspective, the new MSD due to cross band interference from two UL bands for CA_n5-n28 don’t further restrict RF components performances.

Observation 3: as the minimum RF components performances have been guaranteed by two kinds of requirements (band n28 30MHz REFSENS and MSD due to from n5 Tx interference for CA_n5-n28), the new MSD due to cross band interference from two UL bands for CA_n5-n28 don’t further restrict RF components performances.

Although working group has agreed to introduce the MSD due to cross band interference from two UL bands for CA_n5-n28, not sure whether it is meaningless to extend it to other band combinations since the RF components performances have been guaranteed by MSD due to cross band isolation from one UL band or REFSENS. And additional UE test burden can be observed.

Proposal 1: since the RF components performances have been guaranteed by MSD due to cross band isolation from one UL band or REFSENS in addition to potential test burden concerns, whether to further specify such kind MSD for the other band combination should be discussed case by case.

If RAN4 identify some new restrictions/ meaning on RF components performance by specifying MSD due to cross band isolation from two UL bands, RAN4 can consider to extend this kind of MSD for the other band combinations considering the following principles.

1) One UL band has a first adjacent channel leakage interference on the Rx band, and another band has a first or/and second adjacent channel leakage interference on the same Rx band.

2) There is no MSD due to two UL IMD interference.

3) Only for sub-1GHz band combinations

Proposal 2: If RAN4 identify some new restrictions/ meaning on RF components performance by specifying MSD due to cross band isolation from two UL bands, RAN4 can consider to extend this kind of MSD for the other band combinations considering the following principles.

1) One UL band has a first adjacent channel leakage interference on the Rx band, and another band has a first or/and second adjacent channel leakage interference on the same Rx band.

2) There is no MSD due to two UL IMD interference.

3) Only consider new sub-1GHz band combinations to avoid NBC issue.

- Summary

Observation 1: Currently, -78.5dBm REFSENS was specified for band n28 30MHz considering the interference from band n28 UL with 25 RB (15kHz SCS) UL configuration. And the minimum duplexer performance and PA linearity performance for band n28 have been guaranteed by band n28 30MHz REFSENS.

Observation 2: Currently, 17.5dB was specified for band n28 5MHz MSD due to cross band isolation from n5 Tx interference. And the minimum filter performance and PA linearity performance for band n5 have been guaranteed by the one UL band cross band isolation MSD definition.

Observation 3: as the minimum RF components performances have been guaranteed by two kinds of requirements (band n28 30MHz REFSENS and MSD due to from n5 Tx interference for CA_n5-n28), the new MSD due to cross band interference from two UL bands for CA_n5-n28 don’t further restrict RF components performances.

Proposal 1: since the RF components performances have been guaranteed by MSD due to cross band isolation from one UL band or REFSENS in addition to potential test burden concerns, whether to further specify such kind MSD for the other band combination should be discussed case by case.

Proposal 2: If RAN4 identify some new restrictions/ meaning on RF components performance by specifying MSD due to cross band isolation from two UL bands, RAN4 can consider to extend this kind of MSD for the other band combinations considering the following principles.

1) One UL band has a first adjacent channel leakage interference on the Rx band, and another band has a first or/and second adjacent channel leakage interference on the same Rx band.

2) There is no MSD due to two UL IMD interference.

3) Only consider new sub-1GHz band combinations to avoid NBC issue.

- References

[1] R4-2306468, WF on CA_n5-n28 and CA_n8-n20-n28, Huawei, HiSilicon

[2] R4-2202036, Measurements for CA_n5-n28 MSD, Skyworks Solutions, Inc.