3GPP TSG-RAN WG4 Meeting # 107                               R4-2308576

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on UE RF requirements

Agenda Item:	9.3.3

Document for:	Approval

Introduction

In previous meeting, WF [1] on UE RF for 5G Broadcast was approved with the following options. One of the tough topics is related to how to specify ACS requirements. The following way forward was approved.

Option 1 (Nokia): -25 dB with frequency offset at 6, 7, and 8 MHz

Option 2 (Qualcomm): -33 dB with frequency offset at 10 MHz (same as the existing 10 MHz spec)

Option 3 (SWR):  -33 dB with frequency offset at 6, 7, and 8 MHz

Way forward:  Companies are invited to study options 2 and 3.

In this paper, we’d like to provide our views about UE ACS requirements for 6MHz/7MHz/8MHz. Additionally, we’d like to provide our views on REFSENS, maximum input level and how to implement channel bandwidth.

Discussion on REFSENS

In previous meeting, we have the following way forward [2, 3] for REFSENS. As companies may want to reduce the test efforts, it seems that RAN4 need to trade off these channel bandwidth 6, 7, 8 and 10 MHz.

For discussion - The values of -99.2, -98.5, and -97.9 dBm are agreed for 6, 7, and 8 MHz PMCH channel assuming 10 MHz UE receive channel filter. - FFS on whether all bandwidths are needed to be tested. Way forward: At least the following points are to be considered in deriving the reference sensitivity - Which bandwidth shall be used for noise integration? Possibilities include 6, 7, 8, and 10 MHz. - What signal shall be defined for refsens? Possibilities include a newly defined 6, 7, and 8 MHz PMCH or an existing 10 MHz PDSCH.

REFSENS: - The 470 – 698 MHz band and/or the 612 – 652 MHz band is specified with Option 1. 6, 7, and 8 MHz PMCH for reference sensitivity Option 2. 10 MHz PDSCH for reference sensitivity Way forward: FFS these options

In last meeting, we provide a compromise to make progress, but it seems not acceptable for companies. As there is no objective to specify 10MHz requirements, what we can go is only option 1.

Proposal 1: As there is no objective to specify 10MHz requirements, only option 1 can be considered.

Option 1: The 470 – 698 MHz band and/or the 612 – 652 MHz band is specified with 6, 7, and 8 MHz PMCH for reference sensitivity

Discussion on maximum input level

In last meeting, companies provides two options for specifying the maximum input level in contribution [4]. The in-band blocker power was specified to be -15 dBm in Band 71 to accommodate up to 1 MW broadcast transmission and -22 dBm in Band n105 to accommodate up to 200 kW broadcast transmission. Thus, two options are observed for maximum input level.

Option 1: -15dBm maximum input level assuming 1MW broadcast transmission.

Option 2: -22dBm maximum input level assuming 200kW broadcast transmission.

Even if the broadcast transmission power was assumed in that paper, the path loss model is unclear. After digging some papers, I found that the measurement data [5] was used to derive the -15dBm value recorded in TR 36.755 [6] as below.

The case 3 blocker was defined to be -30 dBm to account for the Tx power of the TV broadcaster as well as the propagation loss to the UE receiver. The wanted signal power level is set at 6 to 9 dB above reference sensitivity and the blocker is defined as a 5 MHz LTE signal. The Tx power of the TV broadcaster for MediaFLO is 50 kW and the separation from edge to edge is 12 MHz and 6 MHz between Band 17 downlink and 700 MHz blocks D and E, respectively. The propagation loss was predicted based on models and drive tests including data provided in [R4-100430]. It was concluded that with high probability, the Rx power level at the UE antenna should be -30 dBm or lower. Thus, the Rx blocker level was defined to be -30 dBm. For the 600 MHz band, a similar situation exists with TV broadcast in adjacent channels. However, one significant difference is that power of the TV broadcast at 600 MHz is up to 1 MW. Assuming a similar propagation loss in 600 MHz compared to 700 MHz, the power level reaching the UE receiver is 13 dB higher for 600 MHz band compared to Band 17. However, considering that the propagation is slightly better at 600 MHz than 700 MHz, the maximum Rx power with high probability is 15 dBm higher, resulting in an Rx blocker level of -15 dBm. In FCC R&O 14-50, the television signal strength received by the UE is estimated at -23 dBm. This derivation is based on a received power level of -15 dBm coupled with a UE Rx antenna gain of -8 dBi. However, the 3GPP specifications also account for the case where the antenna gain may be as high as 0 dBi for different types of devices, for example, fixed or nomadic. In TS 36.101, it is stated that a reference antenna with a gain of 0 dBi is assumed for each antenna port. Taking this into account, the received power level is then also -15 dBm, consistent with the proposal here.

Not sure whether broadcast operators can provide some measurement datas to derive the maximum input level for UE. If not, I think what RAN4 can do is just to further trade off the values from -15dBm to -22dBm based on the assumption on broadcast transmission power.

Proposal 2: it’s recommended for companies to provide the some measurement datas to derive the maximum input level for UE. Otherwise, RAN4 can just further trade off the values from -15dBm to -22dBm based on the assumption on broadcast transmission power.

Discussion on UE ACS requirements

UE ACS is a major requirement to guarantee the system coexistence between two adjacent channels. When RAN4 specify this UE ACS requirement, the major factor which should be considered as the first priority is system performance instead of UE filter implementation. The logic is how we can design a UE to guarantee the adjacent channel coexistence not the other way around.

Observation 1: UE ACS is a major requirement to guarantee the system coexistence between two adjacent channels. RAN4 can’t specify this requirement only based on UE filter implementation.

Table 1 the ACIR result of different BS ACLR and UE ACS

| BS ACLR | UE ACS | ACIR BS-UE |
| --- | --- | --- |
| 45 | 33 | 32.7342762 |
| 51 | 33 | 32.9317087 |
| 55 | 33 | 32.972684 |
| 61 | 33 | 32.9931223 |
| 42 | 33 | 32.4850306 |
| BS ACLR | UE ACS | ACIR BS-UE |
| 45 | 25 | 24.9567863 |
| 51 | 25 | 24.9891047 |
| 55 | 25 | 24.9956592 |
| 61 | 25 | 24.9989092 |
| 42 | 25 | 24.9142 |
| BS ACLR | UE ACS | ACIR BS-UE |
| 45 | 30 | 29.8647908 |
| 51 | 30 | 29.9656391 |
| 55 | 30 | 29.9862881 |
| 61 | 30 | 29.9965516 |
| 42 | 30 | 29.7342762 |

In table 1, we calculated the ACIR results based on different BS ACLR and UE ACS requirement. It can be observed that UE ACS requirement is the bottleneck of system DL ACIR coexistence performance. For 25dB UE ACS, even if BS ACLR is improved as 61dB level, the DL ACIR can’t be higher than 25dB.

Observation 2: based on the analysis above, UE ACS requirement will have a big impact on system DL ACIR coexistence performance. The system DL ACIR coexistence performance for 25dB UE ACS is much worse than 33dB UE ACS, not to mention 16dB UE ACS.

In last meeting, operators have confirmed that only the following deployment should be considered.

Figure 2 the coordinated network deployment

For coordinated network deployment, option 3 is enough to guarantees the coordinated two channels to meet the same DL system coexistence performance as terrestrial network.

Option 3 (SWR):  -33 dB with frequency offset at 6, 7, and 8 MHz

Proposal 3: Option 3 (-33 dB with frequency offset at 6, 7, and 8 MHz) can be specified for the coordinated network deployment.

Discussion on Specification of Channel bandwidth

In last meeting, we have the following way forward.

Channel bandwidth of 10 MHz for the UE has been assumed, but there are differing views on whether this detail needs to be included in the specification. - Option 1: Include 10 MHz channel bandwidth and 6, 7, and 8 MHz 5G terrestrial broadcast bandwidth (a la R4-2305824) - Option 2: Do not include 10 MHz channel bandwidth as this is an implementation detail. The specification shall only mention 6, 7, and 8 MHz 5G terrestrial broadcast bandwidths - Option 3: Include 10 MHz channel bandwidth, but instead of referring to 6, 7, and 8 MHz 5G terrestrial broadcast bandwidth, refer to PMCH-bandwidth of 30, 35, and 40 RB’s as specified in 36.331. Way forward: FFS these options

However, if companies take a look at current revised WID [7], there is no such objective to specify 10MHz channel bandwidth.

Based on a part 1 REL-17 RAN1/2/3 WI (LTE_terr_bcast_bands_part1) this part 2 REL-18 RAN4 WI will specify bands and requirements in a REL-independent way starting from REL-17: - For MBMS-dedicated cells: - Specify band(s), within the portion of UHF spectrum allocated to broadcast systems (~470 - ~694/698 MHz, depending on the region), that support the channelization available to broadcasting operators [RAN4]. - RAN4 to define the exact band plan suitable for broadcasting operators’ spectrum allocation - RAN4 to define REFSENS for this band(s) - For BS the requirements for signal quality, occupied bandwidth, ACLR, unwanted emissions shall be applied as provided by the corresponding regulatory bodies in the different regions for 6/7/8 MHz, as applicable. The specifications shall contain corresponding references [RAN4]. - Specify UE requirements for 6/7/8 MHz, including the following [RAN4]: - Define RX requirements (e.g. maximum input level, blocking, spurious response, wideband intermodulation, ACS) to accommodate 6/7/8 MHz PMCH. - Reuse existing requirements for 10 MHz as much as possible. NOTE: New (dedicated) channel filters (e.g. 6/7/8 MHz filters) are not considered for the UE. NOTE 1: This work item shall not affect any functionality of unicast LTE, it targets only MBMS- dedicated cells. NOTE 2: RAN4 shall assume that coexistence among different systems in the portion of the UHF band allocated to broadcast (~470 - ~694/698 MHz) is ensured through coordination, in line with regional and national regulation.

Proposal 4: Option 2 is aligned with the objective of current WID.

-	Option 2: Do not include 10 MHz channel bandwidth as this is an implementation detail.  The specification shall only mention 6, 7, and 8 MHz 5G terrestrial broadcast bandwidths

Summary

Proposal 1: As there is no objective to specify 10MHz requirements, only option 1 can be considered.

- Option 1: The 470 – 698 MHz band and/or the 612 – 652 MHz band is specified with 6, 7, and 8 MHz PMCH for reference sensitivity

Proposal 2: it’s recommended for companies to provide the some measurement datas to derive the maximum input level for UE. Otherwise, RAN4 can just further trade off the values from -15dBm to -22dBm based on the assumption on broadcast transmission power.

Observation 1: UE ACS is a major requirement to guarantee the system coexistence between two adjacent channels. RAN4 can’t specify this requirement only based on UE filter implementation.

Observation 2: based on the analysis above, UE ACS requirement will have a big impact on system DL ACIR coexistence performance. The system DL ACIR coexistence performance for 25dB UE ACS is much worse than 33dB UE ACS, not to mention 16dB UE ACS.

Proposal 3: Option 3 (-33 dB with frequency offset at 6, 7, and 8 MHz) can be specified for the coordinated network deployment.

Proposal 4: Option 2 is aligned with the objective of current WID.

-	Option 2: Do not include 10 MHz channel bandwidth as this is an implementation detail.  The specification shall only mention 6, 7, and 8 MHz 5G terrestrial broadcast bandwidths

- References

[1] R4-2220501, WF on UE RF for 5G broadcast, Qualcomm

[2] R4-2303489, WF on RF requirements for 5G Broadcast, Qualcomm Incorporated

[3] R4-2306566, WF on UE RF requirements for 5G broadcast, Qualcomm Incorporated

[4] R4-2302705, UE requirements for 5G broadcast, Qualcomm Incorporated

[5] R4-100430, TV transmission power at UE antenna port, Nokia

[6] TR 36.755, US 600 MHz Band for LTE

[7] RP-220518, Revised WID: New bands and bandwidth allocation for LTE based 5G terrestrial broadcast - part 2, EBU, Qualcomm