3GPP TSG-RAN WG4 Meeting # 107                               R4-2308583

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on RF architecture and potential MSD issues for CA_n26-n28

Agenda Item:	8.4.2.3

Document for:	Approval

Introduction

In RAN4#106bis meeting, we have the following WF [1] on RF architecture and critical issues for CA_n26-n28. During the discussion, there are two cases whether to restrict the frequency range of band n28.

Case 1: lower 30MHz of n28 (UL: 703~733MHz, DL: 758~788MHz) is supported. Case 2: entire n28 frequency range (UL: 703~748MHz, DL: 758~803MHz) is supported.

In this paper, we’d like to further discuss the RF architecture and evaluate the potential MSD issues for CA_n26-n28.

Discussion on RF architecture

In the way forward [1], three potential UE RF architectures for CA_n26A-n28A case 1 and case 2 are proposed to be considered as shown below.

Architecture 1 and 2: Although two low band antennas can be implemented, it’s very challenge to implement Quadplexer/Triplexer with two 11/10MHz gap for n28B DL/n26 UL/n26 DL.

Architecture 3: Generally, three antenna solution can leverage the existing duplexer to achieve this band combination without implementing complicated Quadplexer and Triplexer. However, UE may have to overcome some difficulties when three low band antennas are implemented in smart phone.

Proposal 1: three-antennas solution can be assumed as baseline for real implementation or MSD evaluation considering the difficulties of Quadplexer/Triplexer implementation.

| RF architecture | Case 1 | Case 2 |
| --- | --- | --- |
| Architecture 1: Quadplexer solution for CA_n26-n28 with two antennas |  |  |
| Architecture 2: Triplexer solution for CA_n26-n28 with two antennas |  |  |
| Architecture 3: Three-antenna solution for CA_n26-n28 |  |  |

Discussion on Delta Tib and Rib

In WF [2] for CA_n5-n28, we have an agreement on Delta Tib and Rib. The values for CA_n5-n28 can be reused for CA_n26-n28.

Proposal 2: The Delta Tib and Rib values for CA_n5-n28 can be reused for CA_n26-n28.

Table 1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n26A-n28A | n26 | 0.7 |
|  | n28 | 0.7 |

Table 2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n26A-n28A | n26 | 0.2 |
|  | n28 | 0.2 |

Discussion on MSD due to cross band isolation

Based on the WF, RAN4 identified the MSD issue due to cross band isolation for CA_n26-n28 two cases below.

| MSD category | Case 1 | Case 2 |
| --- | --- | --- |
| Cross band isolation issue | The 2nd adjacent channel leakage may fall into n28A DL frequency range (774~788) when 20MHz UL CBW is assumed for band n26. | The 1st adjacent channel leakage may fall into n28 DL frequency range (796~803) when 20MHz UL CBW is assumed for band n26. |

Since the frequency gaps between n28 DL and n26 UL are different between two cases, filter performance and PA leakage interference in 1st and 2nd adjacent channel are different. Thus, we provide the following evaluation for these two cases assuming three-antennas architecture.

Case 1 (n28A filter is assumed)

For this case, since n28A Rx filter can provide enough rejection at n26 UL frequency range, the main issue is the performance of band n26 Tx filter at 758~788MHz. We assume two values for the evaluation, 35dB and 45dB for band n26 Tx filter rejection at 758~788MHz.

|  | parameters | n28 main path | n28 diversity path | parameters | n28 main path | n28 diversity path |
| --- | --- | --- | --- | --- | --- | --- |
| transmit power for n26, dBm |  | 23 | 23 |  | 23 | 23 |
| Tx band n26 BW(MHz, Lcrb) | 20 |  |  | 20 |  |  |
| RFFE loss, dB | 4 |  |  | 4 |  |  |
| Diplexer isolation at n26 uplink freq，dB | 10 |  |  | 10 |  |  |
| antenna isolation, dB | 10 |  |  | 10 |  |  |
| n28 receival singal at ANT port, dBm |  | 13 | 13 |  | 13 | 13 |
| n28 filter rejection at 814~849MHz, dB | 33 |  |  | 33 |  |  |
| signal After n28 filter, dBm |  | -24 | -24 |  | -24 | -24 |
| Typical receiver IIP2, dB | 50 |  |  | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -98 | -98 |  | -98 | -98 |
| front-end loss | 4 |  |  | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -94 | -94 |  | -94 | -94 |
| noise figure dB | 10.5 |  |  | 10.5 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -163.5 |  |  | -163.5 |  |  |
| Rx band n28 BW(MHz, NRB) | 4.5 |  |  | 4.5 |  |  |
| Thermal noise, dBm |  | -96.97 | -96.97 |  | -96.97 | -96.97 |
| ACLR2, dB | 43 |  |  | 43 |  |  |
| n26 PA leakage PSD at PA output port at 2nd adjacent channel, dBm/MHz |  | -33.01 |  |  | -33.01 |  |
| n26 PA noise PSD at PA output port, dBm/Hz | -125 |  |  | -125 |  |  |
| n26 PA noise PSD at PA output port, dBm/RxBW |  | -26.48 |  |  | -26.48 |  |
| n26 Tx filter rejection at 758~788MHz, dB | 35 |  |  | 45 |  |  |
| n26 PA noise power at Rx n28 ant port at 758~788MHz, dBm |  | -71.48 | -71.48 |  | -81.48 | -81.48 |
| Total noise level at ANT port |  | -71.44 | -71.44 |  | -81.13 | -81.13 |
| SNR requirement for QPSK | -1 |  |  | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -72.44 | -72.44 |  | -82.13 | -82.13 |
| Implementation Margin, dB | 2.5 |  |  | 2.5 |  |  |
| combined REFSENS(5MHz BW), dBm | -72.95 |  |  | -82.64 |  |  |
| MSD | 25.55 |  |  | 15.86 |  |  |

Case 2 (n28B filter is assumed)

For this case, since some legacy n28B Rx filter can provide enough rejection at n26 UL frequency range (e.g. 12dB), TX IM2 will have serious impacts on the REFSENS. In addition, we also provide a set of better rejection performance (Set 2) for comparison.

|  | Set 1 | Set 2 |
| --- | --- | --- |
| n28B Rx filter rejection at 814~849MHz | 12 dB | 33 dB |
| n26 Tx filter rejection at 773~803MHz | 35 dB | 45 dB |

|  | parameters | n28 main path | n28 diversity path | parameters | n28 main path | n28 diversity path |
| --- | --- | --- | --- | --- | --- | --- |
| transmit power for n26, dBm |  | 23 | 23 |  | 23 | 23 |
| Tx band n26 BW(MHz, Lcrb) | 20 |  |  | 20 |  |  |
| RFFE loss, dB | 4 |  |  | 4 |  |  |
| Diplexer isolation at n26 uplink freq，dB | 10 |  |  | 10 |  |  |
| antenna isolation, dB | 10 |  |  | 10 |  |  |
| n28 receival singal at ANT port, dBm |  | 13 | 13 |  | 13 | 13 |
| n28 filter rejection at 814~849MHz, dB | 12 |  |  | 33 |  |  |
| signal After n28 filter, dBm |  | -3 | -3 |  | -24 | -24 |
| Typical receiver IIP2, dB | 50 |  |  | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -56 | -56 |  | -98 | -98 |
| front-end loss | 4 |  |  | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -52 | -52 |  | -94 | -94 |
| noise figure dB | 10.5 |  |  | 10.5 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -164 |  |  | -163.5 |  |  |
| Rx band n28 BW(MHz, NRB) | 4.5 |  |  | 4.5 |  |  |
| Thermal noise, dBm |  | -96.97 | -96.97 |  | -96.97 | -96.97 |
| ACLR1, dB | 30 |  |  | 30 |  |  |
| ACLR2, dB | 43 |  |  | 43 |  |  |
| n26 PA leakage PSD at PA output port at 1st adjacent channel, dBm/MHz |  | -20.01 |  |  | -20.01 |  |
| n26 PA leakage PSD at PA output port at 2nd adjacent channel, dBm/MHz |  | -33.01 |  |  | -33.01 |  |
| n26 PA noise PSD at PA output port, dBm/Hz | -125 |  |  | -125 |  |  |
| n26 PA noise PSD at PA output port, dBm/RxBW |  | -13.47817482 |  |  | -13.47817482 |  |
| n26 Tx filter rejection at 758~788MHz, dB | 35 |  |  | 45 |  |  |
| n26 PA noise power at Rx n28 ant port at 758~788MHz, dBm |  | -58.48 | -58.48 |  | -68.48 | -68.48 |
| Total noise level at ANT port |  | -51.12 | -51.12 |  | -68.46 | -68.46 |
| SNR requirement for QPSK | -1 |  |  | -1.00 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -52.12 | -52.12 |  | -69.46 | -69.46 |
| Implementation Margin, dB | 2.5 |  |  | 2.5 |  |  |
| combined REFSENS(5MHz BW), dBm | -52.6 |  |  | -69.97 |  |  |
| MSD | 45.87 |  |  | 28.53 |  |  |

Observation 1: Since 1st and 2nd adjacent channel leakage fall into the DL frequency range of band n28, serious REFSENS degradation can be observed for both case 1 and case 2.

Proposal 3: the MSD requirements due to cross band isolation for CA_n26-n28 can be further discussed after RAN4 reach some consensus on filter performance and n26 PA linearity performance.

Discussion on MSD due to IMD interference for case 2

The Band n26 and Band n28 UL IMD products are shown as below.

Table 2 Band n26 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 703 | 748 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 146 | 66 | 1517 | 1597 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 880 | 995 | 557 | 682 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2331 | 2446 | 2220 | 2345 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1694 | 1844 | 1260 | 1430 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3145 | 3295 | 2923 | 3093 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 132 | 292 | 3034 | 3194 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2178 | 1963 | 2693 | 2508 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 616 | 411 | 1141 | 946 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3626 | 3841 | 3959 | 4144 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3737 | 3942 | 3848 | 4043 |

Based on Table 2, 3rd order IMD may also fall into Rx frequencies of bands n26 when both Band n26 and Band n28 transmit the UL signals.

814

|  | UL frequency range (MHz) | DL frequency range (MHz) |
| --- | --- | --- |
| n26 | 814~819 | 859~864 |
| n28 | 743~748 | 798~803 |

Thus, we can calculate the frequency range which is hit by IMD3 from UL CA_n26-n28 as 880~895 MHz. It can be observed the IMD3 missed the DL channel of band n26 by 5MHz.

Proposal 4: Since the IMD3 from UL CA_n26-n28 missed the DL channel of band n26, there is no need to consider the MSD due to IMD3 for CA_n26-n28.

- Summary

Proposal 1: three-antennas solution can be assumed as baseline for real implementation or MSD evaluation considering the difficulties of Quadplexer/Triplexer implementation.

Proposal 2: The Delta Tib and Rib values for CA_n5-n28 can be reused for CA_n26-n28.

Table 1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n26A-n28A | n26 | 0.7 |
|  | n28 | 0.7 |

Table 2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n26A-n28A | n26 | 0.2 |
|  | n28 | 0.2 |

Observation 1: Since 1st and 2nd adjacent channel leakage fall into the DL frequency range of band n28, serious REFSENS degradation can be observed for both case 1 and case 2.

Proposal 3: the MSD requirements due to cross band isolation for CA_n26-n28 can be further discussed after RAN4 reach some consensus on filter performance and n26 PA linearity performance.

Proposal 4: Since the IMD3 from UL CA_n26-n28 missed the DL channel of band n26, there is no need to consider the MSD due to IMD3 for CA_n26-n28.

- References

[1] R4-2306471, WF on RF architecture and critical issues for CA_n26-n28, Huawei, HiSilicon

[2] R4-2306468, WF on CA_n5-n28 and CA_n8-n20-n28, Huawei, HiSilicon