3GPP TSG-RAN WG4 Meeting # 107                               		R4-2308594

Incheon, KR, May 22 – May 26, 2023

Agenda item:	9.6.3

Source: 	ZTE Corporation

Title: 	Discussion on UE RF requirements for FDD band (L+S band) for IoT NTN operation

Document for:	Approval

Introduction

In RAN#99 meeting, the work item [RP-230792] on FDD band (L+S band) for IoT NTN operation was approved. Part of the objective is as follows:

- Specify the following new FDD frequency bands for NB-IoT/eMTC NTN operation:

  - 1610 – 1626.5 MHz in UL, and 2483.5 – 2500 MHz in DL

- Support UE power class 3 (+23dBm) and power class 5 (+20dBm)

- In this contribution, we want to share some initial views on UE RF requirements for FDD band (L+S band) for IoT NTN operation.

Discussion

## 2.1 TX–RX frequency separation

The default E-UTRA TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for the new FDD frequency bands for NB-IoT/eMTC NTN operation can be specified in Table 2.1-1 as the starting point.

Table 2.1-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 254 | 873.5 MHz |

Proposal 1: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the TX–RX frequency separation defined as Table 2.1-1 can be used as the starting point.

## 2.2 UE maximum output power

Based on the objective of the WID, both UE power class 3 (+23dBm) and power class 5 (+20dBm) are supported in the new FDD frequency bands for NB-IoT/eMTC NTN operation. Both band 256 and band 255 have already supported power class 5. For the extended L-band ( band [253]), power class 5 also have been approved at last meeting. In addition, previous devices that supported PC5 could also attempt to access NTN network by band 254.

Base on the above analysis, the UE maximum output power for category M1 and category NB1 and NB2 can be specified in Table 2.2-1 and 2.2-2, respectively.

Table 2.2-1: UE Power Class for category M1

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 254 |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |

Table 2.2-2: UE Power Class for category NB1 and NB2

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 254 | 23 | +/-2 | 20 | +/-2 |

Proposal 2: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the UE maximum output power for category M1 and category NB1 and NB2 can be specified in Table 2.2-1 and 2.2-2, respectively.

## 2.3 Spurious emission band UE co-existence

The new FDD frequency bands is adjacent to the UL of NTN band 255 and the UE co-existence emission requirements for band 255 can be reused to the new FDD frequency bands as a baseline. In addition, for NR_NTN_LSband, the following agreement has been approved.

Agreement： - UE co-existence requirements: - Use L-band UE co-existence requirements as a baseline considering additional bands as follows: - NR band n54, n77, n78, n105; E-UTRA band 31, 54, 72, 73, 87, 88, 103; - Other bands can be considered during next meetings.

Therefore, additional bands of NR bands n54, n105 and E-UTRA band 31, 54, 72, 73, 87, 88, 103 should also be added to the list of protected bands.

Proposal 3: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the UE co-existence emission requirements for band 255 can be reused as a baseline and additional bands of NR band n54, n105 and E-UTRA band 31, 54, 72, 73, 87, 88, 103 should be added to the list of protected bands.

## 2.4 Reference sensitivity power level

The new FDD frequency bands is adjacent to the DL of NTN band 256 and the reference sensitivity power level REFSENS of band 256 can be reused to the new FDD frequency bands as shown in Table 2.4-1 and Table 2.4-2.

Table 2.4-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 254 | -102.2 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101. |  |  |

Table 2.4-2: Reference sensitivity for UE category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B in TS 36.102 | - 108.2 |

Proposal 4: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the reference sensitivity level for eMTC NTN can be defined as Table 2.4-1 and the reference sensitivity level for NB-IoT can be defined as Table 2.4-2.

## 2.5 Out-of-band blocking

For out-of-band blocking requirements of 254, the general requirements for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz can be reused. Out-of-band blocking requirements for band n53 and band 255 also used this general requirements. Therefore, for band 254, the out-of-band blocking requirements for category M1 and category NB1 and NB2 can be specified in Table 2.5-1 and 2.5-2, respectively.

Table 2.5-1: Out of-band blocking for category M1 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 254 | Finterferer (C`W) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band 256 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

Table 2.5-2: Out-of-band blocking parameters for category NB1 and NB2 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -153 |
| 254 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 [7] with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Band 256 lower frequency ranges are modified to enable specific implementations. NOTE 3: For operating bands which downlink band frequency range is between 1475.9 MHz < f < 2690 MHz the power level of the interferer (PInterferer) for Range 3 shall be modified to: -20 dBm for the frequency range which is bounded by FDL_low- 200 MHz of the lowest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz and FDL_high + 200 MHz of the highest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz.” NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |  |  |  |  |  |

Proposal 5: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the out-of-band blocking requirement for eMTC NTN can be defined as Table 2.5-1 and the out-of-band blocking requirement for NB-IoT can be defined as Table 2.5-2.

Conclusions

In this contribution, we want to share some further views on UE RF requirements for Introduction of the satellite L-/S-band. Some proposals are made as following:

Proposal 1: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the TX–RX frequency separation defined as Table 2.1-1 can be used as the starting point.

Proposal 2: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the UE maximum output power for category M1 and category NB1 and NB2 can be specified in Table 2.2-1 and 2.2-2, respectively.

Proposal 3: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the UE co-existence emission requirements for band 255 can be reused as a baseline and additional bands of NR band n54, n105 and E-UTRA band 31, 54, 72, 73, 87, 88, 103 should be added to the list of protected bands.

Proposal 4: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the reference sensitivity level for eMTC NTN can be defined as Table 2.4-1 and the reference sensitivity level for NB-IoT can be defined as Table 2.4-2.

Proposal 5: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the out-of-band blocking requirement for eMTC NTN can be defined as Table 2.5-1 and the out-of-band blocking requirement for NB-IoT can be defined as Table 2.5-2.

References

- RP-230792, “New WID: FDD band (L+S band) for IoT NTN operation”, MediaTek Inc. Apple, Globalstar, Kepler Communications, Qualcomm, ZTE