3GPP TSG-RAN WG4 Meeting #107 	R4-2308622

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on system parameters for NCR

Agenda item:	8.29.1.1

Document for:	Discussion

Introduction

The RAN4#106bis-e electronic meeting continued the discussion on Rel-18 Network Controlled Repeaters. Agreements reached at this meeting are captured in Way forward [1].

This paper contributes to the discussion on two open issues of NCR system parameters: NCR-MT system parameters and NCR-MT feature list.

Discussion

NCR-MT system parameters

As defined in the WF [1], NR Integrated access and backhaul (IAB) specified in TS 38.174 [2] can be taken as a starting point for defining system parameters for NCR-MT:

Take IAB-MT approach as starting point i.e. to set the channel bandwidth per operating band to be optional for NCR-MT.

- The bandwidth aspect is also specified in a companion paper related to NCR-MT RF requirements [3]: Currently TS 38.106 [4] specifies the passband and nominal channel bandwidth as follows:

- Passband: The frequency range in which the repeater operates in with operational configuration, this frequency range can correspond to one or several consecutive nominal channels, if they are not consecutive each subset of channels shall be considered as an individual passband, a repeater can have one or several passbands, all channels within the passband(s) shall belong to a single operator or collaborating operators.

- Nominal channel bandwidth: Bandwidth calculated as min (100MHz, BWpassband) in FR1 or min (400MHz, BWpassband) in FR2. If this bandwidth is not defined for BS channel bandwidth for the operating band, nominal channel bandwidth shall be defined as the widest BS channel bandwidth for the operating band which is narrower than BWpassband.

As MT operation is introduced to NCR, the NCR-MT channel bandwidth it could be defined similarly to the TS 38.174 [2] specification. NCR-MT channel bandwidth: RF bandwidth supporting a single NCR-MT RF carrier with the transmission bandwidth configured in the uplink or downlink.

It is propose to define the NCR-MT channel bandwidth according to the IAB-MT channel definition of TS 38.174.

NCR-MT feature list

In WF [1], it is suggested to further discuss NCR-MT feature list, FFS whether feature list is required or not for NCR-fwd from RAN4 perspective.

RAN1 has endorsed the initial feature list for NCR [5]. RAN1 will further define a separate UE capability will be used to indicate support for access link beam index updates or if the capability will be merged with the semi-persistent beam indication capability.

In our contribution of RAN2 [6] discusses about RAN1 endorsed the initial feature list for NCR.

The feature list annexed [6], with forthcoming changes, is considered sufficient to meet RAN4 requirements.

Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: As MT operation is introduced to NCR, the NCR-MT channel bandwidth it could be defined similarly to the TS 38.174 [3] specification. NCR-MT channel bandwidth: RF bandwidth supporting a single NCR-MT RF carrier with the transmission bandwidth configured in the uplink or downlink.

Proposal 1: is to define the NCR-MT channel bandwidth according to the IAB-MT channel definition of TS 38.174.

Observation 2: RAN1 has endorsed the initial feature list for NCR [5]. RAN1 will further define a separate UE capability will be used to indicate support for access link beam index updates or if the capability will be merged with the semi-persistent beam indication capability.

Proposal 2: The feature list annexed [6], with forthcoming changes, is considered sufficient to meet RAN4 requirements.

- References

- R4-2305931, WF on system parameters and RF requirements, ZTE, RAN4 #106-bis,

- TS 38.174, NR; Integrated access and backhaul radio transmission and reception

- R4-2308624, Discussion of RF requirement for NCR-MT, Nokia, Nokia Shanghai Bell,

- TS 38.106, NR; NR Repeater Radio Transmission and Reception.

- R1-2304282, Initial RAN1 UE features list for Rel-18 NR after RAN1#112bis-e, e-Meeting,

- R2-2305051, NCR access link beam index update capability, Nokia, Nokia Shanghai Bell,