3GPP TSG-RAN WG4 Meeting #107 	R4-2308624

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RF requirement for NCR-Fwd

Agenda item:	8.29.2.1

Document for:	Approval

Introduction

Agreements and open issues from RAN4#106-bis-e are summarized in Way Forward document [1]. This paper discusses the open issues regarding the RF requirements for NCR-Fwd.

Discussion

FDM operation

At the RAN4#106-bis-e meeting it was agreed that the conducted requirements for repeater type 1-C specified in TS 38.106 [2] could be applied to the NCR type 1-C Fwd part. Similarly, it was agreed in the meeting that the radiated requirements for repeater 2-O in the sub-clause 7 of TS 38.106 [2] specification could be reused for NCR type 2-O Fwd parts. Open issues were defined for FDM operation:

Further check whether FDM operation allowed or not; if allowed, further check any impact on the emission requirements for FDM operation

Multi-beam FDM is not supported for NCR. FDM for multiple UEs could be supported on the same beam.

Transmit ON/OFF power and transition power

During the RAN4#106-bis-e meeting, it was decided to further investigate the following options in relation to transmit ON/OFF power and transition power:

- For NCR-MT and NCR-Fwd switch ON-OFF together, proposed to have further study for it; - For NCR-MT and NCR-Fwd switch ON-OFF individually, propose to Reuse the Rel-17 repeater type 1-C requirements;

NCR-Fwd is in the OFF state when it is not otherwise controlled by the BS. NCR-Fwd is OFF when NCR-MT is in the RRC_idle state or off, but there are other states when NCR-Fwd is OFF (e.g. BS may configure NCR-Fwd to be OFF when NCR-MT is in RRC_IDLE or RRC_CONNECTED state).

NCR-MT can be individually switched ON-OFF during operation. NCR-Fwd is always in the OFF state when NCR-MT is OFF. When NCR-MT is in the ON state, the ON-OFF states of NCR-Fwd depend on the decision taken by the BS and applied by NCR-MT.

Conclusion

In the paper, the following Observations and Proposal were made:

Observation 1: Multi-beam FDM is not supported for NCR. FDM for multiple UEs could be supported on the same beam.

Observation 2: NCR-Fwd is in the OFF state when it is not otherwise controlled by the BS. NCR-Fwd is OFF when NCR-MT is in the RRC_idle state or off, but there are other states when NCR-Fwd is OFF (e.g. BS may configure NCR-Fwd to be OFF when NCR-MT is in RRC_IDLE or RRC_CONNECTED state).

Proposal 1: NCR-MT can be individually switched ON-OFF during operation. NCR-Fwd is always in the OFF state when NCR-MT is OFF. When NCR-MT is in the ON state, the ON-OFF states of NCR-Fwd depend on the decision taken by the BS and applied by NCR-MT.

- References

- R4-2305931, WF on system parameters and RF requirements, ZTE, RAN4 #106-bis, E-meeting, April 17 – April 26,2023.

- TS 38.106, NR; NR Repeater Radio Transmission and Reception.