3GPP TSG-RAN WG4 Meeting #107	R4-2308664

Incheon, KR, 22 – 26 May, 2023

Title: 	Discussion on NW A requirements with MUSIM gaps

Source: 	Huawei, HiSilicon

Agenda item:	8.26.2.3

Document for:	Discussion

Introduction

RRM requirements for NW A with MUSIM gaps configured are discussed in RAN4#106-bis-e and the outcomes are captured in [1]. Based on [1] the following issues need to be further discussed.

- General principle

- Parameters for L1/L3 measurement requirements

In this paper we will provide our views on RRM requirements for NW A with MUSIM gaps.

Discussion

General principle

Issue 3-1-1: Principle on layer 1 and layer 3 measurement requirements after gap collision handling - Proposals - P1: Frameworks of LBT failure in NR-U design can be used as starting point when discussing NW A L3/L1 requirement impact due to MUSIM gaps (Apple) - P2: Reuse the same principle of Rel-17 concurrent gaps WI to define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Ntotal /Navailable for network A requirements when MUSIM gaps are configured. (CMCC xiaomi vivo ZTE oppo Huawei Ericsson Qualcomm MTK Nokia) - P3: RAN4 to postpone the detail NW-A’s requirement discussion until RAN4 achieves the consensus on MUSIM gaps’ priority. (Ericsson Nokia) Agreements: No Candidate options: Recommendations: Discuss at the next meeting

In Rel-17 con-MG discussion, RAN4 developed a more generic approach to account for the impact of multiple MGs when defining measurement requirements. The scaling factor accounting for multiple MG impact is defined by counting the total number of measurement occasion in a window and the number of us-able occasions in the same window. It is straightforward to re-use the same approach for MUSIM gaps.

Proposal 1: Re-use the Rel-17 con-MG approach as baseline to define NW-A measurement requirements with MUSIM gaps, i.e. counting Navailable, Noutside_MG and Ntotal.

Parameters for L1/L3 measurement requirements

Issue 3-1-2: On parameters for L1/L3 measurement requirements - Proposals - P1: (CMCC): - For L3 measurement, Navailable need to be updated to cover MUSIM gaps - For L1 measurement, Noutside_MG and Navailable need to be updated to cover MUSIM gaps - P2: The following parameters need to be updated to account for collisions with MUSIM gaps (MTK): - Kp for intra-frequency and inter-frequency measurements without gaps (Xiaomi) - Kgap for intra-frequency and inter-frequency measurements with gaps (Xiaomi) - Kgap_EUTRA for inter-RAT measurements - Kp_CSI-RS for CSI-RS L3 measurements - Kp,PRS,I for NR positioning measurements - CSSFintra for intra-frequency measurements - CSSFinter for intra-frequency measurements - CSSFinterRAT for intra-RAT measurements - P scaling factor for L1-RSRP and L1-SINR measurements (xiaomi) - P3: Suggest the following update on parameters for L1/L3 measurement requirements (vivo ZTE) - For SSB based or CSI-RS based RLM, BFD and CBD, scaling factor P can be reused without any update, the definition of Ntotal, Noutside_MG and Navailable need updated. - For intra-frequency and inter-frequency measurements without gaps, scaling factor Kp can be reused and the definition of Ntotal and Navailable will be updated. - For intra-frequency and inter-frequency measurements with gaps, scaling factor Kgap can be reused and the definition of Ntotal and Navailable will be updated. - For inter-RAT E-UTRAN TDD/FDD measurement, scaling factor Kgap_EUTRA can be reused and the definition of Ntotal and Navailable will be updated. - For L1-RSRP/L1-SINR measurement, scaling factor P can be reused and the definition of Ntotal, Noutside_MG and Navailable will be updated. - For NR measurement for positioning, scaling factor Kp,PRSIi can be reused and the definition of Ntotal and Navailable will be updated. - For CSI-RS based L3 measurements, scaling factor Kp_CSI-RS can be reused and the definition of Ntotal and Navailable will be updated. - P4: (Huawei) - For L3 measurement outside MG, Kp in the requirements is updated - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion or non-dropped MUSIM gap occasion within the window W. - For L3 and positioning measurement with MG, existing requirements can be re-used. - For L1 measurement outside MG, Navailable, Noutside_MG in the requirements are updated - Noutside_MG is the number of SSB resource occasions that are not overlapped with any MG nor MUSIM gap within the window W - Navailable is the number of SSB resource occasions that are not overlapped with any MG, MUSIM gap nor any SMTC occasion within the window W - P5: Clarification for L3 measurement without gap, SMTC should not be fully overlapping with MUSIM gap (oppo) - P6: (Qualcomm) For intra-frequency and inter-frequency measurements without gaps in network A, modify the scaling factor Kp as follows: - The duration of the window W is max(SMTC period, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer - Ntotal is the total number of SMTC occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions For inter-frequency and inter-frequency measurements with gaps in network A, modify the scaling factor Kgap as follows: - The duration of the window W is max(SMTC period, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer - Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions - Navailable is the number of SMTC occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions For inter-RAT measurements with gaps in network A, modify the scaling factor Kgap_EUTRA as follows: - The duration of the window W is MGRP_max, where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within FR1 - Ntotal is the total number of associated measurement gap occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions - Navailable is the number of non-dropped associated measurement gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions For CSI-RS L3 intra-frequency measurements without gaps modify the scaling factor Kp_CSI-RS as follows: - The duration of the window W is max(CSI-RS period, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer - Ntotal is the total number of CSI-RS occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions - Navailable is the number of CSI-RS occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions For CSI-RS L3 inter-frequency measurements with gaps modify the scaling factor Kp_CSI-RS as follows: - The duration of the window W is max(CSI-RS period, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer - Ntotal is the total number of CSI-RS occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions - Navailable is the number of CSI-RS occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions For NR positioning measurements with gaps modify the scaling factor Kp,PRS,i as follows: - The duration of the window W is max(TPRS,i, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the positioning frequency layer - Ntotal is the total number of associated measurement gap occasions covering PRS occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions - Navailable is the number of non-dropped associated measurement gap occasions covering PRS occasions within the window W, after accounting for measurement gap and MUSIM gap collisions Clarify the definition of CSSFintra for intra-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted. Clarify the definition of CSSFinter for inter-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted. Clarify the definition of CSSFinterRAT for intra-RAT measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted. For L1-RSRP and L1-SINR measurements modify the scaling factor P as follows: - The duration of the window W is max(TL1, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the serving cell - Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions - Noutside_MG is the total number of SSB resource occasions that do not overlap with measurement gap occasions, MUSIM gap occasions nor SMTC occasions within the window W - Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions Agreements: No Candidate options: Recommendations: postpone the discussion until issue 3-1-1 is solved.

For L3 measurement outside MG, Kp is used in the requirements and with con-MG, Kp is defined as Kp = Ntotal / Navailable. In our view, the only change needed with MUSIM gaps is that the Navailable should exclude SMTC occasions that are overlapped with MUSIM gaps.

Proposal 2: For L3 measurement outside MG, Kp in the requirements is updated

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion or non-dropped MUSIM gap occasion within the window W.

For L3 measurement with MG, Kgap is used in the requirements and with con-MG, Kp is defined as Kp = Ntotal / Navailable. In our view, the existing definition of Ntotal and Navailable can be re-used with MUSIM gaps because existing Navailable is already based on non-dropped MG occasions. The same also applies to positioning measurement with MG.

Proposal 3: For L3 and positioning measurement with MG, existing requirements can be re-used.

For L1 measurement, P is used in the requirements and with con-MG, P is defined as Ntotal / Navailable

- Ntotal / Noutside_MG in FR1

- Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

- Ntotal / Navailable in FR2 with Navailable > 0.

In our view, the only change needed with MUSIM gaps is that the Noutside_MG and Navailable should exclude SMTC occasions that are overlapped with MUSIM gaps.

Proposal 4: For L1 measurement outside MG, Navailable, Noutside_MG in the requirements are updated

- Noutside_MG is the number of SSB resource occasions that are not overlapped with any MG nor MUSIM gap within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any MG, MUSIM gap nor any SMTC occasion within the window W

Conclusions

In this paper we provided our views on RRM requirements for NW A with MUSIM gaps.

Proposal 1: Re-use the Rel-17 con-MG approach as baseline to define NW-A measurement requirements with MUSIM gaps, i.e. counting Navailable, Noutside_MG and Ntotal.

Proposal 2: For L3 measurement outside MG, Kp in the requirements is updated

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion or non-dropped MUSIM gap occasion within the window W.

Proposal 3: For L3 and positioning measurement with MG, existing requirements can be re-used.

Proposal 4: For L1 measurement outside MG, Navailable, Noutside_MG in the requirements are updated

- Noutside_MG is the number of SSB resource occasions that are not overlapped with any MG nor MUSIM gap within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any MG, MUSIM gap nor any SMTC occasion within the window W

Reference

- R4-2306357, WF on NR Dual TxRx Multi-SIM, vivo