3GPP TSG-RAN WG4 Meeting #107	R4-2308667

Incheon, KR, 22 – 26 May, 2023

Title: 	Discussion on RRM requirements for SL positioning

Source: 	Huawei, HiSilicon

Agenda item:	8.23.3.2

Document for:	Discussion

Introduction

RRM requirements for SL positioning are discussed in RAN4#106-bis-e, and outcomes are captured in WF [1]. Based on [1] the following issues are to be further discussed.

- General issue

- Core requirements

- Perf requirements

- Transmit timing of SL UE

- Other

In this paper we will provide our views on RRM requirements for SL positioning.

Discussion

General issue

Issue 1-1-2: Applicable CBW and SCS: Agreements: - Specify RRM requirement for sidelink positioning up to 40MHz CBW and all supported SCS for sidelink for FR1. - FFS for support of larger CBW for up to 100 MHz

We suggest to define measurement period requirements agnostic to the BW of SL PRS. For Uu positioning this is also the case, i.e. the core requirements are applicable for all PRs BWs. In the Perf part, RAN4 can further discuss for which BW ranges to define accuracy requirements also considering progress in other WIs.

Proposal 1: The measurement period requirements are agnostic to the BW of SL PRS. FFS the applicable PRS BW for defining accuracy requirements.

Issue 1-1-7: Spec structure: < Way forward > - Option 1: - RAN4 to discuss, from spec maintenance point of view, whether common measurement period requirements can be defined for all SL PRS measurements. - Option 2: - RAN4 to define the RRM core and performance requirements in sub clause 9.9X, 10.1X and 13.X for SL positioning RRM core requirements, RRM UE performance requirements and RRM gNB performance requirements, respectively.

For NR positioning, requirements for different PRS measurements are specified in different clauses, but most of the requirements are same. The main difference is the applicability of the requirements. For SL, we suggest RAN4 to consider defining common measurement period requirements for all measurements. Of course, this does not mean every parameter in the measurement period would be same for all measurements, or the applicability of the requirements would be same for all measurements. This would be up to the technical discussion as usual. Our suggestion is mainly from spec maintenance point of view, i.e. to avoid repeating same measurement period framework multiple times for different measurements.

Proposal 2: RAN4 to discuss, from spec maintenance point of view, whether common measurement period requirements can be defined for all SL PRS measurements.

On the clauses to capture the requirements, we think it may be better to capture the core (measurement period) requirements in clause 12 together with other SL related RRM requirements. Performance requirements should be captured in clause 10 as in option2, and so far we see no requirements on gNB side needs to be defined for SL positioning.

Issue 1-1-8: Terminology: < Way forward > - Option 1: - RAN4 to define the RRM core and performance requirements for Sidelink positioning using terminology of “V2X” instead of “SL” in order to maintain uniformity in the spec while referring to V2X/SL requirements, as specified in current specification. - Option 2: - Use the SL terminology in RAN4 requirements.

We prefer to use the term “SL” for SL positioning related requirements. We understand the existing RRM requirements are for “V2X”, but for SL positioning, V2X is just one of the considered scenarios, and other scenarios like public safety and IIOT are also in the scope. Using “V2X” may be misleading to people outside RRM session and cause inconsistency with other WGs.

Proposal 3: RAN4 to use consistent terms with other WGs for SL positioning related requirements.

Core requirements

Issue 1-2-1: Whether to define measurement period requirements for the following measurement types: Agreements: - Define measurement period requirements for the following measurement types - SL-PRS RSRP - SL-PRS RSRPP - SL-PRS RSTD - SL-PRS Rx-Tx - FFS for SL-PRS RTOA, SL-PRS AOA/ZOA

We support to define measurement period requirements for all measurement types.

For Uu positioning, there is no core requirements for UL-RTOA or AOA/ZOA because they are measured by gNB and it was agreed to not define core requirements for gNB measurements. For SL positioning, SL-PRS RTOA and SL-PRS AOA/ZOA are also measured by UE, and we do not see any issue in defining measurement period for them.

Proposal 4: RAN4 to define measurement period requirements for all measurement types.

Issue 1-1-5: Applicable number of samples: Agreements: - RAN4 to discuss the number of samples for SL PRS measurement period and for accuracy requirements.

We support to define SL PRS measurement period requirements using 1 sample as baseline.

For many use cases of SL positioning, latency is an important requirement. On the other hand, we believe the Es/Iot side condition for SL PRS measurement can be better than PRS because anchor and target UEs are not supposed to be far apart. In addition, symbol for AGC is reserved in SL frame structure. All these factors make 1 sample measurement suited for SL PRS measurement. We are open to further discussing whether to support multiple samples in defining requirements.

Proposal 5: Measurement based on single sample is used as baseline for SL PRS measurement period and for accuracy requirements.

Issue 1-5-2: Impact of S-SSB < Way forward > - Option 1: - Considering that the resources for SL PRS are TDMed with that for S-SSB, the sharing mechanism such as CSSF or K defined in NR Uu-link cannot be reused for sidelink and a new sharing mechanism between SL PRS and S-SSB should be considered.

One issue raised up by companies last meeting is the sharing between SL PRS measurement and other SL measurements. We think it is a valid issue to be investigated. On the other hand, the issue may also depend on RAN1 progress, e.g. multiplexing between SL PRS and other SL signals/channels.

Proposal 6: RAN4 to discuss possible sharing between SL PRS measurement and other SL measurements based on RAN1 agreements.

Issue 1-2-2: How to define the measurement period requirements: < Way forward > - Option 1: - RAN4 waits for more RAN1/2 progress before discussing exact SL PRS measurement period requirements. - Option 2: - RAN4 prioritizes measurement period requirements for SL-PRS-RSRP and further discuss whether the measurement period requirements for other SL PRS-based measurements are different from that for SL-PRS-RSRP. - Option 3: - For the measurement requirements for SL positioning, the R17 PRS measurement requirements can be the baseline taking the following possible differences into account: - UE processing capability - Number of samples - Number of Rx beams - Gap sharing with Uu positioning and/or RRM measurement

As to the exact measurement period, we think RAN4 needs to wait for more RAN1/2 progress. In last meeting, some companies proposed to re-use existing PRS requirements as baseline. We think it may be too early to make such agreements, as the PHY and higher layer procedure for SL PRS measurement may be different from PRS. For example, what kind of processing capability will be defined for SL PRS measurement is still unclear from RAN1.

Proposal 7: RAN4 waits for more RAN1/2 progress before discussing exact SL PRS measurement period requirements.

Perf requirements

Issue 1-3-1: Whether to define accuracy requirements for the following measurement types < Way forward > - FFS whether to define measurement accuracy requirements for the following measurement types: - SL-PRS RSRP - SL-PRS RSRPP - SL-PRS RSTD - SL-PRS Rx-Tx - SL-PRS RTOA, - SL-PRS AOA/ZOA

Issue 1-3-6: Reference time for SL PRS based RTOA measurement < Way forward > - Option 1: - RAN4 to discuss whether and how to define absolute accuracy requirements for SL RTOA measurements. - In particular, RAN4 would need to discuss how to account for timing uncertainty of acquiring the RTOA Reference Time (T0) in the requirements.

We think the issue can be postponed to the Perf part.

Likely we will follow the same principle in Uu positioning because the difficulties in defining accuracy are same if the measurement definition are similar between Uu and SL. Since RAN1 has not finalized measurement definition for all types, we suggest RAN4 to make decision in Perf part.

Proposal 8: RAN4 to decide for which measurement types to define accuracy requirements in the Perf part.

Issue 1-3-3: Channel conditions for accuracy requirements: Agreements: - For the accuracy requirements for SL positioning, RAN4 to discuss the scenarios and channel models used for simulation and wait for more progress from RAN1 on the SL PRS configuration.

In Uu positioning, both AWGN and fading channel are considered in accuracy requirements, and for timing related measurements, separate requirements for defined due to large difference in the performance. We think the same approach can be used in SL positioning. On the exact channel model for fading, we suggest to consider TDL-A as baseline which was used for Uu in FR1 at least for the simulation evaluation. We are also open to other views.

Proposal 9: RAN4 to consider AWGN and [TDL-A] channel for simulation evaluation.

Issue 1-3-5: Side condition < Way forward > - Option 1: - Define requirements for SL PRS measurement based on Es/Iot side condition of [-6, 0 or 3] dB.

The Es/Iot side condition will impact the development of the requirements e.g. the number of samples. Uu PRS measurements are conditioned on different Es/Iot levels between 0dB and down to -13dB. RAN4 could check whether they are applicable for SL positioning scenarios.

In Figure 1, we illustrate the CDF of SNR for the first n=8 strongest RSUs in a high way scenario which was evaluated during the SI phase. RSUs are deployed on each side of the high way, separated by 200m. The width of the high way is 24m.

Figure 1: CDF of SNR for the first n=8 strongest RSUs in a high way scenario

It can be seen that due to small distance between UE and RSUs in SL positioning, the SNR is higher compared to -13dB assumed for NR positioning. Even looking at the 8-th strongest RSU, 5%-tile UE can still see above -6dB SNR. We are open to discuss the exact side condition, e.g. whether higher condition like 0dB or 3dB should be used assuming typical positioning fix does not require 8 RSUs.

Proposal 10: Define requirements for SL PRS measurement based on Es/Iot side condition of [-6, 0 or 3] dB.

Transmit timing of SL UE

Issue 1-4-1: Timing error limit requirements Agreements: - RAN4 to discuss the impact of UE timing error requirements on the sidelink positioning measurement period and accuracy requirements. < Way forward > - Option 1: - RAN4 needs to specify a more stringent timing error limit requirement for SL positioning. - Can be necessary the anchor UE’s transmission time error requirement. (e.g. less than 10 nsec which equates to about 3m distance offset) - Option 2: - RAN4 can reuse current Te requirements in order to define the performance requirements. In parallel RAN4 could check feasibility for improved Te requirements. - Option 3: - Reuse the existing Te requirements for SL positioning in Rel-18. - Option 4: - Other options are not precluded. Issue 1-4-2: Transmission time adjustment < Way forward > - Option 1: - RAN4 needs to specify transmission time adjustment with high granularity requirement for SL positioning. - Option 2: - No new requirements related to transmit timing adjustment for SL positioning.

In last meeting, some companies raised up the issue related to UE transmit timing accuracy in SL. The current accuracy requirements is around 12Ts, and it may cause synchronization error among anchor UEs and thus degradation in the positioning accuracy.

We agree that this may be an issue, but we understand the same issue exists also for Uu positioning, e.g. there is no explicit requirement on the gNB transmit accuracy, and for TDD the phase error between gNBs can be up to 3us. 3us was derived based on the need of data communication, and there is no additional requirement for positioning. The same principle should be used for SL.

In synchronization issue can be solved by NW implementation, e.g. a PRU can be used to calibrate the timing error between gNBs or anchor UEs. However, this is out of RAN4 scope, and we do not see the need to define new requirements on the transmit timing accuracy or timing adjustment.

Proposal 11: Re-use the existing Te requirements for SL positioning in Rel-18. No new requirements related to transmit timing adjustment for SL positioning are defined.

Other

Issue 1-5-1: Impact of SL-DRX < Way forward > - Option 1: - RAN4 to prioritize non-DRX case for the measurement period for SL PRS-based measurements. - Option 2: - RAN4 to discuss whether the measurement period for SL PRS-based measurements is impacted by the SL-DRX cycle length.

We suggest to prioritize the requirements for non-DRX.

On one hand, for many use cases of SL positioning, latency is an important requirement, while power consumption may not be a big issue for SL UEs. On the other hand, once the requirements for non-DRX are stable, it can be adapted to DRX case. How UE would measure SL PRS in case of DRX also depends on procedure of SL PRS measurement discussed in RAN1.

Proposal 12: RAN4 to prioritize non-DRX case for the measurement period for SL PRS-based measurements.

Issue 1-4-3: Synchronization source change Agreements: - RAN4 to discuss the impact of a synchronization source change on an SL positioning measurement (e.g., on measurement performance, measurement procedure, UE behaviour, etc.).

Issue 1-5-3: Impact of UE mobility < Way forward > - Option 1: - Anchor UEs should be capable of simultaneous transmission SL-PRS. - Target UE can be capable of simultaneous SL-PRS measurement with UE capability. - Option 2: - RAN4 to study the impact of change of network coverage scenario of SL UE(s) on measurement requirement, measurement accuracy, etc.

In last meeting, some companies proposed to discuss the impact of SyncRef UE change or coverage status change. We think they are valid points, but it may be too early to discuss transmission case before the basic requirements are stable.

Proposal 13: RAN4 to discuss the impact of transition (sync source change or coverage status change) after requirements for non-transition case are stable.

In last meeting, some companies proposed to discuss simultaneous Tx/Rx capability of SL PRS for anchor and target UE. We assume this is RAN1 scope as in Uu positioning.

Proposal 14: Simultaneous Tx/Rx capability is up to RAN1 discussion.

Issue 1-5-5: Initiation/cease of SL transmissions for positioning < Way forward > - Option 1: - RAN4 to define the requirements for initiation/cease of SL transmissions for positioning once the UE behavior and procedure is defined in RAN1.

In last meeting, some companies proposed to define requirements for initiation/cease of SL transmissions for positioning once the UE behavior and procedure is defined in RAN1. We think it is a valid point.

RAN4 has defined the requirements on initiation/cease of SLSS transmissions, which is based on RAN1 defined UE behaviour and RSRP or PSBCH-RSRP measurement. Similar requirements can be discussed after UE behaviour on SL PRS transmission is clear from RAN1.

Proposal 15: RAN4 to discuss the requirements for initiation/cease of SL PRS once the UE behavior and procedure is defined in RAN1.

Conclusions

In this paper we provided our views on RRM requirements for SL positioning.

Proposal 1: The measurement period requirements are agnostic to the BW of SL PRS. FFS the applicable PRS BW for defining accuracy requirements.

Proposal 2: RAN4 to discuss, from spec maintenance point of view, whether common measurement period requirements can be defined for all SL PRS measurements.

Proposal 3: RAN4 to use consistent terms with other WGs for SL positioning related requirements.

Proposal 4: RAN4 to define measurement period requirements for all measurement types.

Proposal 5: Measurement based on single sample is used as baseline for SL PRS measurement period and for accuracy requirements.

Proposal 6: RAN4 to discuss possible sharing between SL PRS measurement and other SL measurements based on RAN1 agreements.

Proposal 7: RAN4 waits for more RAN1/2 progress before discussing exact SL PRS measurement period requirements.

Proposal 8: RAN4 to decide for which measurement types to define accuracy requirements in the Perf part.

Proposal 9: RAN4 to consider AWGN and [TDL-A] channel for simulation evaluation.

Proposal 10: Define requirements for SL PRS measurement based on Es/Iot side condition of [-6, 0 or 3] dB.

Proposal 11: Re-use the existing Te requirements for SL positioning in Rel-18. No new requirements related to transmit timing adjustment for SL positioning are defined.

Proposal 12: RAN4 to prioritize non-DRX case for the measurement period for SL PRS-based measurements.

Proposal 13: RAN4 to discuss the impact of transition (sync source change or coverage status change) after requirements for non-transition case are stable.

Proposal 14: Simultaneous Tx/Rx capability is up to RAN1 discussion.

Proposal 15: RAN4 to discuss the requirements for initiation/cease of SL PRS once the UE behavior and procedure is defined in RAN1.

Reference

- R4-2306351, WF on RRM requirements for NR sidelink positioning and carrier phase positioning, CATT